3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250450

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3689 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to multiple UTRAN testcases for handling of LAU Request and RAU Request. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of 9.4.2.4.4, in local tree lt_Body, the test step ts_MM_LupProc3RAU is called to handle the LAU procedure and the subsequent RAU procedure. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. This issue is observed in testcases 9.4.1, 9.4.2.4.1, 9.4.2.4.4, 9.4.5.3, 9.4.7 and 9.4.8 To address this issue, R5s160153 was raised, but there are some UEs that transmit RAU/ATTACH REQ after the LAU ACC is transmitted. The solution provided in MCCTF160 comments for R5s160153 to above problem does not take this into account, and sometimes the testcases fail. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 9.4.1 : In localTree lt_SwitchCellToA_LupPag and lt_SwitchCellToB_LupPag2, called test step ts_SetDiscardIntegrityFailure ( TRUE ) beore ts_GMM_PrepRAU, and called ts_SetDiscardIntegrityFailure ( FALSE) after ts_GMM_RAU_Accept(tsc_CellA) For 9.4.2.4.1 : In localTree lt_Body, called ts_SetDiscardIntegrityFailure ( TRUE ) before ts_MM_LupInit2wIP, and called ts_RRC_Delay(200) followed by ts_SetDiscardIntegrityFailure ( FALSE ) before ts_RRC_ConnRel. Please see below for details. For 9.4.2.4.4 : In current TTCN implementation, lt_Body ts_MM_LupProc3RAU is called. In test step ts_MM_LupProc3RAU, called test step ts_SetDiscardIntegrityFailure (TRUE) before test step ts_MM_LupInit2wIP and then called test stepts_SetDiscardIntegrityFailure ( FALSE) before calling ts_RRC_ConnRel. For 9.4.5.3 : In current TTCN implementation, lt_Body ts_MM_LupProc3RAU is called. In test step ts_MM_LupProc3RAU, called test step ts_SetDiscardIntegrityFailure (TRUE) before test step ts_MM_LupInit2wIP and then called test stepts_SetDiscardIntegrityFailure ( FALSE) before calling ts_RRC_ConnRel. Also, in current TTCN implementation, lt_Body ts_MM_IMSI_Attach is called. In the test sep ts_MM_IMSI_Attachlt_IMSI_Attach, called test step ts_SetDiscardIntegrityFailure ( TRUE ) after ts_RRC_ConnEst and called test steps ts_RRC_Delay(200) followed by ts_SetDiscardIntegrityFailure ( FALSE ) in localTree lt_LocUpdateProcContinue. For 9.4.7 : In localTree lt_Body, called the test step ts_RRC_Delay(200) before ts_SetDiscardIntegrityFailure ( FALSE). This is to accommodate a UE that initiates the ATTACH REQ after LAU ACC. For 9.4.8: In localTree lt_Continue, called the test step ts_RRC_Delay(200) before ts_SetDiscardIntegrityFailure ( FALSE). This is to accommodate a UE that initiates the ATTACH REQ after LAU ACC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A compliant device may incorrectly fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1, 9.4.2.4.1, 9.4.2.4.4, 9.4.5.3, 9.4.7 and 9.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	5

2.1 Change 1	6

2.2 Change 2	7

2.3 Change 3	9

2.4 Change 4	10

2.5 Change 5	12

2.6 Change 6	13

# Overview

This document lists all the essencial changes needed to correct problems in the ATS iwd-TTCN2-B2012-03_D24wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Object name | - tc_9_4_1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.1, in local tree lt_SwitchCellToB_LupPag2, at line #44 LAU REQ is expected, and ATTACH REQ is expected to be handled in defaults. Similarly, in localTree lt_SwitchCellToA_LupPag, at line #28 LAU REQ is expected, and ATTACH REQ is expected to be handled in defaults. In both situations, as soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. |
| Summary of change | In localTree lt_SwitchCellToA_LupPag and lt_SwitchCellToB_LupPag2, called test step ts_SetDiscardIntegrityFailure ( TRUE ) beore ts_GMM_PrepRAU, and called ts_SetDiscardIntegrityFailure ( FALSE) after ts_GMM_RAU_Accept(tsc_CellA) |
| MCC160 |  |

Before change:

After Change:

## 2.2 Change 2

| Object name | - tc_9_4_2_4_1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.2.4.1, in local tree lt_Body, at line #20 test step ts_MM_LupInit2wIP is called to handle the expected LAU REQ, and ATTACH REQ is expected to be handled in defaults. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. To address this issue, R5s160153 was raised, but there are some UEs that transmit RAU/ATTACH REQ after the LAU ACC is transmitted. The solution provided in MCCTF160 comments for R5s160153 to above problem does not take this into account, and sometimes the testcases fail. |
| Summary of change | In localTree lt_Body, called ts_SetDiscardIntegrityFailure ( TRUE ) before ts_MM_LupInit2wIP, and called ts_RRC_Delay(200) followed by ts_SetDiscardIntegrityFailure ( FALSE ) before ts_RRC_ConnRel. Please see below for details. |
| MCC160 |  |

Before change:

After Change:

## 2.3 Change 3

| Object name | - ts_MM_LupProc3RAU |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.2.4.4, in local tree lt_Body ts_MM_LupProc3RAU is called. In this, at line#3 LAU is expected, and subsequent RAU is handled in defaults. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. |
| Summary of change | In test step ts_MM_LupProc3RAU, called test step ts_SetDiscardIntegrityFailure (TRUE) before test step ts_MM_LupInit2wIP and then called test stepts_SetDiscardIntegrityFailure ( FALSE) before calling ts_RRC_ConnRel. |
| MCC160 |  |

Before change:

After Change:

## 2.4 Change 4

| Object name | - ts_MM_IMSI_Attach |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.5.3, in local tree lt_Body ts_MM_LupProc3RAU is called. This is now already handled via Change 3 above. Later in localTree lt_Body ts_MM_IMSI_Attach is called. In this, LAU is expected (line #13 and #18), and subsequent RAU is handled in defaults. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. To address this issue, R5s160153 was raised, but there are some UEs that transmit RAU/ATTACH REQ after the LAU ACC is transmitted. The solution provided in MCCTF160 comments for R5s160153 to above problem does not take this into account, and sometimes the testcases fail. |
| Summary of change | In the test sep ts_MM_IMSI_Attachlt_IMSI_Attach, called test step ts_SetDiscardIntegrityFailure ( TRUE ) after ts_RRC_ConnEst and called test steps ts_RRC_Delay(200) followed by ts_SetDiscardIntegrityFailure ( FALSE ) in localTree lt_LocUpdateProcContinue. |
| MCC160 |  |

Before change:

After Change:

## 2.5 Change 5

| Object name | - tc_9_4_7 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.7, in local tree lt_Body, at line #20 test step ts_MM_LupInit2wIP is called to handle the expected LAU REQ, and ATTACH REQ is expected to be handled in defaults. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. To address this issue, R5s160153 was raised, but there are some UEs that transmit RAU/ATTACH REQ after the LAU ACC is transmitted. The solution provided in MCCTF160 comments for R5s160153 to above problem does not take this into account, and sometimes the testcases fail. |
| Summary of change | In localTree lt_Body, called the test step ts_RRC_Delay(200) before ts_SetDiscardIntegrityFailure ( FALSE). This is to accommodate a UE that initiates the ATTACH REQ after LAU ACC. Please see below. |
| MCC160 |  |

Before change:

After Change:

## 2.6 Change 6

| Object name | - tc_9_4_8 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 9.4.7, in local tree lt_Continue, at line #35 test step ts_MM_LupInit2wIP is called to handle the expected LAU REQ, and ATTACH REQ is expected to be handled in defaults. As soon as the SS receives the LAU REQ, the security procedures are triggered and RB3 will be suspended until the securitymode procedure is completed. If the UE transmits the RAU REQ during this time (when the RB3 is suspended), the RLC PDUs will be only delivered to higher layers when RLC is resumed and this would result in integrity failure as RAU Request will not be integrity protected (as its sent before the security procedure) and the SS is activated to check for integrity. To address this issue, R5s160153 was raised, but there are some UEs that transmit RAU/ATTACH REQ after the LAU ACC is transmitted. The solution provided in MCCTF160 comments for R5s160153 to above problem does not take this into account, and sometimes the testcases fail. |
| Summary of change | In localTree lt_Continue, called the test step ts_RRC_Delay(200) before ts_SetDiscardIntegrityFailure ( FALSE). This is to accommodate a UE that initiates the ATTACH REQ after LAU ACC. |
| MCC160 |  |

Before change:

After Change: