3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250452

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4940 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 8.5.4.1 for LTE band 106 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_900MHz_US-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LTE band 106 has been introduced by R5-242393, but the corresponding capability check is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the specification of the PICS item - Updated derivation of supported EUTRA bands from PICS for v9e0 - Implemented check of LTE band 106 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability check of LTE band 106 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | n/a |
| --- | --- |
| Reason for change | PICS item for LTE band 106 is missing . |
| Summary of change | Added the specification of the PICS item |
| TTCN module | EUTRA_NB_Parameters.ttcn |

After Change

… modulepar boolean pc_eBand103_Supp := false; // @desc Support of Band 103 // Reference: 36.523-2 A.4.3.1-1/103 modulepar boolean pc_eBand106_Supp := false; // @desc Support of Band 106 // Reference: 36.523-2 A.4.3.1-1/106 modulepar boolean pc_eBand10_Supp := false; // @desc Support of Band 10 // Reference: 36.523-2 A.4.3.1-1/10 …

## Change 2

| Function name | f_DeriveSuppEutraBandsFromPics_v9e0 |
| --- | --- |
| Reason for change | Derivation of supported EUTRA bands from PICS for v9e0 does not account for LTE band 106 |
| Summary of change | Updated derivation of supported EUTRA bands from PICS for v9e0 |
| TTCN module | EUTRA_AuxiliaryCapCheckFunctions.ttcn |

Before Change

function f_DeriveSuppEutraBandsFromPics_v9e0(B1_Type p_BitValuseForUndefinedBand := '0'B ) return bitstring { … p_BitValuseForUndefinedBand &//Band 102 not yet defined; f_ConvertBoolToBit(pc_eBand103_Supp) & //@sic R5-225938 sic@ p_BitValuseForUndefinedBand &//Band 104 not yet defined p_BitValuseForUndefinedBand &//Band 105 not yet defined p_BitValuseForUndefinedBand &//Band 106 not yet defined p_BitValuseForUndefinedBand &//Band 107 not yet defined … }

After Change

function f_DeriveSuppEutraBandsFromPics_v9e0(B1_Type p_BitValuseForUndefinedBand := '0'B ) return bitstring { … p_BitValuseForUndefinedBand &//Band 102 not yet defined; f_ConvertBoolToBit(pc_eBand103_Supp) & //@sic R5-225938 sic@ p_BitValuseForUndefinedBand &//Band 104 not yet defined p_BitValuseForUndefinedBand &//Band 105 not yet defined f_ConvertBoolToBit(pc_eBand106_Supp) & //@sic R5s250452 sic@ p_BitValuseForUndefinedBand &//Band 107 not yet defined … }

## Change 3

| Function name | f_CheckUE_SupportedEUTRA_BandsExt |
| --- | --- |
| Reason for change | Check of LTE band 106 is not implemented |
| Summary of change | Implemented check of LTE band 106 |
| TTCN module | EUTRA_CapabilityFunctions.ttcn |

Before Change

function f_CheckUE_SupportedEUTRA_BandsExt(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { … // appearance of band 103 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 38, 1); // band 103 [38 = 103-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 38, 1); // @sic R5-225938 sic@ // appearance of NTN bands 252-256 // not checked @sic R5-255055, R5s250332 sic@ } }

After Change

function f_CheckUE_SupportedEUTRA_BandsExt(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { … // appearance of band 103 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 38, 1); // band 103 [38 = 103-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 38, 1); // @sic R5-225938 sic@ // appearance of band 106 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 41, 1); // band 106 [41 = 106-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 41, 1); // @sic R5s250452 sic@ // appearance of NTN bands 252-256 // not checked @sic R5-255055, R5s250332 sic@ } }