3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250457

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 1000 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS CEN test case 21.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 21.30 to the IMS_EUTRA test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 21.30 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1 Verification Test Summary	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	4

2.3	Change 3	6

3	Branches executed	11

4	Execution Log Files	11

4.1	Qualcomm Snapdragon Auto 5G Modem-RF	11

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 21.30 which is part of the IMS_EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## 1.1 Verification Test Summary

Test Case: 	21.30

Test Group:                         \IMS\21\IMS_eCall_PSAPCallbackTestcases.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk38

System Simulator used:	Rohde & Schwarz CMW500

UE used:	Qualcomm Snapdragon Auto 5G Modem-RF UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_EUTRA_IPCAN_EmergencyCall |
| --- | --- |
| Reason for change | In testcase 21.30, the call to function f_IMS_ECall_AbnormalCase_IMS2 f_IMS_IPCAN_StartProcedure triggers the call f_EUTRA_IPCAN_MainLoop f_EUTRA_IPCAN_StartProcedure with argument v_Command.TestProcedure set as IPCAN_EmergencyCall_Aborted. This results in call of f_EUTRA_IPCAN_StartProcedure f_EUTRA_IPCAN_EmergencyCall. In the function f_EUTRA_IPCAN_EmergencyCall, v_StartIPfor2ndPDN is set as false, and passed to function f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_12. Because of this setting, the function f_EUTRA_StartIPfor2ndPDN is never invoked, and this results in IP routing mismatch. This needs to be addressed. |
| Summary of change | Updated the function definition for f_EUTRA_IPCAN_EmergencyCall by adding a new argument p_StartIPfor2ndPDN, which is set by default to false. Within this function, assigned v_StartIPfor2ndPDN := p_StartIPfor2ndPDN. |
| TTCN module | IPCAN_EUTRA.ttcn |

Before Change:

function f_EUTRA_IPCAN_EmergencyCall(EUTRA_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType, EPS_BearerIdentity p_DefaultBearerId, boolean p_ConnectionEstablished) runs on EUTRA_PTC return integer { /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5-253122, R5-253123: p_VoiceBearerToBeEstablished to be set to false for IPCAN_EmergencyCall_Aborted sic@ */ /* @sic R5-255081: return integer for improved management of EPS bearers sic@ */ var boolean v_StartIPfor2ndPDN := false; /* @sic R5s150412 sic@ */ var EPS_BearerIdentity v_DedicatedBearerIdSpeech; var EPS_BearerIdentity v_DedicatedBearerIdRTT; var NAS_MSG_Indication_Type v_NasInd; var integer v_NumberOfAddDedicatedBearers; // Table 4.5A.4.3-1 step 2 - 8 if (not p_ConnectionEstablished) { /* @sic R5s250249: "not" sic@ */ f_EUTRA_508_EstablishIMSEmergencyCall_Steps2_8(p_CellId); } // Table 4.5A.4.3-1 step 9 v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(p_CellId); // Table 4.5A.4.3-1 step 10 - 12 f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_12(p_CellId, p_DefaultBearerId, v_NasInd, v_StartIPfor2ndPDN); // Table 4.5A.4.3-1 step 13 - 15 select (p_ProcedureType) { case (IPCAN_EmergencyCall_Aborted) { v_NumberOfAddDedicatedBearers := 0; // no dedicated bearer to be established } case (IPCAN_EmergencyCall_WithRTT) { // @sic R5-253113: test case 19.1.7 (emergency call with RTT) sic@ v_DedicatedBearerIdSpeech := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); v_DedicatedBearerIdRTT := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId, 2); f_EUTRA_ActivateDedicatedEpsBearer_SpeechWith2ndMedia(p_CellId, RTT, p_DefaultBearerId, v_DedicatedBearerIdSpeech, v_DedicatedBearerIdRTT, PDN_2); v_NumberOfAddDedicatedBearers := 2; } case else { v_DedicatedBearerIdSpeech := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); f_EUTRA_ActivateDedicatedEpsBearer_Speech(p_CellId, p_DefaultBearerId, v_DedicatedBearerIdSpeech, PDN_2); v_NumberOfAddDedicatedBearers := 1; } } return v_NumberOfAddDedicatedBearers; }

After Change:

function f_EUTRA_IPCAN_EmergencyCall(EUTRA_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType, EPS_BearerIdentity p_DefaultBearerId, boolean p_ConnectionEstablished, boolean p_StartIPfor2ndPDN := false /*WA#21_30*/) runs on EUTRA_PTC return integer { /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5-253122, R5-253123: p_VoiceBearerToBeEstablished to be set to false for IPCAN_EmergencyCall_Aborted sic@ */ /* @sic R5-255081: return integer for improved management of EPS bearers sic@ */ var boolean v_StartIPfor2ndPDN := p_StartIPfor2ndPDN; //WA#21_30 /* @sic R5s150412 sic@ */ var EPS_BearerIdentity v_DedicatedBearerIdSpeech; var EPS_BearerIdentity v_DedicatedBearerIdRTT; var NAS_MSG_Indication_Type v_NasInd; var integer v_NumberOfAddDedicatedBearers; // Table 4.5A.4.3-1 step 2 - 8 if (not p_ConnectionEstablished) { /* @sic R5s250249: "not" sic@ */ f_EUTRA_508_EstablishIMSEmergencyCall_Steps2_8(p_CellId); } // Table 4.5A.4.3-1 step 9 v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(p_CellId); // Table 4.5A.4.3-1 step 10 - 12 f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_12(p_CellId, p_DefaultBearerId, v_NasInd, v_StartIPfor2ndPDN); // Table 4.5A.4.3-1 step 13 - 15 select (p_ProcedureType) { case (IPCAN_EmergencyCall_Aborted) { v_NumberOfAddDedicatedBearers := 0; // no dedicated bearer to be established } case (IPCAN_EmergencyCall_WithRTT) { // @sic R5-253113: test case 19.1.7 (emergency call with RTT) sic@ v_DedicatedBearerIdSpeech := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); v_DedicatedBearerIdRTT := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId, 2); f_EUTRA_ActivateDedicatedEpsBearer_SpeechWith2ndMedia(p_CellId, RTT, p_DefaultBearerId, v_DedicatedBearerIdSpeech, v_DedicatedBearerIdRTT, PDN_2); v_NumberOfAddDedicatedBearers := 2; } case else { v_DedicatedBearerIdSpeech := f_EUTRA_IncrementEpsBearerId(p_DefaultBearerId); f_EUTRA_ActivateDedicatedEpsBearer_Speech(p_CellId, p_DefaultBearerId, v_DedicatedBearerIdSpeech, PDN_2); v_NumberOfAddDedicatedBearers := 1; } } return v_NumberOfAddDedicatedBearers; }

## Change 2

| Function name | f_EUTRA_IPCAN_StartProcedure |
| --- | --- |
| Reason for change | Please see change 1 above. |
| Summary of change | For the case IPCAN_EmergencyCall_Aborted, called v_NumberOfAddDedicatedBearers := f_EUTRA_IPCAN_EmergencyCall(p_CellId, p_ProcedureType, v_NextBearerId, p_StateInfo.ConnectionEstablished, true /*WA#21_30*/); instead of v_NumberOfAddDedicatedBearers := f_EUTRA_IPCAN_EmergencyCall(p_CellId, p_ProcedureType, v_NextBearerId, p_StateInfo.ConnectionEstablished); |
| TTCN module | IPCAN_EUTRA.ttcn |

Before Change:

function f_EUTRA_IPCAN_StartProcedure(IMS_IPCAN_CO_ORD_PORT p_Port, EUTRA_IPCAN_StateInfo_Type p_StateInfo, PDN_Index_Type p_PdnIndex, EUTRA_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType) runs on EUTRA_PTC return EUTRA_IPCAN_StateInfo_Type { /* @sic R5s130681: DISCARD_UM_DATA sic@ */ /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5s150412: new parameter p_IsEmergencyCallConfiguration, R5s150900 p_IsEmergencyCallConfiguration removed again sic@ */ /* @sic R5-255081: p_StateInfo for improved management of EPS bearers sic@ */ var EPS_BearerIdentity v_NextBearerId := fl_EUTRA_IPCAN_StateInfo_NextAvailableEpsBearerId(p_StateInfo); var EPS_BearerIdentity v_DedicatedBearer; var integer v_NumberOfAddDefaultBearers := 0; var integer v_NumberOfAddDedicatedBearers := 0; select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // initial registration if (f_GetTestcaseAttrib_Eutra_DualIMSRegistration(testcasename())) { // @sic R5-176254 sic@ f_EUTRA_IPCAN_Dual_IMSRegistration(p_CellId); v_NumberOfAddDefaultBearers := 2; } else { f_EUTRA_IPCAN_InitialRegistration(p_CellId); v_NumberOfAddDefaultBearers := 1; } } case (IPCAN_EmergencyCall_NormalService, // emergency call, normal service, uses 112 as default emg number IPCAN_EmergencyCall_OptionalPDNRelease, // @sic R5-177122: same as IPCAN_EmergencyCall_NormalService for StartProcedure sic@ IPCAN_EmergencyCall_Aborted, // @sic R5-253122, R5-253123: eCall gets aborted => no dedicated bearer needed sic@ IPCAN_EmergencyCall_WithRTT) { // @sic R5-254917: emergency call configured with RTT sic@ v_NumberOfAddDefaultBearers := 1; v_NumberOfAddDedicatedBearers := f_EUTRA_IPCAN_EmergencyCall(p_CellId, p_ProcedureType, v_NextBearerId, p_StateInfo.ConnectionEstablished); } ..SKIPPED CODE>>

After Change:

function f_EUTRA_IPCAN_StartProcedure(IMS_IPCAN_CO_ORD_PORT p_Port, EUTRA_IPCAN_StateInfo_Type p_StateInfo, PDN_Index_Type p_PdnIndex, EUTRA_CellId_Type p_CellId, IMS_TestProcedure_Type p_ProcedureType) runs on EUTRA_PTC return EUTRA_IPCAN_StateInfo_Type { /* @sic R5s130681: DISCARD_UM_DATA sic@ */ /* @sic R5s140490 change 7 MCC160 implementation: p_ConnectionEstablished sic@ */ /* @sic R5s150412: new parameter p_IsEmergencyCallConfiguration, R5s150900 p_IsEmergencyCallConfiguration removed again sic@ */ /* @sic R5-255081: p_StateInfo for improved management of EPS bearers sic@ */ var EPS_BearerIdentity v_NextBearerId := fl_EUTRA_IPCAN_StateInfo_NextAvailableEpsBearerId(p_StateInfo); var EPS_BearerIdentity v_DedicatedBearer; var integer v_NumberOfAddDefaultBearers := 0; var integer v_NumberOfAddDedicatedBearers := 0; select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // initial registration if (f_GetTestcaseAttrib_Eutra_DualIMSRegistration(testcasename())) { // @sic R5-176254 sic@ f_EUTRA_IPCAN_Dual_IMSRegistration(p_CellId); v_NumberOfAddDefaultBearers := 2; } else { f_EUTRA_IPCAN_InitialRegistration(p_CellId); v_NumberOfAddDefaultBearers := 1; } } case (IPCAN_EmergencyCall_NormalService, // emergency call, normal service, uses 112 as default emg number IPCAN_EmergencyCall_OptionalPDNRelease, // @sic R5-177122: same as IPCAN_EmergencyCall_NormalService for StartProcedure sic@ IPCAN_EmergencyCall_Aborted, // @sic R5-253122, R5-253123: eCall gets aborted => no dedicated bearer needed sic@ IPCAN_EmergencyCall_WithRTT) { // @sic R5-254917: emergency call configured with RTT sic@ v_NumberOfAddDefaultBearers := 1; v_NumberOfAddDedicatedBearers := f_EUTRA_IPCAN_EmergencyCall(p_CellId, p_ProcedureType, v_NextBearerId, p_StateInfo.ConnectionEstablished, true /*WA#21_30*/); } <<SKIPPED CODE>>

## Change 3

| Function name | f_EUTRA_IPCAN_ConnectionRelease |
| --- | --- |
| Reason for change | In the current TTCN implementation after the eCall is over the rrc connection is released. At this time, all the DRBs are released (including the one associated with EPS bearer ID 6). After the successful PSAP callback, when the function f_EUTRA_IPCAN_ConnectionRelease is called, for the case IPCAN_MT_SpeechCall IPCAN_PSAPCallback the TTCN assigns : case (IPCAN_PSAPCallback) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration): gets released to avoid unexpected signalling in the postamble */ } Later in the function f_EUTRA_IPCAN_ConnectionRelease f_EUTRA_IPCAN_DeactivateEPSBearers is called, this results in an error to release the DRB associated with EPS bearer ID 6, as this has already been released. This needs to be addressed. Also, after the EPS bearers are deactivated after releasing the emergency call, the PDN_2 needs to be stopped, so that the IP routing for PSAP call could be reconfigured on PDN_1. |
| Summary of change | Commented out the assignment v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; Called f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); Please see below. |
| TTCN module | IPCAN_EUTRA.ttcn |

Before Change:

function f_EUTRA_IPCAN_ConnectionRelease(IMS_IPCAN_CO_ORD_PORT p_Port, EUTRA_IPCAN_StateInfo_Type p_StateInfo, EUTRA_CellId_Type p_CellId, IMS_TestConfiguration_Type p_TestConfiguration, IMS_TestProcedure_Type p_ProcedureType) runs on EUTRA_PTC return EUTRA_IPCAN_StateInfo_Type { /* @sic R5-145787: new parameter p_IsXCAP sic@ */ /* NOTE: p_IsXCAP is indepedent from p_ProcedureType */ /* @sic R5s150338: v_DrbConfigList removed sic@ */ /* @sic R5s150900: p_IsEmergencyCallConfiguration sic@ */ /* @sic R5s160247: p_IsXCAP and p_IsEmergencyCallConfiguration replaced by p_TestConfiguration sic@ */ /* @sic R5-255081: p_StateInfo for improved management of EPS bearers sic@ */ var EPS_BearerIdentityList v_DedicatedBearerIdList := {}; var EPS_BearerIdentityList v_DefaultBearerIdList := {}; var boolean v_IsXCAP := (p_TestConfiguration == IPCAN_XCAP); /* @sic R5-145787: we need to know when it is an XCAP case sic@ */ var boolean v_Keep2ndPDN := v_IsXCAP and pc_XCAP_over_Internet_APN; var boolean v_ReleaseInternetPDN := false; var integer v_IncrDefault; var integer v_IncrDedicated; select (p_ProcedureType) { case (IPCAN_InitialRegistration) { v_ReleaseInternetPDN := true; } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling, IPCAN_EmergencyCall_Aborted) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService, IPCAN_ECallOnly) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H @sic R5-172067: explicit release of dedicated EPS bearer sic@ */ if (p_TestConfiguration != IPCAN_PSAPCallback) { /* In case of PSAP callback the emergency PDN shall be kept otherwise it can be released as postamble */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration): When there is no PSAP callback the connection release happens at the very end of the test case and the emergency PDN is released to avoid unexpected signalling in the postamble */ } } case (IPCAN_EmergencyCall_WithRTT) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId3, tsc_EpsDedicatedBearerId2 }; /* '8'H (RTT), '7'H (speech) */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration) */ } case (IPCAN_EmergencyCall_OptionalPDNRelease) { /* @sic R5-177122 sic@ */ // TS 34.229-1 Annex C.32a Steps 6-7 // @sic R5-187683 sic@ - f_EUTRA_IPCAN_DeactivateEPSBearer(p_CellId, tsc_EpsDedicatedBearerId2); f_EUTRA_DeactivateEPS_BearerContext_Optional_DiscReq(p_CellId, tsc_EpsDefaultBearerId2); /* release of default bearer according to 4.5A.15A */ } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall, IPCAN_ECallOnly_TestCall) { if (p_StateInfo.NumberOfDedicatedBearersInUse > 0) { /* NOTE: in test cases for unsuccessful behaviour the EPS bearer may not be established */ select (p_TestConfiguration) { case (IPCAN_SpeechAndEmergencyCall) { /* @sic R5s160247 sic@ */ v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId3, tsc_EpsDedicatedBearerId }; /* '8'H (emergency call), '6'H (speech call); * default bearers are at '5'H (normal registration), '7'H (emergency registration) * NOTE: In IMS_EUTRA there is only TC_19_1_5 with TestConfiguration IPCAN_SpeechAndEmergencyCall * which puts an ongoing call on hold to make an emergency call but when the emergency call is released * the dedicated EPS bearer of the emergency call is kept until release of the normal call * (i.e. there is no IPCAN_ENDPROCEDURE command for the emergency call) */ } case (IPCAN_EmergencyCall) { /* @sic R5-251274, R5-251275: test cases 21.25/26 sic@ */ v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H */ } case (IPCAN_PSAPCallback) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration): gets released to avoid unexpected signalling in the postamble */ } case else { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId }; /* '6'H */ } } } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { // release dedicated bearers v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId, tsc_EpsDedicatedBearerId2 }; /* '6'H (speech), '7'H (video) */ } case (IPCAN_XCAP_Signalling) { /* @sic R5s141051 change 7: release the 2ndPDN sic@ */ if (not pc_XCAP_over_Internet_APN) { /* @sic R5s160834 sic@ */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2ndPDN }; /* @sic R5-145787 sic@ For XCAP TCs, 2nd PDN needs to be released */ } else { v_ReleaseInternetPDN := true; /* @sic R5s190722 - MCC160 Implementation sic@ */ } } } if (p_ProcedureType != IPCAN_XCAP_Signalling) { /* XCAP is special case: PDN is mapped to tsc_EpsDefaultBearerId2ndPDN ('C'H) */ v_IncrDefault := - lengthof(v_DefaultBearerIdList); v_IncrDedicated := - lengthof(v_DedicatedBearerIdList); fl_EUTRA_IPCAN_StateInfo_IncrNumberOfBearers(p_StateInfo, v_IncrDefault, v_IncrDedicated); } f_EUTRA_IPCAN_DeactivateEPSBearers(p_CellId, v_DedicatedBearerIdList & v_DefaultBearerIdList); f_EUTRA_RRC_ConnectionRelease(p_CellId); /* release RRC connection; @sic R5s140092 change 13: v_DrbConfigList sic@ @sic R5s150338: no v_DrbReleaseList anymore sic@ */ p_StateInfo.ConnectionEstablished := false; if (v_ReleaseInternetPDN) { if (f_GetTestcaseAttrib_Eutra_DualIMSRegistration(testcasename())) {// @sic R5-176254 sic@ f_EUTRA_InitialRegistration_ReleaseInternetPDN(p_CellId, tsc_EpsDefaultBearerId2); } else if (tsc_EUTRA_Registration_MultiplePDN and not v_Keep2ndPDN) { // @sic R5-145787 sic@ f_EUTRA_InitialRegistration_ReleaseInternetPDN(p_CellId); // @sic R5-160947 sic@ } } if (p_ProcedureType==IPCAN_InitialRegistration) { select (p_TestConfiguration) { /* @sic R5s150900, R5s160247 sic@ */ case (IPCAN_EmergencyCall) { f_EUTRA_StartIPfor2ndPDN(p_CellId, PDN_2, tsc_DRB2); } case (IPCAN_SpeechAndEmergencyCall) { f_EUTRA_StartIPfor2ndPDN(p_CellId, PDN_2, tsc_DRB3); } } } f_IMS_IPCAN_SendCoOrdMsg(p_Port); return p_StateInfo; }

After Change:

function f_EUTRA_IPCAN_ConnectionRelease(IMS_IPCAN_CO_ORD_PORT p_Port, EUTRA_IPCAN_StateInfo_Type p_StateInfo, EUTRA_CellId_Type p_CellId, IMS_TestConfiguration_Type p_TestConfiguration, IMS_TestProcedure_Type p_ProcedureType) runs on EUTRA_PTC return EUTRA_IPCAN_StateInfo_Type { /* @sic R5-145787: new parameter p_IsXCAP sic@ */ /* NOTE: p_IsXCAP is indepedent from p_ProcedureType */ /* @sic R5s150338: v_DrbConfigList removed sic@ */ /* @sic R5s150900: p_IsEmergencyCallConfiguration sic@ */ /* @sic R5s160247: p_IsXCAP and p_IsEmergencyCallConfiguration replaced by p_TestConfiguration sic@ */ /* @sic R5-255081: p_StateInfo for improved management of EPS bearers sic@ */ var EPS_BearerIdentityList v_DedicatedBearerIdList := {}; var EPS_BearerIdentityList v_DefaultBearerIdList := {}; var boolean v_IsXCAP := (p_TestConfiguration == IPCAN_XCAP); /* @sic R5-145787: we need to know when it is an XCAP case sic@ */ var boolean v_Keep2ndPDN := v_IsXCAP and pc_XCAP_over_Internet_APN; var boolean v_ReleaseInternetPDN := false; var integer v_IncrDefault; var integer v_IncrDedicated; select (p_ProcedureType) { case (IPCAN_InitialRegistration) { v_ReleaseInternetPDN := true; } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling, IPCAN_EmergencyCall_Aborted) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService, IPCAN_ECallOnly) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H @sic R5-172067: explicit release of dedicated EPS bearer sic@ */ if (p_TestConfiguration != IPCAN_PSAPCallback) { /* In case of PSAP callback the emergency PDN shall be kept otherwise it can be released as postamble */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration): When there is no PSAP callback the connection release happens at the very end of the test case and the emergency PDN is released to avoid unexpected signalling in the postamble */ } } case (IPCAN_EmergencyCall_WithRTT) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId3, tsc_EpsDedicatedBearerId2 }; /* '8'H (RTT), '7'H (speech) */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration) */ } case (IPCAN_EmergencyCall_OptionalPDNRelease) { /* @sic R5-177122 sic@ */ // TS 34.229-1 Annex C.32a Steps 6-7 // @sic R5-187683 sic@ - f_EUTRA_IPCAN_DeactivateEPSBearer(p_CellId, tsc_EpsDedicatedBearerId2); f_EUTRA_DeactivateEPS_BearerContext_Optional_DiscReq(p_CellId, tsc_EpsDefaultBearerId2); /* release of default bearer according to 4.5A.15A */ } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall, IPCAN_ECallOnly_TestCall) { if (p_StateInfo.NumberOfDedicatedBearersInUse > 0) { /* NOTE: in test cases for unsuccessful behaviour the EPS bearer may not be established */ select (p_TestConfiguration) { case (IPCAN_SpeechAndEmergencyCall) { /* @sic R5s160247 sic@ */ v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId3, tsc_EpsDedicatedBearerId }; /* '8'H (emergency call), '6'H (speech call); * default bearers are at '5'H (normal registration), '7'H (emergency registration) * NOTE: In IMS_EUTRA there is only TC_19_1_5 with TestConfiguration IPCAN_SpeechAndEmergencyCall * which puts an ongoing call on hold to make an emergency call but when the emergency call is released * the dedicated EPS bearer of the emergency call is kept until release of the normal call * (i.e. there is no IPCAN_ENDPROCEDURE command for the emergency call) */ } case (IPCAN_EmergencyCall) { /* @sic R5-251274, R5-251275: test cases 21.25/26 sic@ */ v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H */ } case (IPCAN_PSAPCallback) { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId2 }; /* '7'H */ //WA#21_30 COMMENTED OUT v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2 }; /* '6'H (emergency registration): gets released to avoid unexpected signalling in the postamble */ } case else { v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId }; /* '6'H */ } } } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { // release dedicated bearers v_DedicatedBearerIdList := { tsc_EpsDedicatedBearerId, tsc_EpsDedicatedBearerId2 }; /* '6'H (speech), '7'H (video) */ } case (IPCAN_XCAP_Signalling) { /* @sic R5s141051 change 7: release the 2ndPDN sic@ */ if (not pc_XCAP_over_Internet_APN) { /* @sic R5s160834 sic@ */ v_DefaultBearerIdList := { tsc_EpsDefaultBearerId2ndPDN }; /* @sic R5-145787 sic@ For XCAP TCs, 2nd PDN needs to be released */ } else { v_ReleaseInternetPDN := true; /* @sic R5s190722 - MCC160 Implementation sic@ */ } } } if (p_ProcedureType != IPCAN_XCAP_Signalling) { /* XCAP is special case: PDN is mapped to tsc_EpsDefaultBearerId2ndPDN ('C'H) */ v_IncrDefault := - lengthof(v_DefaultBearerIdList); v_IncrDedicated := - lengthof(v_DedicatedBearerIdList); fl_EUTRA_IPCAN_StateInfo_IncrNumberOfBearers(p_StateInfo, v_IncrDefault, v_IncrDedicated); } f_EUTRA_IPCAN_DeactivateEPSBearers(p_CellId, v_DedicatedBearerIdList & v_DefaultBearerIdList); if ((p_ProcedureType == IPCAN_EmergencyCall_NormalService)){//WA#21_30 f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); } f_EUTRA_RRC_ConnectionRelease(p_CellId); /* release RRC connection; @sic R5s140092 change 13: v_DrbConfigList sic@ @sic R5s150338: no v_DrbReleaseList anymore sic@ */ p_StateInfo.ConnectionEstablished := false; if (v_ReleaseInternetPDN) { if (f_GetTestcaseAttrib_Eutra_DualIMSRegistration(testcasename())) {// @sic R5-176254 sic@ f_EUTRA_InitialRegistration_ReleaseInternetPDN(p_CellId, tsc_EpsDefaultBearerId2); } else if (tsc_EUTRA_Registration_MultiplePDN and not v_Keep2ndPDN) { // @sic R5-145787 sic@ f_EUTRA_InitialRegistration_ReleaseInternetPDN(p_CellId); // @sic R5-160947 sic@ } } if (p_ProcedureType==IPCAN_InitialRegistration) { select (p_TestConfiguration) { /* @sic R5s150900, R5s160247 sic@ */ case (IPCAN_EmergencyCall) { f_EUTRA_StartIPfor2ndPDN(p_CellId, PDN_2, tsc_DRB2); } case (IPCAN_SpeechAndEmergencyCall) { f_EUTRA_StartIPfor2ndPDN(p_CellId, PDN_2, tsc_DRB3); } } } f_IMS_IPCAN_SendCoOrdMsg(p_Port); return p_StateInfo; }

# Branches executed

The test case was executed on on EUTRA band 1 using ciphering algorithm eea1 and integrity algorithm eia1 on Qualcomm Snapdragon Auto 5G Modem-RF UE.

# Execution Log Files

## Qualcomm Snapdragon Auto 5G Modem-RF

The Qualcomm Snapdragon Auto 5G Modem-RF UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_30_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s250458: Supporting information for Addition of IMS eCall over LTE CEN test case 21.30 |
| --- | --- |