3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250470

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4943 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SSAC test case 13.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In current TTCN, while implementing prose-CR R5-255062, it seems the cell attenuation, right before initial switch on, got removed. - Incorrect Cell configured with CellA, not aligned with prose - v_PcoFromUE is not initialized with the corresponding IE contents in PDN_CONNECTIVITY_REQUEST message |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added call to function f_TC_13_5_1_EUTRA() to the set the initial cell power. - Updated the cell configuration by replacing Cell A with Cell1. - Added the missing initialization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_13_5_1_EUTRA() |
| --- | --- |
| Reason for change | In current TTCN, while implementing prose-CR R5-255062, it seems the cell attenuation, right before initial switch on, got removed. |
| Summary of change | Added call to function f_TC_13_5_1_EUTRA() to the set the initial cell power. |
| TTCN module | AccessControl.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_13_5_1_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SSAC / 0% access probability for MTSI MO speech call var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); // @sic R5-255062 sic@ f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (f_GetDefaultValueNetworkFeatureSupport()); // Now move to Idle Updated f_RegistrationCSnotAvailable(eutra_Cell1, PREAMBLE); // @sic R5-255062 sic@ f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @sic R5s230790 sic@

After Change:

f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); // First switch cell back on f_EUTRA_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE); // @sic R5-255062 sic@ f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (f_GetDefaultValueNetworkFeatureSupport()); // Now move to Idle Updated f_RegistrationCSnotAvailable(eutra_Cell1, PREAMBLE); // @sic R5-255062 sic@ f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @sic R5s230790 sic@

## Change 2

| Function name | function f_TC_13_5_1_EUTRA() |
| --- | --- |
| Reason for change | Incorrect Cell configured with CellA, not aligned with prose |
| Summary of change | Updated the cell configuration by replacing Cell A with Cell1 |
| TTCN module | AccessControl.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_13_5_1_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SSAC / 0% access probability for MTSI MO speech call var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); // @sic R5-255062 sic@ f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (f_GetDefaultValueNetworkFeatureSupport()); // Now move to Idle Updated f_RegistrationCSnotAvailable(eutra_Cell1, PREAMBLE); // @sic R5-255062 sic@ f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @sic R5s230790 sic@ //@siclog "Steps 10-15 Void" siclog@ @sic R5-155175 sic@ //@siclog "Steps 16-17A" siclog@ f_IMS_EmergencyCallReleaseMO(v_InviteRequest); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN);

After Change:

function f_TC_13_5_1_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SSAC / 0% access probability for MTSI MO speech call var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); // @sic R5-255062 sic@ f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (f_GetDefaultValueNetworkFeatureSupport()); // Now move to Idle Updated f_RegistrationCSnotAvailable(eutra_Cell1, PREAMBLE); // @sic R5-255062 sic@ f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @sic R5s230790 sic@

## Change 3

| Function name | function f_RegistrationCSnotAvailable |
| --- | --- |
| Reason for change | v_PcoFromUE is not initialized with the corresponding IE contents in PDN_CONNECTIVITY_REQUEST message |
| Summary of change | Added the missing initialization. |
| TTCN module | EUTRA_NASCommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_RegistrationCSnotAvailable(EUTRA_CellId_Type p_CellId, IDLEUPDATED_Type p_Type) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var APNandPCOs_Type v_APNandPCOs; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE; var template (value) ProtocolConfigOptions v_PcoToUE; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (omit) ESM_Cause v_ESM_Cause := omit; v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, p_Type, NORMAL); v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_EPS_TI := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses // Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6) if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){ v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE);

After Change:

function f_RegistrationCSnotAvailable(EUTRA_CellId_Type p_CellId, IDLEUPDATED_Type p_Type) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var APNandPCOs_Type v_APNandPCOs; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE; var template (value) ProtocolConfigOptions v_PcoToUE; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (omit) ESM_Cause v_ESM_Cause := omit; v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, p_Type, NORMAL); v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_EPS_TI := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses v_PcoFromUE := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; // Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6) if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){ v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE);