3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250203

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3687 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-17 NR NTN test case 6.7.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 6.7.1.1 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.7.1.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

2.4	Change 4	10

3	Branches executed	11

4	Execution Log Files	11

4.1	Samsung Exynos 2500	11

5	References	12

[1] R5s250204: Supporting information for addition of NR-NTN testcase 6.7.1.1	12

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.1.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.7.1.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_6_7_1_1_NR5GC() |
| --- | --- |
| Reason for change | UE’s initial position should be set/preconfigured in order for it to successfully complete Preamble step – Required Prose CR submitted in support. |
| Summary of change | Add preconfigure_NTN_UE call to configure UE location before preamble. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comments | Accepted in principle and conditional to prose CR agreement at RAN5#108. See MCC160 Implementation below. With MCC160 change proposed along with prose CR to be submitted, UE will met preconditions (no UE location preconfigured) without the necessity of additional signaling before starting the test procedure sequence. |

Before Change:

function f_TC_6_7_1_1_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / GNSS location // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_28); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B, -, -, -, -, -, {NoNTN}); f_NR_TestBody_Set(true); fl_TC_6_7_1_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change:

function f_TC_6_7_1_1_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / GNSS location // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_28); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_PreconfigureNTN_UE(); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B, -, -, -, -, -, {NoNTN}); f_NR_TestBody_Set(true); fl_TC_6_7_1_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

MCC160 implementation

function f_TC_6_7_1_1_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / GNSS location // System information combination NR-29 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_29); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_UT_SwitchOnUE(UT, false); //Make sure that No UE location is pre-configured f_UT_ConfigureNTNPosition(UT,"No UE location is pre-configured"); //Now Power OFF the UE f_UT_PowerOffUE (UT); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0A, -, -, -, -, -, {NoNTN}); f_NR_TestBody_Set(true); fl_TC_6_7_1_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

## Change 2

| Function name | fl_TC_6_7_1_1_Body () |
| --- | --- |
| Reason for change | As per preamble condition UE must reach the Switch off state (state 0N-B) and therefore has deregistered. Hence, at Step 6 UE will attempt initial registration and not send Service Request. Hence, steps from 38.508-1 4.5.2.2 are applicable.-Required Prose CR submitted in support. |
| Summary of change | - Add Step to send AT Command to disable UE positioning engine/remove preconfigured position. - Modify step 6 to use proper steps for verification of UE’s connection on cell after UE position is configured. |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comments | - Not needed. MCC160 implementation for change 1, will set the expected state UE with no location pre-configured for pretest conditions. - Accepted in principle, but implemented differently. Please see implementation below of fl_TC_6_7_1_1_Body. Conditional to prose CR agreement at RAN5#108. |

Before Change:

function fl_TC_6_7_1_1_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; timer t_WaitRRCSetupRequest; // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell1, 360.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 1 within 6 minutess"); // @siclog "Step 3" siclog@ // Power Off the UE f_UT_SwitchOffUE(UT, false); // @siclog "Step 4" siclog@ // Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) f_NR5GC_SwitchOnAndResetIMS(); f_NR_PreconfigureNTN_UE(); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 360.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5: No RRCSetupRequest received within 6 minutes"); } } // @siclog "Step 6-11" siclog@ // Steps 3 to 8 of the registration procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_6_7_1_1_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); timer t_WaitRRCSetupRequest; f_UT_MMI_ProvideLocationInfoToUE(UT); // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell1, 360.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 1 within 6 minutess"); // @siclog "Step 3" siclog@ // Power Off the UE f_UT_SwitchOffUE(UT, false); // @siclog "Step 4" siclog@ // Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) f_NR5GC_SwitchOnAndResetIMS(); f_NR_PreconfigureNTN_UE(); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 360.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5: No RRCSetupRequest received within 6 minutes"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); // @siclog "Step 6-11" siclog@ // Steps 3 to 8 of the registration procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_NoSecurityProtection); //f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NasInd.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_NasInd, v_PLMN, TESTMode_OFF, Initial_NoSecurity); f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_NasInd, TESTMode_OFF, -, Initial_NoSecurity); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. //f_NR_RRCRelease(nr_Cell1); }

## Change 3

| Function name | fl_TC_6_7_1_1_Body() |
| --- | --- |
| Reason for change | At Step 6 UL grant was not started by TTCN. Hence, TTCN needs to configure UL grants after Step 6 in order to perform registration procedure. |
| Summary of change | Added f_NR_ULGrantConfiguration_Start after rrcSetupRequest is received. |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comments | Accepted in principle, but implemented differently. Please see implementation below of fl_TC_6_7_1_1_Body. |

Before Change:

function fl_TC_6_7_1_1_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; timer t_WaitRRCSetupRequest; // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell1, 360.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 1 within 6 minutess"); // @siclog "Step 3" siclog@ // Power Off the UE f_UT_SwitchOffUE(UT, false); // @siclog "Step 4" siclog@ // Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) f_NR5GC_SwitchOnAndResetIMS(); f_NR_PreconfigureNTN_UE(); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 360.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5: No RRCSetupRequest received within 6 minutes"); } } // @siclog "Step 6-11" siclog@ // Steps 3 to 8 of the registration procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); }

After Change:

function fl_TC_6_7_1_1_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); timer t_WaitRRCSetupRequest; f_UT_MMI_ProvideLocationInfoToUE(UT); // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell1, 360.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 1 within 6 minutess"); // @siclog "Step 3" siclog@ // Power Off the UE f_UT_SwitchOffUE(UT, false); // @siclog "Step 4" siclog@ // Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) f_NR5GC_SwitchOnAndResetIMS(); f_NR_PreconfigureNTN_UE(); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 360.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5: No RRCSetupRequest received within 6 minutes"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); // @siclog "Step 6-11" siclog@ // Steps 3 to 8 of the registration procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_NoSecurityProtection); //f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NasInd.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_NasInd, v_PLMN, TESTMode_OFF, Initial_NoSecurity); f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_NasInd, TESTMode_OFF, -, Initial_NoSecurity); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. //f_NR_RRCRelease(nr_Cell1); }

MCC160 implementation

function fl_TC_6_7_1_1_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); timer t_WaitRRCSetupRequest; // @siclog "Step 1" siclog@ // Power on the UE f_NR5GC_SwitchOnAndResetIMS(); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell1, 360.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 1 within 6 minutess"); // @siclog "Step 3" siclog@ // Power Off the UE f_UT_SwitchOffUE(UT, false); // @siclog "Step 4" siclog@ // Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) f_NR5GC_SwitchOnAndResetIMS(); f_NR_PreconfigureNTN_UE(); // @siclog "Step 5" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? t_WaitRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, 360.0)); alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_WaitRRCSetupRequest.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5"); } [] t_WaitRRCSetupRequest.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.7.1.1 Step 5: No RRCSetupRequest received within 6 minutes"); } } f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); // @siclog "Step 6-23" siclog@ // Steps 3 to 20 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_NasInd, TESTMode_OFF, -, Initial_NoSecurity); }

## Change 4

| Function name | f_TC_6_7_1_1_NR5GC() |
| --- | --- |
| Reason for change | Apply changes as a result of CR R5-252351 |
| Summary of change | Modify to use System Combination NR-29 |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comments | Accepted |

Before Change:

function f_TC_6_7_1_1_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / GNSS location // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_28); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_PreconfigureNTN_UE(); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B, -, -, -, -, -, {NoNTN}); f_NR_TestBody_Set(true); fl_TC_6_7_1_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change:

function f_TC_6_7_1_1_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / GNSS location // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_29); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_PreconfigureNTN_UE(); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B, -, -, -, -, -, {NoNTN}); f_NR_TestBody_Set(true); fl_TC_6_7_1_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

## Additional Change from MCC160

| Function name | f_NR5GC_RRC_Idle_Steps5_9_AKA() |
| --- | --- |
| Reason for change | To avoid potential runtime error if GMM_MobilityInfo_Type has not been initialised |
| Summary of change | If GMM_MobilityInfo_Type is not a value call f_Check_NG_RegistrationReqMsg |
| TTCN module | NR5GC_NASSteps.ttcn |

function f_NR5GC_RRC_Idle_Steps5_9_AKA (inout GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type,

NR_CellId_Type p_NR_CellId,

NG_NAS_MSG_Indication_Type p_ReceivedMsg,

NAS_PlmnId p_PLMN,

TypeOfRegistration_Type p_RegType := Initial_Secure,

InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED,

integer p_AssignmentMode := 1

)  runs on NR5GC_PTC return NasCount_Type

{

var template NAS_SecurityReturnType v_NAS_SecurityReturnValue;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg := p_ReceivedMsg;

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var SecurityHeaderType v_SentSecurityHeader;

var template (present) SecurityHeaderType v_SecurityHeaderResponse;

if (not isvalue(p_GMM_MobilityInfo_Type)) {

// if this function hasn't already been called, call it now to fill p_GMM_MobilityInfo_Type

f_Check_NG_RegistrationReqMsg (p_GMM_MobilityInfo_Type, v_ReceivedMsg.Pdu, Initial_NoSecurity);

}

if(p_RegType == Initial_NoSecurity) { // @sic R5s190109 sic@

v_SentSecurityHeader := tsc_SHT_NoSecurityProtection;

v_SecurityHeaderResponse := tsc_SHT_NoSecurityProtection;

} else {

v_SentSecurityHeader := tsc_SHT_IntegrityProtected_Ciphered;

v_SecurityHeaderResponse := tsc_SHT_IntegrityProtected_Ciphered; // @sic R5s190109 sic@

}

...

}

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms on Anritsu 5G Conformance Test platform in GSO.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_7_1_1_LOG.xml

- PICS/PIXIT parameter file:

TC_6_7_1_1_PIXIT.txt

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

## [1] R5s250204: Supporting information for addition of NR-NTN testcase 6.7.1.1