3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250337

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0983 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to emergency call test case 19.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current TTCN implementation the Global release function at step 16-17 is releasing the secured ports which are further required at steps 26-30(IMS emergency call setup with PSAP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new parameter in function f_IMS_EmergencyCallReleaseMO and f_IMS_GlobalRelease and added handling in f_IMS_GlobalRelease to skip security release for this test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: additional change 1 |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_IMS_EmergencyCallReleaseMO |
| --- | --- |
| Reason for change | In current TTCN implementation the Global release function at step 16-17 is releasing the secured ports which are further required at steps 26-30(IMS emergency call setup with PSAP). |
| Summary of change | Added a new parameter p_withGlobalRelease in function f_IMS_EmergencyCallReleaseMO to avoid security release |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment | Accepted in principle. New parameter named p_WithGlobalRelease. MCC160 implementation see below. |

Before Change:

function f_IMS_EmergencyCallReleaseMO(INVITE_Request p_InviteRequest, boolean p_AutonomousCallRelease := false) runs on IMS_PTC { /* @sic R5s141286 change 2: new parameter p_AutonomousCallRelease sic@ */ f_IMS_CallReleaseMO(p_InviteRequest, -, REGULAR_EMERGENCY, p_AutonomousCallRelease); f_IMS_GlobalRelease(tsc_IMS_WaitForTcpClose); /* @sic R5s130988 change 1 - MCC160 implementation sic@ * @sic R5s140219 - additional changes sic@ * NOTE: for emergency call there is no user-initiated emergency deregistration (see 24.229 cl. 5.1.6.6); * nevertheless as working assumption the UE is expected to close its TCP (client) connection and if not it is closed by the IP_PTC; * this may cause issues when - for what reason ever - the UE wants to re-open the TCP connction ("WAIT TIME" acc. to RFC 793 clause 3.2, figure 6; see R5w140110); * but even if we would keep the UE's client connection any SS' client connection shall be closed (currently we don't distinguish these but close all) * @sic R5s250228 change 2.1.7: f_IMS_GlobalRelease instead of f_IMS_TCP_CloseHandling sic@ */ }

After Change:

function f_IMS_EmergencyCallReleaseMO(INVITE_Request p_InviteRequest, boolean p_AutonomousCallRelease := false, boolean p_withGlobalRelease := true) runs on IMS_PTC { /* @sic R5s141286 change 2: new parameter p_AutonomousCallRelease sic@ */ f_IMS_CallReleaseMO(p_InviteRequest, -, REGULAR_EMERGENCY, p_AutonomousCallRelease); f_IMS_GlobalRelease(tsc_IMS_WaitForTcpClose, -, - , p_withGlobalRelease); /* @sic R5s130988 change 1 - MCC160 implementation sic@ * @sic R5s140219 - additional changes sic@ * NOTE: for emergency call there is no user-initiated emergency deregistration (see 24.229 cl. 5.1.6.6); * nevertheless as working assumption the UE is expected to close its TCP (client) connection and if not it is closed by the IP_PTC; * this may cause issues when - for what reason ever - the UE wants to re-open the TCP connction ("WAIT TIME" acc. to RFC 793 clause 3.2, figure 6; see R5w140110); * but even if we would keep the UE's client connection any SS' client connection shall be closed (currently we don't distinguish these but close all) * @sic R5s250228 change 2.1.7: f_IMS_GlobalRelease instead of f_IMS_TCP_CloseHandling sic@ */ }

MCC160 Implementation:

The test case needs is a pure call release at steps 16-17A. The delay allowing the UE to release its TCP connections before release of the security contexts is not needed. See change 3.

function f_IMS_EmergencyCallReleaseMO(INVITE_Request p_InviteRequest, boolean p_AutonomousCallRelease := false, boolean p_WithGlobalRelease := true) runs on IMS_PTC { /* @sic R5s141286 change 2: new parameter p_AutonomousCallRelease sic@ */ f_IMS_CallReleaseMO(p_InviteRequest, -, REGULAR_EMERGENCY, p_AutonomousCallRelease); if (p_WithGlobalRelease == true) { f_IMS_GlobalRelease(tsc_IMS_WaitForTcpClose); /* @sic R5s130988 change 1 - MCC160 implementation sic@ * @sic R5s140219 - additional changes sic@ * NOTE: for emergency call there is no user-initiated emergency deregistration (see 24.229 cl. 5.1.6.6); * nevertheless as working assumption the UE is expected to close its TCP (client) connection and if not it is closed by the IP_PTC; * this may cause issues when - for what reason ever - the UE wants to re-open the TCP connction ("WAIT TIME" acc. to RFC 793 clause 3.2, figure 6; see R5w140110); * but even if we would keep the UE's client connection any SS' client connection shall be closed (currently we don't distinguish these but close all) * @sic R5s250228 change 2.1.7: f_IMS_GlobalRelease instead of f_IMS_TCP_CloseHandling sic@ */ } }

## Change 2

| Function name | function f_TC_19_3_4_IMS2 |
| --- | --- |
| Reason for change | Please see Change 1 |
| Summary of change | Passing value as ‘false’ for p_withGlobalRelease p_WithGlobalRelease parameter |
| TTCN module | IMS_CC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_19_3_4_IMS2() runs on IMS_PTC { var INVITE_Request v_InviteRequest; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); //@siclog "Steps 1-9" siclog@ @sic R5-155175 sic@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // steps 1-15 of 4.5A.4 f_IMS_EmergencyRegistration(); v_InviteRequest := f_IMS_EmergencyCallSetup(-, v_GeolocationUsage); // geolocation information to be used //@siclog "Steps 10-15 Void" siclog@ @sic R5-155175 sic@ //@siclog "Steps 16-17A" siclog@ f_IMS_EmergencyCallReleaseMO(v_InviteRequest); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN);

After Change:

function f_TC_19_3_4_IMS2() runs on IMS_PTC { var INVITE_Request v_InviteRequest; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); //@siclog "Steps 1-9" siclog@ @sic R5-155175 sic@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // steps 1-15 of 4.5A.4 f_IMS_EmergencyRegistration(); v_InviteRequest := f_IMS_EmergencyCallSetup(-, v_GeolocationUsage); // geolocation information to be used //@siclog "Steps 10-15 Void" siclog@ @sic R5-155175 sic@ //@siclog "Steps 16-17A" siclog@ f_IMS_EmergencyCallReleaseMO(v_InviteRequest, -, false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN);

## Change 3

| Function name | function f_IMS_GlobalRelease |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | Added new parameter p_withSecurityRelease and Skipped security release if this parameter is set to false. |
| TTCN module | IMS_Procedures_Common.ttcn |
| MCC160 Comment | Not needed if change 1 is implemented as proposed ! |

Before Change:

function f_IMS_GlobalRelease(float p_DelayTime := 0.0, IMS_SecurityReleaseMode_Type p_SecurityReleaseMode := full, boolean p_WaitForCNF := true) runs on IMS_PTC { /* @sic R5s170468: p_SecurityReleaseMode introduced sic@ */ /* @sic R5s221193: p_WaitForCNF; closing of TCP connections and servers shall be fully done before IP PTC sends CNF sic@ */ /* @sic R5s250228 change 2.1.2: p_DelayTime, p_WaitForCNF := true sic@ */ f_Delay(p_DelayTime); f_IMS_Register_SecurityRelease(p_SecurityReleaseMode, p_WaitForCNF); /* release registration information at IPCAN @sic R5s170231 - Additional changes: f_IMS_Register_SecurityRelease instead of f_IMS_RegisterInfo_Release sic@ */ f_IMS_PTC_Reset(); /* reset ImsInfo and release security @sic R5s150031: common function f_IMS_PTC_Reset sic@ */ }

After Change:

function f_IMS_GlobalRelease(float p_DelayTime := 0.0, IMS_SecurityReleaseMode_Type p_SecurityReleaseMode := full, boolean p_WaitForCNF := true, boolean p_withSecurityRelease := true) runs on IMS_PTC { /* @sic R5s170468: p_SecurityReleaseMode introduced sic@ */ /* @sic R5s221193: p_WaitForCNF; closing of TCP connections and servers shall be fully done before IP PTC sends CNF sic@ */ /* @sic R5s250228 change 2.1.2: p_DelayTime, p_WaitForCNF := true sic@ */ f_Delay(p_DelayTime); if(p_withSecurityRelease) { f_IMS_Register_SecurityRelease(p_SecurityReleaseMode, p_WaitForCNF); /* release registration information at IPCAN @sic R5s170231 - Additional changes: f_IMS_Register_SecurityRelease instead of f_IMS_RegisterInfo_Release sic@ */ f_IMS_PTC_Reset(); /* reset ImsInfo and release security @sic R5s150031: common function f_IMS_PTC_Reset sic@ */ } }

Additional change 1 from MCC160 to keep trackof a minor change

2 function calls replaced by an equivalent function call in IMS test case 19.5.9 in TTCN module IMS_CC_EmergencyCallTestcases

function f_TC_19_5_9_IMS1() runs on IMS_PTC { /* IMS implementation for PDN1 - Registration */ var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var SipUrl v_ContactUrl; var template (value) Reginfo_Type v_XmlMessage; var boolean v_WaitForPublish := true; /* @sic R5-155916: handling PUBLISH during registration sic@ */ f_IMS_PTC_Init(PDN_1); // @sic R5s140329 sic@ f_IMS_IPCAN_Init(IPCAN, IPCAN_EmergencyCall); // @sic R5s140329 sic@ f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_NULL); // @sic R5s250337 add. change 1 sic@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_InitialRegistration); …