3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250362

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3742 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 11.5.1 to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.5.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 11.5.1	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	11

2.4	Change 4	14

2.5	Change 5	16

2.6	Change 6	17

3	Branches executed	18

4	Execution Log Files	18

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.	18

5	References	19

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.5.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Kalahasti Narendra

Narendra.kalahasti@anritsu.com

Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.5.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Verification Status:	PASS

# Corrections required to testcase 11.5.1

## Change 1

| Function name | fl_TC_11_5_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | 1. The function f_EUTRA38_TrackingAreaUpdateFromN1 will expect by default that UE came from EUTRA cell, however in this case it was NR cell instead. 2. UE previously registered in NR shall have mapped security context during step 54 procedure. 3. Test defines power levels in step 53 for E-UTRA Cell 1 as "Non-suitable "off" cell". |
| Summary of change | 1. Corrected 3rd parameter in the f_EUTRA38_TrackingAreaUpdateFromN1 2.Function f_EUTRA38_NR5GC_S1ToN1_IdleMode is called with 2nd parameter “true” to reflect mapped security context. 3. E-UTRA Cell 1 power level changed to "Non-suitable "off" cell". |
| TTCN module | ECall_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | - Accepted. - Accepted. - Accepted. |

Before Change:

function fl_TC_11_5_1_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def); // @siclog "Step 53" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableCellRS_EPRE, tsc_NonSuitableCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); } // End of fl_TC_11_5_1_NR5GC_EUTRA_TestBody()

After Change:

function fl_TC_11_5_1_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def, omit); // @siclog "Step 53" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); } // End of fl_TC_11_5_1_NR5GC_EUTRA_TestBody()

## Change 2

| Function name | fl_TC_11_5_1_Body |
| --- | --- |
| Reason for change | - Procedure in steps 6-14 is the initial procedure without any security established prior these steps. - Current implementation will handle first any number of PDUs declared with correspondant pics and later will handle emergency PDU. This may work for eCall Capable UEs but in case of eCall Only UEs emergency and other PDUs needs to be handled in parallel. - The coordination in current implementation is incorrect. Prior the call release in step 29 the IMS2 is sending coordination “Emergency Call Established” that needs to be caugth by NR5GC first. Secondly, the NR5GC needs to signal that call is going to be released in this step and then again in step 44. |
| Summary of change | - Security set to Initial_NoSecurity for steps 6-14. - Reworked PDU stablishment to handle configured PDUs in parallel of emergency PDU. Note that f_NR5GC_eCallSteps24_26 is defined in change 5. - Correction to coordination handling in 2 places. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | - Accepted. - Alternative solution implemented: split existing function f_NR5GC_ECallOnly_Steps3_23 - Accepted. |

Before Change:

// @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_Secure); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Steps 15A" siclog@ // @sic R5-R5-251283 sic@/ // The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. f_NR5GC_Registration_Complete(nr_Cell1); // @siclog "Steps 16-19" siclog@ // @sic R5-251283 sic@/ // Void // EXCEPTION: Step 20 can occur in parallel with steps 21-26 // @sic R5-251283 sic@/ while (v_N < 2){ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_N := v_N + 1; if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerESMMsg), ?, -, cr_NG_Request_Type('001'B))) { // @siclog "Step 20" siclog@s // Steps 20A of Table 4.9.29.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_PDUSessionEstablishment (nr_Cell1,1,tsc_NR_RbId_SRB2); } else if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerESMMsg), ?, -, cr_NG_Request_Type('011'B))) { // @siclog "Step 21" siclog@ // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST message? // @siclog "Steps 22-26" siclog@ // Steps 22-26 of Table 4.9.29.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_NR5GC_eCallSteps21_26(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21 PASS"); } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect ULInformationTransfer message Request type"); } } // @siclog "Steps 27-28" siclog@ // Release eCall over IMS using the generic procedure described in TS 38.508-1 [4] subclause 4.9.12B. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //Trigger eCall release over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8 f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); //IMS messages have completed in IMS PTC, now to handle NAS and RRC actions. // @siclog "Step 29" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3444 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3444_Min.start; // @siclog "Step 30" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 31-43" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 44-45" siclog@ // Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 46" siclog@

After Change:

// @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_NoSecurity); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Steps 16-20a1" siclog@ // The SS transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1); // @siclog "Step 21" siclog@ // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST message? f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, // @sic R5s210134 sic@ -, -, crs_SSC_Mode(-, '001'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, (cr_NG_Request_Type('001'B), cr_NG_Request_Type('011'B)), -, -, -, -, -, -, -, -, -, -, false); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21 PASS"); // @siclog "Steps 22-26" siclog@ // Steps 24-26 of Table 4.9.29.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_NR5GC_eCallSteps24_26(nr_Cell1); // @siclog "Steps 27-28" siclog@ // Release eCall over IMS using the generic procedure described in TS 38.508-1 [4] subclause 4.9.12B. // Void //Catch the last coordination from f_IMS_NR5GC_ECall running on IMS2 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //Trigger eCall release over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8 f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); //IMS messages have completed in IMS PTC, now to handle NAS and RRC actions. // @siclog "Step 29" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3444 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3444_Min.start; // @siclog "Step 30" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 31-43" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 44-45" siclog@ // Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 46" siclog@

#### MCC160 Implementation

function f_NR5GC_ECallOnly_Steps3_23(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC

{

//  Steps 3-20

// Steps 2 to 15 of generic procedure 4.5.2.2-2

// @sic R5s230685 sic@

f_NR5GC_Registration(p_NR_CellId, -, Initial_NoSecurity);

f_NR5GC_ECallOnly_Steps20A_23(p_NR_CellId);

}

function f_NR5GC_ECallOnly_Steps20A_23(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var boolean v_InitialRxd := false;

var boolean v_EmgRxd := false;

var default v_MyDefaultVar;

// Clear store of msgs received in parallel

f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@

// Then activate store for any new msgs received - this works for Modification Reqs too

v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId));

while (not v_InitialRxd or not v_EmgRxd) {

if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) {

v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1);

} else {

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp);

}

if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType, cr_NG_Request_Type('001'B))) { // initial

v_InitialRxd := true;

f_NR5GC_PDUSessionEstablishment (p_NR_CellId, 1, v_SRB,

-, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -, -, -, false, 1);

}

else if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType, cr_NG_Request_Type('011'B))) { // emergency

v_EmgRxd := true;

f_NR5GC_EmergencyCallPDUSessionEstablishment(p_NR_CellId, v_SRB, 1);     /* @sic R5-253127: IPCAN implementation for 34.229-5 test case 11.18 sic@

* NOTE: establishment of PDU sessions for emergency and normal IMS are expected to happen in parallel and

*       especially the case of the establishment request for the normal PDU session being sent after the

*       PDU session modification of the emergency PDU session is not considered */

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong Request Type");

}

v_SRB := tsc_NR_RbId_SRB2;

// Can remove the message that's been processed so that there's only 1 when calling f_NR5GC_PDUSessionEstablishment

f_NR5GC_MsgInDefault_RemoveOneNASMessageReq();

}

deactivate(v_MyDefaultVar);

}

function f_NR5GC_ECallOnly_Steps20A_26(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC

{

f_NR5GC_ECallOnly_Steps20A_23(p_NR_CellId);

// Steps 24-26

f_NR5GC_EmergencyCallPDUSessionModification(p_NR_CellId);

}

function fl_TC_11_5_1_Body()runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var float v_WaitMin := f_NR_SetTimerToleranceMin (nr_Cell1, nasTimer, 120.0);    // step 3

var integer v_N := 0;

timer t_T3444_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 43200.0); // 12 hours

timer t_T3512_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 25200.0); // 7  hours

//@siclog "Step 1" siclog@

//The SS configures:

// - NR Cell 1 as "Serving cell"

f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power");

// @siclog "Step 2" siclog@

// The UE is Switched ON

f_UT_SwitchOnUE(UT);

// @siclog "Step 3" siclog@

// Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 120 seconds?

f_NR_CheckNoRRCSetupReq (nr_Cell1, v_WaitMin);

// @siclog "Step 4" siclog@

// A manual eCall is initiated. (Note 1)

// Note 1: The request to originate a manual eCall may be performed by MMI or AT command.

f_UT_InitiateECall(UT, MANUAL);

// @siclog "Step 5" siclog@

// Check: Does the UE send REGISTRATION REQUEST message with 5GS registration type set to 'initial registration'?

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else{

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 5 PASS");

}

// @siclog "Steps 6-14" siclog@

// Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed.

f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo,

nr_Cell1,

v_ReceivedMsg,

f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)),

TESTMode_OFF,

Initial_Secure);

// @siclog "Step 15" siclog@

// The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512.

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity,

v_GMM_MobilityInfo,

f_NR5GC_GetMobileIdGUTI (nr_Cell1),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)),

{bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}),

-, -, -, -, -,-, -, -, -, -, -, -,

cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

// @siclog "Steps 15A" siclog@ // @sic R5-R5-251283 sic@/

// The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message.

f_NR5GC_Registration_Complete(nr_Cell1);

// @siclog "Steps 16-19" siclog@ // @sic R5-251283 sic@/

// Void

// EXCEPTION: Step 20 can occur in parallel with steps 21-26 // @sic R5-251283 sic@/

f_NR5GC_eCallSteps20A_26(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21 PASS");

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect ULInformationTransfer message Request type");

}

}

// @siclog "Steps 27-28" siclog@

// Release eCall over IMS using the generic procedure described in  TS 38.508-1 [4] subclause 4.9.12B.

// Void

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]);     //Trigger eCall release over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8

f_NR5GC_EmergencyCallReleaseMT(nr_Cell1, false);   //IMS messages have completed in IMS PTC, now to handle NAS and RRC actions.

// @siclog "Step 29" siclog@

// The SS releases the RRC connection. (Note 2)

// Note 2: Timer T3444 of 12hours starts.

f_NR_RRCRelease(nr_Cell1);

t_T3444_Min.start;

// @siclog "Step 30" siclog@

// The SS waits 1 minute.

f_Delay(60.0);

// @siclog "Steps 31-43" siclog@

// Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed.

f_NR5GC_MTSpeechCall(nr_Cell1);

// @siclog "Steps 44-45" siclog@

// Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place.

// Void

f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required

// @siclog "Step 46" siclog@

// The SS releases the RRC connection.

// Note 3: Timer T3512 of 7hours starts.

f_NR_RRCRelease(nr_Cell1);

t_T3512_Min.start;

// @siclog "Step 47" siclog@

// Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? (Note 3)

t_T3512_Min.timeout;

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message failed");

}

else{

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 47 PASS");

}

// @siclog "Step 48" siclog@

// The SS transmits a REGISTRATION ACCEPT message including T3512 value IE.

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity,

v_GMM_MobilityInfo,

f_NR5GC_GetMobileIdGUTI (nr_Cell1),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)),

{bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}),

-, -, -, -, -,-, -, -, -, -, -, -,

cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

// @siclog "Step 49" siclog@

// The UE transmits a REGISTRATION COMPLETE message

f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB2);

// @siclog "Step 50" siclog@

// The SS releases the RRC connection.

f_NR_RRCRelease(nr_Cell1);

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); //Set EUTRA Cell power

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Cell Power");

// @siclog "Step 51" siclog@

// The SS configures:

// - NR Cell 1 as " Non-suitable "off" cell"

f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

// @siclog "Step 52" siclog@

// Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1?

f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1);

f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power");

//@siclog "Step 53" siclog@

//The SS configures:

// - NR Cell 1 as "Serving cell"

f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

// @siclog "Step 54" siclog@

// Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1?

f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -);

// @siclog "Step 55" siclog@

// Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444?

t_T3444_Min.timeout;

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu, crs_DeregisterType('0'B, '0'B, -))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

else{

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS");

}

} //End of fl_TC_11_5_1_Body()

## Change 3

| Function name | fl_TC_11_5_1_Body |
| --- | --- |
| Reason for change | - During steps 47-48 the SRB2 has not been established, so the REG COMPLETE in step 49 should be expected on SRB1. - Correction of NR cell ID is needed in step 51. - At step 55, UE should be in idle state, then previous to reregistration, UE needs to establish a RRC connection. |
| Summary of change | - Changed SRB2 to SRB1 for step 49. - Changed NRcell11 to NRcell1 in step 51 - Reworked reregistration steps. Expecting a RRC connectionr request previous to reregistration message. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | - Accepted. - Accepted. - Accepted. |

Before Change:

// @siclog "Step 49" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB2); // @siclog "Step 50" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); //Set EUTRA Cell power f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); //@siclog "Step 53" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); // @siclog "Step 55" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444? t_T3444_Min.timeout; SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu, crs_DeregisterType('0'B, '0'B, -))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } } //End of fl_TC_11_5_1_Body()

After Change:

// @siclog "Step 49" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); // @siclog "Step 50" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); //Set EUTRA Cell power f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); //@siclog "Step 53" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); // @siclog "Step 55" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444? t_T3444_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu, crs_DeregisterType('0'B, '0'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } } //End of fl_TC_11_5_1_Body()

## Change 4

| Function name | f_IMS_NR5GC_MTSpeechCall |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function, coordination messages are not aligned. Test case will be stuck if pc_Preconditions is false as a coordination message will be not sent IMS component to confirm that PDU session has been modified. |
| Summary of change | Expected coordination message with specific contents in case of no preconditions. |
| TTCN module | IMS_NR5GC_Functions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1a_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, pc_Preconditions); f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ }

After Change:

function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_DATA_RSP v_IMS_DATA_RSP_ByRef; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1a_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } a_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, v_IMS_DATA_RSP_ByRef); f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ }

## Change 5

| Function name | f_NR5GC_eCallSteps24_26 |
| --- | --- |
| Reason for change | To cover steps 22-26 of 38.508-1 table 4.9.29.2.2-1 correspondingly to the changes proposed in Change 2.2 in this document. |
| Summary of change | New function to cover steps 22-26 of 38.508-1 table 4.9.29.2.2-1. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Not needed. This is the same as existing function f_NR5GC_EmergencyCallPDUSessionModification |

After Change:

function f_NR5GC_eCallSteps24_26(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Add Voice Bearer"); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); }

## Change 6

| Function name | f_GetTestcaseAttrib_ECallOverIMS |
| --- | --- |
| Reason for change | In order to properly set support of IMS emergency in system information the test needs to be included in f_GetTestcaseAttrib_ECallOverIMS. |
| Summary of change | Added 11.5.1 into f_GetTestcaseAttrib_ECallOverIMS. |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_GetTestcaseAttrib_ECallOverIMS(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_5_10_NR5GC") { return true; } case ("TC_11_5_11_NR5GC") { return true; } case ("TC_11_5_12_NR5GC") { return true; } case ("TC_11_5_13_NR5GC") { return true; } case ("TC_11_5_14_NR5GC") { return true; } case ("TC_11_5_3_NR5GC") { return true; } case ("TC_11_5_4_NR5GC") { return true; } case ("TC_11_5_7_NR5GC") { return true; } case ("TC_11_5_8_NR5GC") { return true; } case ("TC_11_5_9_NR5GC") { return true; } } return false; }

After Change:

function f_GetTestcaseAttrib_ECallOverIMS(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_5_10_NR5GC") { return true; } case ("TC_11_5_11_NR5GC") { return true; } case ("TC_11_5_12_NR5GC") { return true; } case ("TC_11_5_13_NR5GC") { return true; } case ("TC_11_5_14_NR5GC") { return true; } case ("TC_11_5_1_NR5GC") { return true; } case ("TC_11_5_3_NR5GC") { return true; } case ("TC_11_5_4_NR5GC") { return true; } case ("TC_11_5_7_NR5GC") { return true; } case ("TC_11_5_8_NR5GC") { return true; } case ("TC_11_5_9_NR5GC") { return true; } } return false; }

# Branches executed

This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA1/EIA1 Integrity and NEA1/EEA1 ciphering algorithms in Anritsu Protocol Conformance Test System ME7834NR.

On ROHDE & SCHWARZ, This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA2 Integrity and NEA2 ciphering algorithms.

# Execution Log Files

## Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

The Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_11_5_1_LOG.html

ROHDE & SCHWARZ\ TC_11_5_1_LOG.txt

- PICS/PIXIT parameter file:

TC_11_5_1_PIXIT.xml

ROHDE & SCHWARZ\ In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250361: Supporting information for addition of NR5GC testcase 11.5.1 in FR1 |
| --- | --- |