3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250367

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3746 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_Rf_Parameters_NRBand_PICS_Supp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation set the pc_type2_PUSCH_RepetitionMultiSlots = True only if parameter type2_PUSCH_RepetitionMultiSlots is reported and not consider the case when UE report it via type2_PUSCH_RepetitionMultiSlots_v1650. Base on 38.306 there is a rule The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCHRepetitionMultiSlots is absent |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change ttcn that pc_type2_PUSCH_RepetitionMultiSlots = True also when UE report it via type2-PUSCH-RepetitionMultiSlots-v1650 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_Check_Rf_Parameters_NRBand_PICS_Supp |
| --- | --- |
| Reason for change | Current implementation set the pc_type2_PUSCH_RepetitionMultiSlots = True only if parameter type2_PUSCH_RepetitionMultiSlots is reported and not consider the case when UE report it via type2_PUSCH_RepetitionMultiSlots_v1650. Base on 38.306 there is a rule The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCHRepetitionMultiSlots is absent |
| Summary of change | Change ttcn that pc_type2_PUSCH_RepetitionMultiSlots = True also when UE report it via type2-PUSCH-RepetitionMultiSlots-v1650 |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted but implemented differently. See MCC160’s implementation below. |

Before Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 R5-246407 R5-251949 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index; var FreqBandIndicatorNR v_Band; … var boolean v_Found_multiCell_PUSCH_DCI_0_3_SameSCS_r18 := false; var boolean v_Found_supportOfSearchSpace_r18_DCI_0_3 := false; var template(value) CA_ParametersNR_v1800.multiCell_PDSCH_DCI_1_3_SameSCS_r18.coScheduledCellIndicationScheme_r18 v_CoScheduledCellIndicationScheme_r18_DCI_1_3; var template(value) CA_ParametersNR_v1800.multiCell_PUSCH_DCI_0_3_SameSCS_r18.coScheduledCellIndicationScheme_r18 v_CoScheduledCellIndicationScheme_r18_DCI_0_3; var template(value) CA_ParametersNR_v1800.multiCell_PDSCH_DCI_1_3_SameSCS_r18.harqFeedbackType_r18 v_HarqFeedbackType_DCI_1_3_r18; … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17@rel18 if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_DynamicChAccess_r16)) { v_Found_ul_DynamicChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ v_Found_ul_Semi_StaticChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ v_Found_rssi_ChannelOccupancyReporting_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandover_r16)) { v_Found_condHandover_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverFailure_r16)) { v_Found_condHandoverFailure_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverTwoTriggerEvents_r16)) { v_Found_condHandoverTwoTriggerEvents_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condPSCellChange_r16)) { v_Found_condPSCellChange_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].activeConfiguredGrant_r16)) { v_Found_activeConfiguredGrant_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sps_r16)) { // @sic R5-236307 sic@ if (not pc_downlinkSPS){ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2 the UE can include this feature only if it indicates support of downlinkSPS"); } v_Found_sps_r16 := true; // Checked as boolean because it is an optional field. No further check of contents } … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17&Rel18 if (not f_NR_CompareCapability_vs_PICS(pc_ul_DynamicChAccess_r16, v_Found_ul_DynamicChAccess_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_DynamicChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_ul_Semi_StaticChAccess_r16, v_Found_ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_Semi_StaticChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_rssi_ChannelOccupancyReporting_r16, v_Found_rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_rssi_ChannelOccupancyReporting_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandover_r16, v_Found_condHandover_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandover_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverFailure_r16, v_Found_condHandoverFailure_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverFailure_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverTwoTriggerEvents_r16, v_Found_condHandoverTwoTriggerEvents_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverTwoTriggerEvents_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condPSCellChange_r16, v_Found_condPSCellChange_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condPSCellChange_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_activeConfiguredGrant_r16, v_Found_activeConfiguredGrant_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_activeConfiguredGrant_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_multi_sps_r16, v_Found_sps_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_multi_sps_r16 not correctly set"); } if (p_Configuration_Type == NR_SA) { // only in TC8.1.5.1.1 if (not f_NR_CompareCapability_vs_PICS(pc_txDiversity_r16, v_Found_txDiversity_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_txDiversity_r16 not correctly set"); } } // @sic R5s240137 sic@

After Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 R5-246407 R5-251949 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index; var FreqBandIndicatorNR v_Band; … var boolean v_Found_multiCell_PUSCH_DCI_0_3_SameSCS_r18 := false; var boolean v_Found_supportOfSearchSpace_r18_DCI_0_3 := false; var boolean v_Found_PUSCH_RepetitionMultiSlots_v1650 := false; var template(value) CA_ParametersNR_v1800.multiCell_PDSCH_DCI_1_3_SameSCS_r18.coScheduledCellIndicationScheme_r18 v_CoScheduledCellIndicationScheme_r18_DCI_1_3; var template(value) CA_ParametersNR_v1800.multiCell_PUSCH_DCI_0_3_SameSCS_r18.coScheduledCellIndicationScheme_r18 v_CoScheduledCellIndicationScheme_r18_DCI_0_3; var template(value) CA_ParametersNR_v1800.multiCell_PDSCH_DCI_1_3_SameSCS_r18.harqFeedbackType_r18 v_HarqFeedbackType_DCI_1_3_r18; … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17@rel18 if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_DynamicChAccess_r16)) { v_Found_ul_DynamicChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ v_Found_ul_Semi_StaticChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ v_Found_rssi_ChannelOccupancyReporting_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandover_r16)) { v_Found_condHandover_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverFailure_r16)) { v_Found_condHandoverFailure_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverTwoTriggerEvents_r16)) { v_Found_condHandoverTwoTriggerEvents_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condPSCellChange_r16)) { v_Found_condPSCellChange_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].activeConfiguredGrant_r16)) { v_Found_activeConfiguredGrant_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].type2_PUSCH_RepetitionMultiSlots_v1650)) { if (isvalue(v_Received_NRCapabilityMsg.phy_Parameters.phy_ParametersCommon.type2_PUSCH_RepetitionMultiSlots)){ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2 the UE can include type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCHRepetitionMultiSlots is absent"); } v_Found_PUSCH_RepetitionMultiSlots_v1650 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sps_r16)) { // @sic R5-236307 sic@ if (not pc_downlinkSPS){ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2 the UE can include this feature only if it indicates support of downlinkSPS"); } v_Found_sps_r16 := true; // Checked as boolean because it is an optional field. No further check of contents } … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17&Rel18 if (not f_NR_CompareCapability_vs_PICS(pc_ul_DynamicChAccess_r16, v_Found_ul_DynamicChAccess_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_DynamicChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_ul_Semi_StaticChAccess_r16, v_Found_ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_Semi_StaticChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_rssi_ChannelOccupancyReporting_r16, v_Found_rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_rssi_ChannelOccupancyReporting_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandover_r16, v_Found_condHandover_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandover_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverFailure_r16, v_Found_condHandoverFailure_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverFailure_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverTwoTriggerEvents_r16, v_Found_condHandoverTwoTriggerEvents_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverTwoTriggerEvents_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condPSCellChange_r16, v_Found_condPSCellChange_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condPSCellChange_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_activeConfiguredGrant_r16, v_Found_activeConfiguredGrant_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_activeConfiguredGrant_r16 not correctly set"); } if (not v_Found_PUSCH_RepetitionMultiSlots_v1650){ if (isvalue(v_Received_NRCapabilityMsg.phy_Parameters.phy_ParametersCommon.type2_PUSCH_RepetitionMultiSlots)){ v_Found_PUSCH_RepetitionMultiSlots_v1650 := true; } } if (not f_NR_CompareCapability_vs_PICS(pc_type2_PUSCH_RepetitionMultiSlots, v_Found_PUSCH_RepetitionMultiSlots_v1650)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_type2_PUSCH_RepetitionMultiSlots not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_multi_sps_r16, v_Found_sps_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_multi_sps_r16 not correctly set"); } if (p_Configuration_Type == NR_SA) { // only in TC8.1.5.1.1 if (not f_NR_CompareCapability_vs_PICS(pc_txDiversity_r16, v_Found_txDiversity_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_txDiversity_r16 not correctly set"); } } // @sic R5s240137 sic@

MCC160 implementation:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 R5-246407 R5-251949 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index; var FreqBandIndicatorNR v_Band; … var boolean v_BWP_SameNumerology_Set := (pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2); //@sic R5-230775 sic@ var boolean v_Found_PUSCH_RepetitionMultiSlots_v1650 := false; //@sic R5-250367 sic@ var boolean v_Type2_PUSCH_RepetitionMultiSlots := false; … v_Received_NRCapabilityMsg := p_NRCapability; v_Type2_PUSCH_RepetitionMultiSlots := isvalue(v_Received_NRCapabilityMsg.phy_Parameters.phy_ParametersCommon.type2_PUSCH_RepetitionMultiSlots); if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters) and isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0])) { // rf_Parameters and rf_Parameters.supportedBandListNR[x] are mandatory, so need to check first … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17@rel18 if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_DynamicChAccess_r16)) { v_Found_ul_DynamicChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ v_Found_ul_Semi_StaticChAccess_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sharedSpectrumChAccessParamsPerBand_r16.rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ v_Found_rssi_ChannelOccupancyReporting_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandover_r16)) { v_Found_condHandover_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverFailure_r16)) { v_Found_condHandoverFailure_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condHandoverTwoTriggerEvents_r16)) { v_Found_condHandoverTwoTriggerEvents_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].condPSCellChange_r16)) { v_Found_condPSCellChange_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].activeConfiguredGrant_r16)) { v_Found_activeConfiguredGrant_r16 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].type2_PUSCH_RepetitionMultiSlots_v1650)) { // @sic R5-250367 sic@ v_Found_PUSCH_RepetitionMultiSlots_v1650 := true; } if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].sps_r16)) { // @sic R5-236307 sic@ if (not pc_downlinkSPS){ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2 the UE can include this feature only if it indicates support of downlinkSPS"); } v_Found_sps_r16 := true; // Checked as boolean because it is an optional field. No further check of contents } … if (match (v_Received_NRCapabilityMsg.accessStratumRelease, (rel16, rel17, rel18))) { // checks Rel16&17&Rel18 if (not f_NR_CompareCapability_vs_PICS(pc_ul_DynamicChAccess_r16, v_Found_ul_DynamicChAccess_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_DynamicChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_ul_Semi_StaticChAccess_r16, v_Found_ul_Semi_StaticChAccess_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ul_Semi_StaticChAccess_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_rssi_ChannelOccupancyReporting_r16, v_Found_rssi_ChannelOccupancyReporting_r16)) { // @sic R5-245527 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_rssi_ChannelOccupancyReporting_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandover_r16, v_Found_condHandover_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandover_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverFailure_r16, v_Found_condHandoverFailure_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverFailure_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condHandoverTwoTriggerEvents_r16, v_Found_condHandoverTwoTriggerEvents_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condHandoverTwoTriggerEvents_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_condPSCellChange_r16, v_Found_condPSCellChange_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_condPSCellChange_r16 not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_activeConfiguredGrant_r16, v_Found_activeConfiguredGrant_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_activeConfiguredGrant_r16 not correctly set"); } if (not (v_Found_PUSCH_RepetitionMultiSlots_v1650 and v_Type2_PUSCH_RepetitionMultiSlots)){ // @sic R5-250367 sic@ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2: The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent"); } v_Found_PUSCH_RepetitionMultiSlots_v1650 := v_Found_PUSCH_RepetitionMultiSlots_v1650 or v_Type2_PUSCH_RepetitionMultiSlots; if (not f_NR_CompareCapability_vs_PICS(pc_type2_PUSCH_RepetitionMultiSlots, v_Found_PUSCH_RepetitionMultiSlots_v1650)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_type2_PUSCH_RepetitionMultiSlots not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_multi_sps_r16, v_Found_sps_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_multi_sps_r16 not correctly set"); } if (p_Configuration_Type == NR_SA) { // only in TC8.1.5.1.1 if (not f_NR_CompareCapability_vs_PICS(pc_txDiversity_r16, v_Found_txDiversity_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_txDiversity_r16 not correctly set"); } } // @sic R5s240137 sic@

## Change 2

| Function name | cr_Phy_ParametersCommon |
| --- | --- |
| Reason for change | Due to above change 1 there is no need to check if UE reported type2_PUSCH_RepetitionMultiSlots in template cr_Phy_ParametersCommon. To check it here can cause a problem as pc_type2_PUSCH_RepetitionMultiSlots can be True when UE report this feature via type2-PUSCH-RepetitionMultiSlots-v1650. In that case type2_PUSCH_RepetitionMultiSlots must be absent |
| Summary of change | Remove checking of type2-PUSCH-RepetitionMultiSlots-v1650 from template cr_Phy_ParametersCommon |
| TTCN module | NR_AuxiliaryCapCheckFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template (present) Phy_ParametersCommon cr_Phy_ParametersCommon := { /* @status APPROVED (ENDC, NR5GC) */ csi_RS_CFRA_ForHO := f_NR_Derive_Phy_ParametersCommon_csi_RS_CFRA_ForHO(), dynamicPRB_BundlingDL := *, sp_CSI_ReportPUCCH := *, sp_CSI_ReportPUSCH := *, nzp_CSI_RS_IntefMgmt := *, type2_SP_CSI_Feedback_LongPUCCH := *, precoderGranularityCORESET := *, dynamicHARQ_ACK_Codebook := supported, // @sic R5-211500 R5-211501 sic@ semiStaticHARQ_ACK_Codebook := *, spatialBundlingHARQ_ACK := f_NR_Derive_Phy_ParametersCommon_spatialBundlingHARQ_ACK(), //@sic R5-225384 sic@ dynamicBetaOffsetInd_HARQ_ACK_CSI := *, pucch_Repetition_F1_3_4 := *, ra_Type0_PUSCH := f_NR_Derive_Phy_ParametersCommon_ra_Type0_PUSCH(), dynamicSwitchRA_Type0_1_PDSCH := f_NR_Derive_Phy_ParametersCommon_dynamicSwitchRA_Type0_1_PDSCH(), //@sic R5-225384 sic@ dynamicSwitchRA_Type0_1_PUSCH := f_NR_Derive_Phy_ParametersCommon_dynamicSwitchRA_Type0_1_PUSCH(), //@sic R5-225384 sic@ pdsch_MappingTypeA := supported, // @sic R5-211500 R5-211501 sic@ - before f_NR_Derive_Phy_ParametersCommon_pdsch_MappingTypeA(), pdsch_MappingTypeB := f_NR_Derive_Phy_ParametersCommon_pdsch_MappingTypeB(), interleavingVRB_ToPRB_PDSCH := f_NR_Derive_Phy_ParametersCommon_interleavingVRB_ToPRB_PDSCH(), interSlotFreqHopping_PUSCH := *, type1_PUSCH_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_type1_PUSCH_RepetitionMultiSlots(), //@sic R5-251949 sic@ type2_PUSCH_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_type2_PUSCH_RepetitionMultiSlots(), //@sic R5-251949 sic@ pusch_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_pusch_RepetitionMultiSlots(), // @sic R5-197070 sic@ pdsch_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_pdsch_RepetitionMultiSlots(), downlinkSPS := f_NR_Derive_Phy_ParametersCommon_downlinkSPS(), configuredUL_GrantType1 := f_NR_Derive_Phy_ParametersCommon_configuredUL_GrantType1(), configuredUL_GrantType2 := f_NR_Derive_Phy_ParametersCommon_configuredUL_GrantType2(), pre_EmptIndication_DL := *,

After Change:

template (present) Phy_ParametersCommon cr_Phy_ParametersCommon := { /* @status APPROVED (ENDC, NR5GC) */ csi_RS_CFRA_ForHO := f_NR_Derive_Phy_ParametersCommon_csi_RS_CFRA_ForHO(), dynamicPRB_BundlingDL := *, sp_CSI_ReportPUCCH := *, sp_CSI_ReportPUSCH := *, nzp_CSI_RS_IntefMgmt := *, type2_SP_CSI_Feedback_LongPUCCH := *, precoderGranularityCORESET := *, dynamicHARQ_ACK_Codebook := supported, // @sic R5-211500 R5-211501 sic@ semiStaticHARQ_ACK_Codebook := *, spatialBundlingHARQ_ACK := f_NR_Derive_Phy_ParametersCommon_spatialBundlingHARQ_ACK(), //@sic R5-225384 sic@ dynamicBetaOffsetInd_HARQ_ACK_CSI := *, pucch_Repetition_F1_3_4 := *, ra_Type0_PUSCH := f_NR_Derive_Phy_ParametersCommon_ra_Type0_PUSCH(), dynamicSwitchRA_Type0_1_PDSCH := f_NR_Derive_Phy_ParametersCommon_dynamicSwitchRA_Type0_1_PDSCH(), //@sic R5-225384 sic@ dynamicSwitchRA_Type0_1_PUSCH := f_NR_Derive_Phy_ParametersCommon_dynamicSwitchRA_Type0_1_PUSCH(), //@sic R5-225384 sic@ pdsch_MappingTypeA := supported, // @sic R5-211500 R5-211501 sic@ - before f_NR_Derive_Phy_ParametersCommon_pdsch_MappingTypeA(), pdsch_MappingTypeB := f_NR_Derive_Phy_ParametersCommon_pdsch_MappingTypeB(), interleavingVRB_ToPRB_PDSCH := f_NR_Derive_Phy_ParametersCommon_interleavingVRB_ToPRB_PDSCH(), interSlotFreqHopping_PUSCH := *, type1_PUSCH_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_type1_PUSCH_RepetitionMultiSlots(), //@sic R5-251949 sic@ type2_PUSCH_RepetitionMultiSlots := *, //@sic R5-251949 sic@ pusch_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_pusch_RepetitionMultiSlots(), // @sic R5-197070 sic@ pdsch_RepetitionMultiSlots := f_NR_Derive_Phy_ParametersCommon_pdsch_RepetitionMultiSlots(), downlinkSPS := f_NR_Derive_Phy_ParametersCommon_downlinkSPS(), configuredUL_GrantType1 := f_NR_Derive_Phy_ParametersCommon_configuredUL_GrantType1(), configuredUL_GrantType2 := f_NR_Derive_Phy_ParametersCommon_configuredUL_GrantType2(), pre_EmptIndication_DL := *,