3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250374

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3751 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_DomainName_DecodeToList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation of the above function results in a IndexOutOfRangeException when testcases 10.1.6.1 is run. fl_TC_10_1_6_1_Bodyf_NR5GC_TriggerSinglePDUSessionf_NR5GC_PDUSessionEstablishmentf_NR5GC_PDUSessionEstablishment_InnerLoopf_Check_NG_PDUSessionEstablishmentReqf_GetPDNTypeFromDNNf_GetPDN_DNNTypeFromAPN--f_IsDNNForIMSf_DomainName_DecodeToList results in the above error. The first iteration of the while loop in the function f_DomainName_DecodeToList is fine, and results in value of i to be 5. During the second loop, this causes the above mentioned error in assignment of v_LableLength := oct2int(p_OctetString[i]); because there is no 5th element in p_OctetString, when DNN=ABCC is set (as per usage of tsc_RandomAPN). This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworked the function f_DomainName_DecodeToList. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.6.1.NR5GC and other NR5GC testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_DomainName_DecodeToList |
| --- | --- |
| Reason for change | The current TTCN implementation of the above function results in a IndexOutOfRangeException when testcases 10.1.6.1 is run. fl_TC_10_1_6_1_Bodyf_NR5GC_TriggerSinglePDUSessionf_NR5GC_PDUSessionEstablishmentf_NR5GC_PDUSessionEstablishment_InnerLoopf_Check_NG_PDUSessionEstablishmentReqf_GetPDNTypeFromDNNf_GetPDN_DNNTypeFromAPN--f_IsDNNForIMSf_DomainName_DecodeToList results in the above error. The first iteration of the while loop in the function f_DomainName_DecodeToList is fine, and results in value of i to be 5. During the second loop, this causes the above mentioned error in assignment of v_LableLength := oct2int(p_OctetString[i]); because there is no 5th element in p_OctetString, when DNN=ABCC is set (as per usage of tsc_RandomAPN). This needs to be addressed. |
| Summary of change | Reworked the function f_DomainName_DecodeToList. Please see below. |
| TTCN module | CommonDefs.ttcn |
| MCC160 Comment | Accepted in principle: The proposal seems not be correct in all cases implemented as shown below. |

Before Change

function f_DomainName_DecodeToList(octetstring p_OctetString) return CharStringList_Type { var CharStringList_Type v_LableList := {} var integer i := 0; var integer k := 0; var integer v_StringLength := lengthof(p_OctetString); var integer v_LableLength; var octetstring v_EncodedLable; while (true) { v_LableLength := oct2int(p_OctetString[i]); i := i + 1; if ((v_LableLength > 0) and (i + v_LableLength <= v_StringLength)) { // @sic R5s250298 sic@ v_EncodedLable := substr(p_OctetString, i, v_LableLength); i := i + v_LableLength; v_LableList[k] := oct2char(v_EncodedLable); k := k + 1; } else { break; } } return v_LableList; }

After Change

function f_DomainName_DecodeToList(octetstring p_OctetString) return CharStringList_Type { var CharStringList_Type v_LableList := {} var integer i := 0; var integer k := 0; var integer v_StringLength := lengthof(p_OctetString); var integer v_LableLength := oct2int(p_OctetString[i]); var octetstring v_EncodedLable; while ((v_LableLength > 0) and (i + v_LableLength <= v_StringLength)) { v_LableLength := oct2int(p_OctetString[i]); i := i + 1; v_EncodedLable := substr(p_OctetString, i, v_LableLength); i := i + v_LableLength; v_LableList[k] := oct2char(v_EncodedLable); k := k + 1; } return v_LableList; }

## MCC160 Implementation

/* * @desc decode domain name being encoded according to RFC 1035 clause 4.1.2 and 23.003 clause 9.1: * the domain name is coded "as a sequence of labels, where each label consists of a length octet followed by that number of octets" (RFC 1035). * According to RFC 1035 the domain name is terminated by a "zero length octet for the null label of the root" in contrast to 23.003 (clause 9.1: "The APN is not terminated by a length byte of zero"). * Nevertheless it is assumed that there is no zero-length label within a domain name. * => The decoding stops at the end of the given octetstring or at the (1st) zero-length * @param p_OctetString * @return CharStringList_Type */ function f_DomainName_DecodeToList(octetstring p_OctetString) return CharStringList_Type { var CharStringList_Type v_LableList := {} var integer i := 0; var integer k := 0; var integer v_StringLength := lengthof(p_OctetString); var integer v_LableLength; var octetstring v_EncodedLable; while (i < v_StringLength) { v_LableLength := oct2int(p_OctetString[i]); i := i + 1; if (v_LableLength > 0) { if (i + v_LableLength > v_StringLength) { FatalError(__FILE__, __LINE__, "Invalid coding of domain name"); } else { v_EncodedLable := substr(p_OctetString, i, v_LableLength); i := i + v_LableLength; v_LableList[k] := oct2char(v_EncodedLable); k := k + 1; } } else { break; // according to RFC 1035 clause 4.1.2 "The domain name terminates with the zero length octet" => the loop terminates even when p_OctetString has further octets } } return v_LableList; }