3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250456

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3764 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.3.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN-3 implementation of test case 8.2.3.3.1 , measurement report is not properly checked for the presence of IE measResultNeighCellListNR-r15 at step 5 and 8. This implementation is not in line with Table 8.2.3.3.1.3.3-6: MeasurementReport (step 5, Table 8.2.3.3.1.3.2-2) and Table 8.2.3.3.1.3.3-7: MeasurementReport (step 8, Table 8.2.3.3.1.3.2-2). Also, this missing implementation can cause 8.2.3.3.1.ENDC to incorrectly pass when it should fail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test case 8.2.3.3.1 implementation at step 5 and step 8 to properly check for the presence of measResultNeighCellListNR-r15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case can incorrectly pass when it should fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN-3 implementation of test case 8.2.3.3.1 , measurement report is not properly checked for the presence of IE measResultNeighCellListNR-r15 at step 5 and 8. This implementation is not in line with Table 8.2.3.3.1.3.3-6: MeasurementReport (step 5, Table 8.2.3.3.1.3.2-2) and Table 8.2.3.3.1.3.3-7: MeasurementReport (step 8, Table 8.2.3.3.1.3.2-2). Also, this missing implementation can cause 8.2.3.3.1.ENDC to incorrectly pass when it should fail |
| Summary of change | Updated test case 8.2.3.3.1 implementation at step 5 and step 8 to properly check for the presence of measResultNeighCellListNR-r15 |
| TTCN module | RRCMeasurement_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted (in principle), but implemented differently (please see MCC160 proposal) |

Before Change:

function fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var integer v_NR_PhysChId_Cell2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc0; var FilterCoefficient v_FilterCoefficientRSRQ := fc0; var FilterCoefficient v_FilterCoefficientSINR := fc0; //@sic R5s190103 sic@ var float v_TimerValue_15s; var float v_TimerValue_10s; var float v_TimerValue_57s; //@sic R5s200411 sic@ var default v_DefaultRef; v_TimerValue_15s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 15.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_57s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 57.0); //@sic R5s200411 sic@ f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); //Acquire information of NR cell 1 and 2 v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_PhysChId_Cell2 := v_EUTRA_NR_SysInfo.NR[1].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ //Acquire information of NR cell 2 // v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[1].SCS); //to be checked (Prose should be changed accordingly) //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to setup inter RAT measurement and //reporting for NR Cell 1 and 2 and receives an RRCConnectionReconfigurationComplete message to confirm the setup of //inter RAT measurements for NR Cell 1 and 2 f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_Periodical_1Entry(v_RRC_TI, v_NR_ARFCN_f1, v_SCS, v_FreqBandIndicatorNR_r15, //@sic R5-194875 sic@ v_ReportAmount, v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR, cs_ReportQuantityNR_r15(true, true, pc_ss_SINR_Meas))); //@sic R5-194850, R5s190103, 5-190621 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T1" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s201023, R5-201632 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 1? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, ?, omit, omit))); //@sic R5s190103 sic@ } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, omit, omit, omit))); }; //@sic R5-194850 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5"); //Activate the default to receive measurement reports for NR Cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T2" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 and NR Cell2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 7" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency //else wait for 15 s to allow change of power levels and UE measurements for NR Cell 2 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ deactivate(v_DefaultRef); //@sic R5s190103 sic@ //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 2? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, ?, omit, omit))); //@sic R5s190103 sic@ } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, omit, omit, omit))); }; //@sic R5-194850 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8"); //@siclog "Step 9" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T3" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@sic R5s190103 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 10" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 and NR Cell 2. if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration message to remove inter-RAT measurement and reporting for NR Cell and //receives an RRCConnectionReconfigurationComplete message to confirm the remove of inter-RAT measurement for NR Cell. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_RemoveMeasId_1Entry(v_RRC_TI, 1)); deactivate(v_DefaultRef); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a MeasurementReport message on EUTRA Cell1 to report periodical reporting for NR Cell 1 and 2 during the next 10s? f_EUTRA_NoMeasurementReport(eutra_Cell1, v_TimerValue_10s, cr_MeasurementReport_CheckAny); }

After Change:

function fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var integer v_NR_PhysChId_Cell2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc0; var FilterCoefficient v_FilterCoefficientRSRQ := fc0; var FilterCoefficient v_FilterCoefficientSINR := fc0; //@sic R5s190103 sic@ var float v_TimerValue_15s; var float v_TimerValue_10s; var float v_TimerValue_57s; //@sic R5s200411 sic@ var default v_DefaultRef; var SRB_COMMON_IND v_SRB_IND_measurement; //WA#WI=1461995 v_TimerValue_15s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 15.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_57s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 57.0); //@sic R5s200411 sic@ f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); //Acquire information of NR cell 1 and 2 v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_PhysChId_Cell2 := v_EUTRA_NR_SysInfo.NR[1].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ //Acquire information of NR cell 2 // v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[1].SCS); //to be checked (Prose should be changed accordingly) //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to setup inter RAT measurement and //reporting for NR Cell 1 and 2 and receives an RRCConnectionReconfigurationComplete message to confirm the setup of //inter RAT measurements for NR Cell 1 and 2 f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_Periodical_1Entry(v_RRC_TI, v_NR_ARFCN_f1, v_SCS, v_FreqBandIndicatorNR_r15, //@sic R5-194875 sic@ v_ReportAmount, v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR, cs_ReportQuantityNR_r15(true, true, pc_ss_SINR_Meas))); //@sic R5-194850, R5s190103, 5-190621 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T1" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s201023, R5-201632 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 1? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, ?, omit, omit))) -> value v_SRB_IND_measurement; //@sic R5s190103 sic@ //WA#WI=1461995 } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, omit, omit, omit))) -> value v_SRB_IND_measurement; //WA#WI=1461995 }; //@sic R5-194850 sic@ // Validate that measResultNeighCells is present for test case 8.2.3.3.1 if (not ispresent(v_SRB_IND_measurement.Signalling.Rrc.Dcch.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultNeighCells)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5: measResultNeighCells is missing from MeasurementReport for NR Cell 1 - UE compliance issue"); } //WA#WI=1461995 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5"); //Activate the default to receive measurement reports for NR Cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T2" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 and NR Cell2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 7" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency //else wait for 15 s to allow change of power levels and UE measurements for NR Cell 2 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ deactivate(v_DefaultRef); //@sic R5s190103 sic@ //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 2? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, ?, omit, omit))) -> value v_SRB_IND_measurement; //@sic R5s190103 sic@ //WA#WI=1461995 } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, omit, omit, omit))) -> value v_SRB_IND_measurement; //WA#WI=1461995 }; //@sic R5-194850 sic@ // Validate that measResultNeighCells is present for test case 8.2.3.3.1 if (not ispresent(v_SRB_IND_measurement.Signalling.Rrc.Dcch.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultNeighCells)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8: measResultNeighCells is missing from MeasurementReport for NR Cell 2 - UE compliance issue"); } //WA#WI=1461995 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8"); //@siclog "Step 9" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T3" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@sic R5s190103 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 10" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 and NR Cell 2. if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration message to remove inter-RAT measurement and reporting for NR Cell and //receives an RRCConnectionReconfigurationComplete message to confirm the remove of inter-RAT measurement for NR Cell. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_RemoveMeasId_1Entry(v_RRC_TI, 1)); deactivate(v_DefaultRef); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a MeasurementReport message on EUTRA Cell1 to report periodical reporting for NR Cell 1 and 2 during the next 10s? f_EUTRA_NoMeasurementReport(eutra_Cell1, v_TimerValue_10s, cr_MeasurementReport_CheckAny); }

MCC160 proposal:

module RRCMeasurement_ENDC_EUTRA { import from CommonDefs all; import from CommonAspDefs all; ….. template (present) MeasResults.measResultNeighCells cr_MeasResultNeighCells_measResultListNR_1Entry(integer p_PhysCellId, template RSRQ_RangeNR_r15 p_RsrqResult_r15, template RS_SINR_RangeNR_r15 p_Rs_Sinr_Result_r15, template MeasResultSSB_IndexList_r15 p_MeasResultSSB_IndexList_r15, template CGI_InfoNR_r15 p_Cgi_Info_r15, template RSRP_RangeNR_r15 p_RsrpResult_r15 := ?) := //@sic R5s200292 sic@ { /* @status APPROVED (ENDC) */ measResultNeighCellListNR_r15 := { cr_MeasResultCellNR(p_PhysCellId, cr_MeasResultNR(p_RsrpResult_r15, //@sic R5s200292 sic@ p_RsrqResult_r15, //@sic R5-186151 sic@ p_Rs_Sinr_Result_r15), //@sic R5-186151 sic@ p_MeasResultSSB_IndexList_r15, p_Cgi_Info_r15) //@sic R5-186151 sic@ } }; template (present) UL_DCCH_Message cr_MeasurementReport_NR_1Entry(integer p_MeasId, integer p_PhysCellId, template RSRQ_RangeNR_r15 p_RsrqResult_r15, template RS_SINR_RangeNR_r15 p_Rs_Sinr_Result_r15, template MeasResultSSB_IndexList_r15 p_MeasResultSSB_IndexList_r15, template CGI_InfoNR_r15 p_Cgi_Info_r15, template RSRP_RangeNR_r15 p_RsrpResult_r15 := ?) := //@sic R5s200292 sic@ /* @status */ { /* @status */ message_ := { c1 := { measurementReport := { criticalExtensions := { c1 := { measurementReport_r8 := { measResults := { measId := p_MeasId, measResultPCell := { rsrpResult := ?, rsrqResult := ? }, measResultNeighCells := cr_MeasResultNeighCells_measResultListNR_1Entry(p_PhysCellId, p_RsrqResult_r15, p_Rs_Sinr_Result_r15, p_MeasResultSSB_IndexList_r15, p_Cgi_Info_r15, p_RsrpResult_r15), measResultForECID_r9 := omit, /* @sic R5s110176 BASELINE MOVING 2011 sic@ */ locationInfo_r10 := omit, /* @sic R5s120178 BASELINE MOVING 2012 sic@ */ measResultServFreqList_r10 := omit, /* @sic R5s120178 BASELINE MOVING 2012 sic@ */ measId_v1250 := *, /* @sic BASELINE MOVING 2015 sic@ */ measResultPCell_v1250 := *, /* @sic BASELINE MOVING 2015 sic@ */ measResultCSI_RS_List_r12 := *, /* @sic BASELINE MOVING 2015 sic@ */ measResultForRSSI_r13 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ measResultServFreqListExt_r13 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ measResultSSTD_r13 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ measResultPCell_v1310 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ ul_PDCP_DelayResultList_r13 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ measResultListWLAN_r13 := *, /* @sic R5s160711 BASELINE MOVING 2016 sic@ */ measResultPCell_v1360 := *, /* @sic R5s170597 BASELINE MOVING 2017 sic@ */ measResultListCBR_r14 := *, /* @sic R5s170597 BASELINE MOVING 2017 sic@ */ measResultListWLAN_r14 := *, /* @sic R5s170597 BASELINE MOVING 2017 sic@ */ measResultServFreqListNR_r15 := *, /* @sic R5s180271 BASELINE MOVING 2018 Phase 1 sic@ */ measResultCellListSFTD_r15 := *, /* @sic R5s180271 BASELINE MOVING 2018 Phase 1 sic@ */ logMeasResultListBT_r15 := *, /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ logMeasResultListWLAN_r15 := *, /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ measResultSensing_r15 := *, /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ heightUE_r15 := *, /* @sic R5s180552 BASELINE MOVING 2018 sic@ */ ul_PDCP_DelayValueResultList_r16 := *, /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ measResultForRSSI_NR_r16 := *, /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ uncomBarPreMeasResult_r17:= *, /* @sic R5s221178 BASELINE MOVING 2022 rel-17 sic@ */ coarseLocationInfo_r17 := * /* @sic R5s221178 BASELINE MOVING 2022 rel-17 sic@ */ }, nonCriticalExtension := * //according to 36.508 4.6.1 table 4.6.1-5 } } } } } } }; template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_NR_2Entries_Gap(RRC_TransactionIdentifier p_RRC_TI, ARFCN_ValueEUTRA_r9 p_EUTRA_DL_CarrierFreq_1, AllowedMeasBandwidth p_MeasurementBandwidth_1, integer p_NR_ARFCN_f1, RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 p_SCS, FreqBandIndicatorNR_r15 p_FreqBandIndicatorNR_r15, //@sic R5-194875 sic@ //RSRP_RangeNR_r15 p_NR_RSRP_Thres, ReportConfigInterRAT.reportAmount p_ReportAmount, template (omit) QuantityConfig p_QuantityConfig, template (value) MeasGapConfig p_MeasGapConfig, template (omit) ReportQuantityNR_r15 p_ReportQuantityCellNR_r15 := omit, boolean p_Fr1_Gap_r15, boolean p_Mgta_r15) := /* @status APPROVED (ENDC) */ cs_RRCConnectionReconfiguration_Common(p_RRC_TI, cs_MeasConfig(omit, cs_MeasObjectToAddModList_2Entries(1, //tsc_IdMeasObject_f1, cs_MeasObject_measObjectEUTRA_Common(f_EUTRA_ARFCN_Value(p_EUTRA_DL_CarrierFreq_1), p_MeasurementBandwidth_1), 2, //tsc_IdMeasObject_NR_f1, cs_MeasObject_measObjectNR(p_NR_ARFCN_f1, p_SCS, p_FreqBandIndicatorNR_r15)), omit, cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_B1_NR, cs_508_ReportConfigInterRAT_Periodical_NR(p_ReportAmount, p_ReportQuantityCellNR_r15)), omit, cs_MeasIdToAddModList_1Entry(1, tsc_IdMeasObject_NR_f1, tsc_IdReportConfig_B1_NR_diff), p_QuantityConfig, p_MeasGapConfig, -, -, -, f_ReturnMeasObjectToAddModList_v9e0(p_EUTRA_DL_CarrierFreq_1), p_Fr1_Gap_r15, p_Mgta_r15)); ……. }