3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250267

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3713 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation fails the testcase when run with a device in Multi-PDU mode. The issue here is that the logical channel ID that is used for looopback data is hardcoded by usage of tsc_LCID_SCG_Bearer. If the UE only established the Internet PDU session, then this works fine. However, for a MultiPDU scenario, this results in the loopback data being sent on the DRB associated with Internet PDU session, and the TTCN expects loopback data on the DRB associated with IMS PDU session which never happens. This results in testcase failure. TTCN CR R5s250158 was raised to address the same issue for testcase 7.1.1.6.2.NR5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared and initialized a variable v_LCID: var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); Used v_LCID instead of tsc_LCID_SCG_Bearer in functions f_TC_7_1_1_6_1_NR_TestBody1 and f_TC_7_1_1_6_1_NR_TestBody2. Please see below. This solution is already implemented for 7.1.1.6.2.NR5GC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	17

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | The current TTCN implementation fails the testcase when run with a device in Multi-PDU mode. The issue here is that the logical channel ID that is used for looopback data is hardcoded by usage of tsc_LCID_SCG_Bearer. If the UE only established the Internet PDU session, then this works fine. However, for a MultiPDU scenario, this results in the loopback data being sent on the DRB associated with Internet PDU session, and the TTCN expects loopback data on the DRB associated with IMS PDU session which never happens. This results in testcase failure. TTCN CR R5s250158 was raised to address the same issue for testcase 7.1.1.6.2.NR5GC. |
| Summary of change | Declared and initialized a variable v_LCID: var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); Used v_LCID instead of tsc_LCID_SCG_Bearer in function f_TC_7_1_1_6_1_NR_TestBody1. Please see below. This solution is already implemented for 7.1.1.6.2.NR5GC |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_2; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; //var SubFrameTiming_Type v_TimingUL_7; //@sic R5s240660 R5s240565r1 sic@ var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingDL_Step11b; //@sic R5s240268 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_0_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing, -, cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('111'B)), cs_NR_RedundancyVersionList_SingleRV(0)); //@sic R5s237339 sic@ //@sic R5s240155 sic@ //@sic R5s240563 sic@ var integer v_RetransSchInt := 20; //@sic R5s250065 sic@ var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; timer t_Watchdog := 10.0; //@sic R5s240565 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 //@sic R5s230300 sic@ if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230088 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // @siclog "Steps 1 - 2" siclog@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); t_Watchdog.start; // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 6-7" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 R5-240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ //@sic R5s240660 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); //v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; //@siclog Step 16 is void sic@ //f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); /* //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); */ //@siclog "Step 19-20" siclog@ //@sic R5s240563 sic@ for (i:=1; i< 4; i:=i+1) //Scheduled 3 retransmissions assuming worst case. { v_TimingDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, i * v_RetransSchInt); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL_2),v_N_PRB,v_I_MCS,true,-,-,v_RVList[i]); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_2), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); } //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,2*const_SPS_Period-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 R5s240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,50))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 10 11a3 or 11b2" siclog@ //@sic R5s230088 sic@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_4) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_5_CRNTI) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11a3 or 11b2"); } } // @siclog "Step 15" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_6) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); /* // @siclog "Step 18" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_NR( v_TimingUL_7) )); //f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); // @siclog "Step 21" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_8) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); */ //@sic R5s240563 R5s240660 R5s240565 sic@ // @siclog "Step 18 & 21" siclog@ alt { []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_Any )) { v_Ack_Count := v_Ack_Count +1; if ((v_Ack_Count + v_Nack_Count) < 4) { repeat; } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_Any )) { v_Nack_Count := v_Nack_Count + 1; if ((v_Ack_Count + v_Nack_Count) < 4) { repeat; } else if (v_Ack_Count == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } } // @siclog "Step 23" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_9) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // @siclog "Step 25 and 28" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_10) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_11) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); t_Watchdog.stop; //@sic R5s240565 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); return (v_TimingDL); }

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_2; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; //var SubFrameTiming_Type v_TimingUL_7; //@sic R5s240660 R5s240565r1 sic@ var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingDL_Step11b; //@sic R5s240268 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_0_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing, -, cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('111'B)), cs_NR_RedundancyVersionList_SingleRV(0)); //@sic R5s237339 sic@ //@sic R5s240155 sic@ //@sic R5s240563 sic@ var integer v_RetransSchInt := 20; //@sic R5s250065 sic@ var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //WA# timer t_Watchdog := 10.0; //@sic R5s240565 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 //@sic R5s230300 sic@ if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230088 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // @siclog "Steps 1 - 2" siclog@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); t_Watchdog.start; // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 6-7" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 R5-240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ //@sic R5s240660 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); //v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; //@siclog Step 16 is void sic@ //f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); /* //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); */ //@siclog "Step 19-20" siclog@ //@sic R5s240563 sic@ for (i:=1; i< 4; i:=i+1) //Scheduled 3 retransmissions assuming worst case. { v_TimingDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, i * v_RetransSchInt); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL_2),v_N_PRB,v_I_MCS,true,-,-,v_RVList[i]); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_2), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); } //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,2*const_SPS_Period-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 R5s240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,50))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 10 11a3 or 11b2" siclog@ //@sic R5s230088 sic@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_4) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_5_CRNTI) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11a3 or 11b2"); } } // @siclog "Step 15" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_6) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); /* // @siclog "Step 18" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_NR( v_TimingUL_7) )); //f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); // @siclog "Step 21" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_8) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); */ //@sic R5s240563 R5s240660 R5s240565 sic@ // @siclog "Step 18 & 21" siclog@ alt { []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_Any )) { v_Ack_Count := v_Ack_Count +1; if ((v_Ack_Count + v_Nack_Count) < 4) { repeat; } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_Any )) { v_Nack_Count := v_Nack_Count + 1; if ((v_Ack_Count + v_Nack_Count) < 4) { repeat; } else if (v_Ack_Count == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } } // @siclog "Step 23" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_9) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // @siclog "Step 25 and 28" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_10) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_11) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); t_Watchdog.stop; //@sic R5s240565 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); return (v_TimingDL); }

## Change 2

| Function name | f_TC_7_1_1_6_1_NR_TestBody2 |
| --- | --- |
| Reason for change | Same as reason mentioned in change 1 |
| Summary of change | Same as reason mentioned in change 1 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC {//@sic R5-245514 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL ; var SubFrameTiming_Type v_TimingUL_1; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks timer t_Watchdog := 1.0; // Re-nitialise Nprb and Imcs for TBsize=352 if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200);//@sic R5-245514 sic@ v_TimingUL_1:= v_TimingDL; select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; //@sic R5s221197 sic@ v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 31" siclog@ //@sic R5s240268 R5-245514 sic@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); //10ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_LclRel); // @siclog "Step 32" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ? , cr_TimingInfo_NR( v_TimingUL_1) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 32"); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 32"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); };

After Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC {//@sic R5-245514 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL ; var SubFrameTiming_Type v_TimingUL_1; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks timer t_Watchdog := 1.0; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //WA# // Re-nitialise Nprb and Imcs for TBsize=352 if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200);//@sic R5-245514 sic@ v_TimingUL_1:= v_TimingDL; select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; //@sic R5s221197 sic@ v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 31" siclog@ //@sic R5s240268 R5-245514 sic@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) }; //WA# v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); //10ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_LclRel); // @siclog "Step 32" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ? , cr_TimingInfo_NR( v_TimingUL_1) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 32"); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 32"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); };