3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250270

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3714 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the 38.523-1 Table 11.3.6.3.2-3, at Step 7 the SS transmits the IP PDU on the DRB asosociated with the Internet PDU session. This would mean that after Step 1N, an Internet PDU session must be established. The current TTCN does not ensure that an Internet PDU session is established after Step 1N. This needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After Step 1N, checked whether the Internet PDU Session is established or not, and based on that implemented TTCN to establish Internet PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | fl_TC_11_3_6and6a_TestBody |
| --- | --- |
| Reason for change | According to the 38.523-1 Table 11.3.6.3.2-3, at Step 7 the SS transmits the IP PDU on the DRB asosociated with the Internet PDU session. This would mean that after Step 1N, an Internet PDU session must be established. The current TTCN does not ensure that an Internet PDU session is established after Step 1N. This needs to be implemented. |
| Summary of change | After Step 1N, checked whether the Internet PDU Session is established or not, and based on that implemented TTCN to establish Internet PDU Session. |
| TTCN module | UAC_NR5GC.ttcn |

Before Change:

function fl_TC_11_3_6and6a_TestBody() runs on NR5GC_PTC { var Octet_Type v_IP_PDU_delay := '02'O; //@sic R5s210669, R5-216194 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (value) QosFlow_Identification_Type v_QoS_Id; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var float v_TimerValue := 21.0; //@sic R5-213506, R5-223437 sic@ var float v_TimerValue1 := 10.0; var float v_TimerValue2 := 30.0; //@sic R5-201227 sic@ timer t_WaitTime; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -82) //@sic R5-236562 sic@ }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -82), //@sic R5-236562 sic@ cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 1A" siclog@ //@sic R5-213506, R5-223437 sic@ //Void //@siclog "Step 1AA" siclog@ //@sic R5-223437 sic@ //Check: Does the UE transmit a RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mcs-PriorityAccess? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mcs_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 1AA"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1AA"); } //@siclog "Steps 1AB - 1AE" siclog@ //@sic R5-223437 sic@ //Steps 2 to 5 of the mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 12. f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Steps 1AF - 1AI" siclog@ //@sic R5-223437 sic@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are performed f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Steps 1AJ - 1AK" siclog@ //@sic R5-223437 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); //@siclog "Step 1AL" siclog@ //@sic R5-223437 sic@ //The SS transmits one IP PDU on the DRB associated with the Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); //@sic R5-244664 sic@ v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})); //@siclog "Step 1AM" siclog@ //The SS transmits an RRCRelease message and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell12); //@siclog "Step 1AN" siclog@ //@sic R5-223437 sic@ //Check: Does the UE transmit an RRCSetupRequest message within 21s on NR Cell 12? (Note 4) //Note 4: The wait time 21s is selected to be more than T390 maximum = (0.7 + 0.6 * rand) * uac-BarringTime(16s) = 20.7s when 'rand' takes the maximum value of 0.99. t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 1AN"); } [] t_WaitTime.timeout{} } //@siclog "Step 1B" siclog@ //@sic R5-213506 sic@ //SS changes SIB1 of NR Cell 12 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit); // @sic R5s210669 sic@ f_NR_ModifySysinfo(nr_Cell12); // @sic R5s210669 sic@ //@sic R5-223437 sic@ //@siclog "Step 1C" siclog@ //Start Timer=30 sec. //Note 3: The wait time 30s is selected to cover (0.7 + 0.6 * rand) * uac-BarringTime(16s) = 20.7s + 2s (IP PDU delay timer) + 5.12s (modification period) = 27.92s rounded up to 28s when 'rand' takes the maximum value of 0.99. //EXCEPTION: Steps 1Da1-1Db16 describe optional behaviour that depends on the UE implementation. t_WaitTime.start(v_TimerValue2); alt { //@siclog "Steps 1Da1 - 1Da2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message and establishmentCause is set to mo-Data? //Stop Timer=30. [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1Da1"); } //@siclog "Step 1Db1" siclog@ //Timer=30 sec expires [] t_WaitTime.timeout { //@siclog "Steps 1Db2 - 1Db9" siclog@ //Steps 1-8 of the generic procedure for NR RRC Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell12); //@siclog "Steps 1Db10 - 1Db11" siclog@ //The SS transmits an OPEN UE TEST LOOP message. //The UE transmits an OPEN UE TEST LOOP COMPLETE message. f_NR5GC_OpenUE_TestLoopMode(nr_Cell12); //@siclog "Steps 1Db12 - 1Db13" siclog@ //The SS transmits a CLOSE UE TEST LOOP message. //The SS transmits a CLOSE UE TEST LOOP message. f_NR5GC_CloseUE_TestLoopModeB(nr_Cell12, v_IP_PDU_delay); //@siclog "Step 1Db14" siclog@ //The SS transmits one IP PDU on the DRB associated with the Internet PDU session. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})); //@siclog "Step 1Db15" siclog@ //The SS transmits an RRCRelease message and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell12); //@siclog "Step 1Db16" siclog@ //Check: Does the UE transmit an RRCSetupRequest message and establishmentCause is set to mo-Data? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1Db16"); } } //@siclog "Steps 1E - 1I" siclog@ //@sic R5-223437 sic@ //Steps 3-7 of the generic procedure for NR RRC Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B)) { // @sic R5s210669, R5s220850 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg, -, -, -, -, -, -, false); //@sic R5-223437 sic@ //EXCEPTION: Steps 1J and 1K can occur in any order. //@siclog "Step 1J" siclog@ //The UE transmits an RRCReconfigurationComplete message. //@siclog "Step 1K" siclog@ //The UE loops back the IP PDU on the DRB associated with the Internet PDU session. interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell12, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){} [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})){} } //@siclog "Step 1L" siclog@ //@sic R5-223437 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff (nr_Cell12, STATE_CONNECTED_3A); //@siclog "Step 1M" siclog@ //@sic R5-223437 sic@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Steps 1N - 1O" siclog@ //@sic R5-223437 sic@ //The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1 with condition Test Mode = On. //EXCEPTION: Step 1O is performed in p pc_noOf_PDUsNewConnection > 0. //The generic procedure in TS 38.508-1 [4] Table 4.5.2.2-4 for UE-requested PDU session establishment performs registration on NR Cell 1 and then release the RRC Connection. f_NR5GC_SwitchOnAndResetIMS(); f_NR5GC_RRC_IdleState1N_Def (nr_Cell1, TESTModeB_ON, -, -, -, '0101'B); //@sic R5s240122 sic@ //@siclog "Step 1P" siclog@ //@sic R5-223437 sic@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 2" siclog@ <<SKIPPED CODE>>

After Change:

function fl_TC_11_3_6and6a_TestBody() runs on NR5GC_PTC { var Octet_Type v_IP_PDU_delay := '02'O; //@sic R5s210669, R5-216194 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (omit) NR_DRBInfo_Type v_DRBInfo; //WA#WI1432563 WA#11_6_3 var template (value) QosFlow_Identification_Type v_QoS_Id; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var float v_TimerValue := 21.0; //@sic R5-213506, R5-223437 sic@ var float v_TimerValue1 := 10.0; var float v_TimerValue2 := 30.0; //@sic R5-201227 sic@ timer t_WaitTime; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -82) //@sic R5-236562 sic@ }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -82), //@sic R5-236562 sic@ cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 1A" siclog@ //@sic R5-213506, R5-223437 sic@ //Void //@siclog "Step 1AA" siclog@ //@sic R5-223437 sic@ //Check: Does the UE transmit a RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mcs-PriorityAccess? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mcs_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 1AA"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1AA"); } //@siclog "Steps 1AB - 1AE" siclog@ //@sic R5-223437 sic@ //Steps 2 to 5 of the mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 12. f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Steps 1AF - 1AI" siclog@ //@sic R5-223437 sic@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are performed f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Steps 1AJ - 1AK" siclog@ //@sic R5-223437 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); //@siclog "Step 1AL" siclog@ //@sic R5-223437 sic@ //The SS transmits one IP PDU on the DRB associated with the Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); //@sic R5-244664 sic@ v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})); //@siclog "Step 1AM" siclog@ //The SS transmits an RRCRelease message and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell12); //@siclog "Step 1AN" siclog@ //@sic R5-223437 sic@ //Check: Does the UE transmit an RRCSetupRequest message within 21s on NR Cell 12? (Note 4) //Note 4: The wait time 21s is selected to be more than T390 maximum = (0.7 + 0.6 * rand) * uac-BarringTime(16s) = 20.7s when 'rand' takes the maximum value of 0.99. t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 1AN"); } [] t_WaitTime.timeout{} } //@siclog "Step 1B" siclog@ //@sic R5-213506 sic@ //SS changes SIB1 of NR Cell 12 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit); // @sic R5s210669 sic@ f_NR_ModifySysinfo(nr_Cell12); // @sic R5s210669 sic@ //@sic R5-223437 sic@ //@siclog "Step 1C" siclog@ //Start Timer=30 sec. //Note 3: The wait time 30s is selected to cover (0.7 + 0.6 * rand) * uac-BarringTime(16s) = 20.7s + 2s (IP PDU delay timer) + 5.12s (modification period) = 27.92s rounded up to 28s when 'rand' takes the maximum value of 0.99. //EXCEPTION: Steps 1Da1-1Db16 describe optional behaviour that depends on the UE implementation. t_WaitTime.start(v_TimerValue2); alt { //@siclog "Steps 1Da1 - 1Da2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message and establishmentCause is set to mo-Data? //Stop Timer=30. [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1Da1"); } //@siclog "Step 1Db1" siclog@ //Timer=30 sec expires [] t_WaitTime.timeout { //@siclog "Steps 1Db2 - 1Db9" siclog@ //Steps 1-8 of the generic procedure for NR RRC Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell12); //@siclog "Steps 1Db10 - 1Db11" siclog@ //The SS transmits an OPEN UE TEST LOOP message. //The UE transmits an OPEN UE TEST LOOP COMPLETE message. f_NR5GC_OpenUE_TestLoopMode(nr_Cell12); //@siclog "Steps 1Db12 - 1Db13" siclog@ //The SS transmits a CLOSE UE TEST LOOP message. //The SS transmits a CLOSE UE TEST LOOP message. f_NR5GC_CloseUE_TestLoopModeB(nr_Cell12, v_IP_PDU_delay); //@siclog "Step 1Db14" siclog@ //The SS transmits one IP PDU on the DRB associated with the Internet PDU session. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})); //@siclog "Step 1Db15" siclog@ //The SS transmits an RRCRelease message and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell12); //@siclog "Step 1Db16" siclog@ //Check: Does the UE transmit an RRCSetupRequest message and establishmentCause is set to mo-Data? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1Db16"); } } //@siclog "Steps 1E - 1I" siclog@ //@sic R5-223437 sic@ //Steps 3-7 of the generic procedure for NR RRC Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B)) { // @sic R5s210669, R5s220850 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg, -, -, -, -, -, -, false); //@sic R5-223437 sic@ //EXCEPTION: Steps 1J and 1K can occur in any order. //@siclog "Step 1J" siclog@ //The UE transmits an RRCReconfigurationComplete message. //@siclog "Step 1K" siclog@ //The UE loops back the IP PDU on the DRB associated with the Internet PDU session. interleave { [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell12, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){} [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell12, v_QoS_Id, {v_IPData})){} } //@siclog "Step 1L" siclog@ //@sic R5-223437 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. f_NR_UE_DeRegisterOnSwitchOff (nr_Cell12, STATE_CONNECTED_3A); //@siclog "Step 1M" siclog@ //@sic R5-223437 sic@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Steps 1N - 1O" siclog@ //@sic R5-223437 sic@ //The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1 with condition Test Mode = On. //EXCEPTION: Step 1O is performed in p pc_noOf_PDUsNewConnection > 0. //The generic procedure in TS 38.508-1 [4] Table 4.5.2.2-4 for UE-requested PDU session establishment performs registration on NR Cell 1 and then release the RRC Connection. f_NR5GC_SwitchOnAndResetIMS(); f_NR5GC_RRC_IdleState1N_Def (nr_Cell1, TESTModeB_ON, -, -, -, '0101'B); //@sic R5s240122 sic@ //WA#WI1432563 WA#11_6_3 : Change start v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0101'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs(nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}); f_NR5GC_PDUSessionEstablishment (nr_Cell1, 1); f_NR_RRCRelease(nr_Cell1); } //WA#WI1432563 WA#11_6_3 : Change stop //@siclog "Step 1P" siclog@ //@sic R5-223437 sic@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.6.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 2" siclog@ <<SKIPPED CODE>>