3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250298

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3723 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IsDNNForIMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation cause that DNN IMS.mnc001.mcc001.gprs will be incorrectly consider like non IMS DNN. Details are explained below for each change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update f_IsDNNForIMS and f_DomainName_DecodeToList that IMS.mnc001.mcc001.gprs will be consider like IMS DNN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IRAT test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Template name | f_IsDNNForIMS |
| --- | --- |
| Reason for change | With current implementation the else if (v_FieldCnt == 4) will never be reached, because is inside if (v_FieldCnt == 1) statement |
| Summary of change | Update the code that if if (v_FieldCnt == 1) is false then else if (v_FieldCnt == 4) will be check. |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |

Before Change:

function f_IsDNNForIMS(octetstring p_DNNFromReq) return boolean { var CharStringList_Type v_FieldList := f_DomainName_DecodeToList(p_DNNFromReq); var integer v_FieldCnt := lengthof(v_FieldList); if (v_FieldCnt == 1) { // DNN = IMS if (match (v_FieldList[0], "IMS")) { return true; } else if (v_FieldCnt == 4) { if ((match (v_FieldList[0], "IMS")) and (match (v_FieldList[3], "gprs"))) { // DNN = IMS.mccMCC.mncMNC.gprs return true; } } } // if none of the above return false; }

After Change:

function f_IsDNNForIMS(octetstring p_DNNFromReq) return boolean { var CharStringList_Type v_FieldList := f_DomainName_DecodeToList(p_DNNFromReq); var integer v_FieldCnt := lengthof(v_FieldList); if (v_FieldCnt == 1) { // DNN = IMS if (match (v_FieldList[0], "IMS")) { return true; } } else if (v_FieldCnt == 4) { if ((match (v_FieldList[0], "IMS")) and (match (v_FieldList[3], "gprs"))) { // DNN = IMS.mccMCC.mncMNC.gprs return true; } } // if none of the above return false; }

## Change 2

| Template name | f_DomainName_DecodeToList |
| --- | --- |
| Reason for change | Current implementation will incorrectly decode DNN to the list for IMS.mnc001.mcc001.gprs. It will be not included .gprs part |
| Summary of change | Update function that also .grps part will be included. |
| TTCN module | CommonDefs.ttcn |

Before Change:

function f_DomainName_DecodeToList(octetstring p_OctetString) return CharStringList_Type { var CharStringList_Type v_LableList := {} var integer i := 0; var integer k := 0; var integer v_StringLength := lengthof(p_OctetString); var integer v_LableLength; var octetstring v_EncodedLable; while (true) { v_LableLength := oct2int(p_OctetString[i]); i := i + 1; if ((v_LableLength > 0) and (i + v_LableLength < v_StringLength)) { v_EncodedLable := substr(p_OctetString, i, v_LableLength); i := i + v_LableLength; v_LableList[k] := oct2char(v_EncodedLable); k := k + 1; } else { break; } } return v_LableList; }

After Change:

function f_DomainName_DecodeToList(octetstring p_OctetString) return CharStringList_Type { var CharStringList_Type v_LableList := {} var integer i := 0; var integer k := 0; var integer v_StringLength := lengthof(p_OctetString); var integer v_LableLength; var octetstring v_EncodedLable; while (true) { v_LableLength := oct2int(p_OctetString[i]); i := i + 1; if ((v_LableLength > 0) and (i + v_LableLength <= v_StringLength)) { //Anritsu TTCN CR required for 11.4.11 v_EncodedLable := substr(p_OctetString, i, v_LableLength); i := i + v_LableLength; v_LableList[k] := oct2char(v_EncodedLable); k := k + 1; } else { break; } } return v_LableList; }