3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250336

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4930 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE-IRAT testcase 6.2.3.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, the coordination message to GERAN is sent only when the UE has fully performed the TAU procedure on the LTE cell. Until this coordination message is received on GERAN PTC, the channel keeps transmitting and this reults in errors happening due to RLC timeouts during non terminated DL data transfer on GERAN side. To avoid this, it is proposed to send the coordination message to GERAN as soon as the UE transmits the rrcConnectionREQ on the LTE cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of Step 5 in TTCN. Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase remains unstable, and can unfailrly fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_3_21_EUTRA |
| --- | --- |
| Reason for change | In the current TTCN implementation, the coordination message to GERAN is sent only when the UE has fully performed the TAU procedure on the LTE cell. Until this coordination message is received on GERAN PTC, the channel keeps transmitting and this reults in errors happening due to RLC timeouts during non terminated DL data transfer on GERAN side. To avoid this, it is proposed to send the coordination message to GERAN as soon as the UE transmits the rrcConnectionREQ on the LTE cell. |
| Summary of change | Updated the implementation of Step 5 in TTCN. Please see below |
| TTCN module | InterRat_CellReSelection_GtoE.ttcn |
| MCCTF160 comment |  |

Before Change:

function f_TC_6_2_3_21_EUTRA() runs on EUTRA_PTC { /* Inter-RAT cell reselection / From GPRS Packet_Transfer (NC0 Mode) to E-UTRA */ //Initialise all cells, security and mobile parameters f_EUTRA_Init(c5);//EUTRA-GERAN configuration f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -70); //Create and configure Cell 1 f_EUTRA_CellConfig_Def(eutra_Cell1); //Receive Authentication parameters updated in the RAU on GERAN side f_EUTRA_InterRAT_InitialiseAuthParams(GERAN); f_IRAT_WaitForCoOrd_Trigger(GERAN); //@siclog "Step 4" siclog@ //Turn cell on f_EUTRA_SetCellPower(eutra_Cell1, -70); f_EUTRA_TestBody_Set(true); f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup (eutra_Cell1); //@sic R5-185004 sic@ //@siclog "Step 5" siclog@ //Check that UE camps on Cell 1 with the generic test procedure TS 36.508 subclause 6.4.2.7A f_EUTRA_IdleMode_TAU_UponMobility_FromOtherRAT(eutra_Cell1, true, false, omit); // @sic R5-123734, R5-1350576, R5-140792 sic@ // Tell the GERAN component that the UE has camped f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); // @sic R5s190131 sic@ //@siclog "Step 2a1 - 2b1" siclog@ f_EUTRA_OptionalMultiPDN_IMSRegistration(eutra_Cell1); //@sic R5-185004 sic@ // Test case now over f_EUTRA_TestBody_Set(false); // Switch off UE and expect Detach Request f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); // @sic R5s210422, R5s210814 sic@ }

After Change:

function f_TC_6_2_3_21_EUTRA() runs on EUTRA_PTC { /* Inter-RAT cell reselection / From GPRS Packet_Transfer (NC0 Mode) to E-UTRA */ //Initialise all cells, security and mobile parameters f_EUTRA_Init(c5);//EUTRA-GERAN configuration f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -70); //Create and configure Cell 1 f_EUTRA_CellConfig_Def(eutra_Cell1); //Receive Authentication parameters updated in the RAU on GERAN side f_EUTRA_InterRAT_InitialiseAuthParams(GERAN); f_IRAT_WaitForCoOrd_Trigger(GERAN); //@siclog "Step 4" siclog@ //Turn cell on f_EUTRA_SetCellPower(eutra_Cell1, -70); f_EUTRA_TestBody_Set(true); f_EUTRA_OptionalMultiPDN_IMSRegistration_PTCSetup (eutra_Cell1); //@sic R5-185004 sic@ //@siclog "Step 5" siclog@ //Check that UE camps on Cell 1 with the generic test procedure TS 36.508 subclause 6.4.2.7A f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell1, mo_Signalling); //WA#WI1440846 WA#6_2_3_21 // Tell the GERAN component that the UE has camped f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); //WA#WI1440846 WA#6_2_3_21 f_EUTRA_TrackingAreaUpdateFromAnotherRAT_WithoutRRCConneReq(eutra_Cell1, //WA#WI1440846 WA#6_2_3_21 tsc_RRC_TI_Def, true, false, omit, // @sic R5-123734 sic@ rrcConnectionRelease, NORMAL); //@siclog "Step 2a1 - 2b1" siclog@ f_EUTRA_OptionalMultiPDN_IMSRegistration(eutra_Cell1); //@sic R5-185004 sic@ // Test case now over f_EUTRA_TestBody_Set(false); // Switch off UE and expect Detach Request f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); // @sic R5s210422, R5s210814 sic@ }