3GPP TSG-RAN5 Meeting #2026-TTCN email	R5s260002

# Online,  12th Dec 2025 - 31st Dec 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3789 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_Rf_Parameters_NRBand_PICS_Supp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is incorrect logic and testcase failed with "Acc. to TS38.306 cl. 4.2.7.2: The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent" when UE report type2-PUSCH-RepetitionMultiSlots-v1650 = TRUE and type2-PUSCH-RepetitionMultiSlots = Omitt |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the logic that TC will not fail when UE report type2-PUSCH-RepetitionMultiSlots-v1650 = TRUE and type2-PUSCH-RepetitionMultiSlots = Omitt |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk50 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_Check_Rf_Parameters_NRBand_PICS_Supp |
| --- | --- |
| Reason for change | There is incorrect logic and testcase failed with "Acc. to TS38.306 cl. 4.2.7.2: The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent" when UE report type2-PUSCH-RepetitionMultiSlots-v1650 = TRUE and type2-PUSCH-RepetitionMultiSlots = Omitt |
| Summary of change | Update the logic that TC will not fail when UE report type2-PUSCH-RepetitionMultiSlots-v1650 = TRUE and type2-PUSCH-RepetitionMultiSlots = Omitt |
| TTCN module | NR_CapabilityFunctions.ttcn |

Before Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 R5-246407 R5-251949 R5-255069 R5-254821 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; … v_Received_NRCapabilityMsg := p_NRCapability; v_Type2_PUSCH_RepetitionMultiSlots := isvalue(v_Received_NRCapabilityMsg.phy_Parameters.phy_ParametersCommon.type2_PUSCH_RepetitionMultiSlots); // Refers to type2-PUSCH-RepetitionMultiSlots … if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].type2_PUSCH_RepetitionMultiSlots_v1650)) { // @sic R5s250367 sic@ v_Found_PUSCH_RepetitionMultiSlots_v1650 := true; } … if (not (v_Found_PUSCH_RepetitionMultiSlots_v1650 and v_Type2_PUSCH_RepetitionMultiSlots)){ // @sic R5s250367 sic@ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2: The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent"); }

After Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 R5-246407 R5-251949 R5-255069 R5-254821 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; … v_Received_NRCapabilityMsg := p_NRCapability; v_Type2_PUSCH_RepetitionMultiSlots := isvalue(v_Received_NRCapabilityMsg.phy_Parameters.phy_ParametersCommon.type2_PUSCH_RepetitionMultiSlots); // Refers to type2-PUSCH-RepetitionMultiSlots … if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].type2_PUSCH_RepetitionMultiSlots_v1650)) { // @sic R5s250367 sic@ v_Found_PUSCH_RepetitionMultiSlots_v1650 := true; } … if (v_Found_PUSCH_RepetitionMultiSlots_v1650 and v_Type2_PUSCH_RepetitionMultiSlots){ // @sic R5s250367 sic@ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2: The UE only includes type2-PUSCH-RepetitionMultiSlots-v1650 if type2-PUSCH-RepetitionMultiSlots is absent"); }