3GPP TSG-RAN5 Meeting #2026-TTCN email	R5s260008

# Online, , 12th Dec 2025 - 31st Dec 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4955 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.5a (NGSO mode) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For NB-IoT NTN, in NGSO mode, at step 4, for sending UL GRANT, ss should count next available npddch period. The offset is calculated by v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer - f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1), where for NGSO, the T_ntn offse equls to Kmac+Koffset as 8, which offset value is smaller and cause the pdcch period sending out of active window, UE in inactive and UE will not detect NPDCCH and miss the ul grant to loopback data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjust one pdcch cycle in advance to make sure ul grant timing is available in Drx_ULRetransmissionTimer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change to the f_TC_22_3_1_5a_NBIOT	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_5a_NBIOT

| Function name | f_TC_22_3_1_5a_NBIOT () |
| --- | --- |
| Reason for change | For NB-IoT NTN, in NGSO mode, at step 4, for sending UL GRANT, ss should count next available npddch period. The offset is calculated by v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer - f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1), where for NGSO, the T_ntn offse equls to Kmac+Koffset as 8, which offset value is smaller and cause the pdcch period sending out of active window, UE in inactive and UE will not detect NPDCCH and miss the ul grant to loopback data. |
| Summary of change | Adjust one pdcch cycle in advance to make sure ul grant timing is available in Drx_ULRetransmissionTimer |
| TTCN module | NBIOT_MAC_NTN.ttcn |

Before Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { ……. //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) v_Drx_ULRetransmissionTimer := v_Drx_ULRetransmissionTimer - 10; // x = 10 @sic R5s240411 sic@ v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(valueof(v_TimingInfo.SubFrame), v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer - f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1) ); // @sic R5s240411, R5s240430 sic@ //WA#WI=1469950 22.3.1.5a pdcch occasion v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); // + delatPDCCH @sic R5s240411 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@sic R5s240411 sic@ //@siclog "Step 5" siclog@ //Void. …… }

After Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { ……. //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) v_Drx_ULRetransmissionTimer := v_Drx_ULRetransmissionTimer - 10; // x = 10 @sic R5s240411 sic@ v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(valueof(v_TimingInfo.SubFrame), v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer - f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1) - f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters)); // @sic R5s240411, R5s240430 sic@ //WA#WI=1469950 22.3.1.5a pdcch occasion v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); // + delatPDCCH @sic R5s240411 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@sic R5s240411 sic@ //@siclog "Step 5" siclog@ //Void. …… }