3GPP TSG-RAN5 Meeting #2026-TTCN email	R5s260009

# Online, , 12th Dec 2025 - 31st Dec 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3790 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.3.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the changes implemented as part of R5s250501, the SS is now creating 3 PDCP DATA PDUs as below PDCP Data PDU #1 Size = 75 octets PDCP Data PDU #2 Size = 79 octets PDCP Data PDU #3 Size = 83 octets Prior to the implementation of this CR, PDCP Data PDUs #1, #2, and #3 were consistently sized at 67 octets. When the CR R5s250501 was implemented, the procedural requirements in Step 8A were not updated accordingly. In this step, the SS allocates three UL grants every 20ms; however, the grant sizes were not scaled to ensure the UE can return each received PDCP PDU within a single looped-back PDCP PDU Current UL grant configuration is suitable for 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 74 octets. But This allocation is insufficient for the UE to perform the loopback of the new, larger PDCP Data PDUs (up to 83 octets). So it is proposed to increase the UL grant size to accommodate maximum PDCP Data PDU size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_ResourceAllocation_UplinkGrant , p_TBS is increased from p_TBS = 608 p_TBS = 736 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.3.ENDC, 7.1.3.5.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk50 related to the title of this CR.

Contact:	Parikshit Bhise
                       parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_5_3_NR_TestBody_Step7to9 |
| --- | --- |
| Reason for change | Due to the changes implemented as part of R5s250501, the SS is now creating 3 PDCP DATA PDUs as below PDCP Data PDU #1 Size = 75 octets PDCP Data PDU #2 Size = 79 octets PDCP Data PDU #3 Size = 83 octets Prior to the implementation of this CR, PDCP Data PDUs #1, #2, and #3 were consistently sized at 67 octets. When the CR R5s250501 was implemented, the procedural requirements in Step 8A were not updated accordingly. In this step, the SS allocates three UL grants every 20ms; however, the grant sizes were not scaled to ensure the UE can return each received PDCP PDU within a single looped-back PDCP PDU Current UL grant configuration is suitable for 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 74 octets. But This allocation is insufficient for the UE to perform the loopback of the new, larger PDCP Data PDUs (up to 83 octets). So it is proposed to increase the UL grant size to accommodate maximum PDCP Data PDU size. |
| Summary of change | In the function f_NR_ResourceAllocation_UplinkGrant , p_TBS is increased from p_TBS = 608 p_TBS = 736 |
| TTCN module | PDCP_TC_Common_NR.ttcn |

Before Change

function f_TC_7_1_3_5_3_NR_TestBody_Step7to9(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId, NR_CellId_Type p_PCellId, SubFrameTiming_Type p_Timing) runs on NR_BASE_PTC { << SKIPPED CODE >> //@siclog "Step 8A" siclog@ // After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell for Connectivity (EN-DC) and PCell for Connectivity (NR-DC). // 44 resp. 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 54 resp. 74 octets if (f_IpAddressIsIPv4(f_LoopbackModeB_IP_Address_UE())) { // @sic R5s200153 sic@ v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(432, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ } else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); } v_TimingUL := f_NR_GetNextSendOccasion_UL(v_NRCellId); // @sic R5s230377 sic@ f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(v_NRCellId, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 3); //@sic R5-197255, R5s200029, R5s230377 sic@ << SKIPPED CODE >> }

After Change

function f_TC_7_1_3_5_3_NR_TestBody_Step7to9(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId, NR_CellId_Type p_PCellId, SubFrameTiming_Type p_Timing) runs on NR_BASE_PTC { << SKIPPED CODE >> //@siclog "Step 8A" siclog@ // After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell for Connectivity (EN-DC) and PCell for Connectivity (NR-DC). // 44 resp. 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 54 resp. 74 octets if (f_IpAddressIsIPv4(f_LoopbackModeB_IP_Address_UE())) { // @sic R5s200153 sic@ v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(432, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ } else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(736, v_NR_UplinkBWP); //WA#MRT1CD-14970 } << SKIPPED CODE >> }