3GPP TSG-RAN5 Meeting #2026-TTCN email	R5s260011

# Online, , 12th Dec 2025 - 31st Dec 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3792 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of the TTCN function f_TC_8_1_2_1_1_NR5GC_TestBody , at steps 12B-12C DRB Id extraction is done incorrectly . v_DRBId := oct2int(f_NR_GetSessionId_ForLastPDUSession()); the above function should be used to extract the session id , and further DRB id associated with the session id to be extracted . This implementation was introduced as part of R5s250454 which is also resulting in run time error |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extract session id , and from the session id , associated DRB id is calculated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Results in run time issue when using the session id In place of DRB id |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk50 related to the title of this CR.

Contact:	Parikshit Bhise
                       parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current implementation of the TTCN function f_TC_8_1_2_1_1_NR5GC_TestBody , at steps 12B-12C DRB Id extraction is done incorrectly . v_DRBId := oct2int(f_NR_GetSessionId_ForLastPDUSession()); the above function should be used to extract the session id , and further DRB id associated with the session id to be extracted . This implementation was introduced as part of R5s250454 which is also resulting in run time error |
| Summary of change | Extract session id , and from the session id , associated DRB id is calculated |
| TTCN module | \NR5GC\8_1_2\RRC_Reconfiguration_NR5GC.ttcn |

Before Change

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ v_DRBId := oct2int(f_NR_GetSessionId_ForLastPDUSession()); // @sic R5s220779 R5-200256 R5s250454 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); << SKIPPED CODE >> }//end of f_TC_8_1_2_1_1_NR5GC_TestBody

After Change

import from CommonDefs all; function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ //v_DRBId := oct2int(f_NR_GetSessionId_ForLastPDUSession()); // @sic R5s220779 R5-200256 R5s250454 sic@ //WA#MRT1CD-15064 var integer v_PDUSessionIdToRelease := oct2int(f_NR_GetSessionId_ForLastPDUSession()); // @sic R5s220779 R5-200256 sic@ //WA#MRT1CD-15064 // Get DRB list for this PDU session and extract the default DRB ID var IntegerList_Type v_ListOfDRBs := f_NR_GetDRBList_ForGivenPDUSession(v_PDUSessionIdToRelease); //WA#MRT1CD-15064 v_DRBId := f_NR_GetDefaultDRB_InDRBList(v_ListOfDRBs); //WA#MRT1CD-15064 v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); << SKIPPED CODE >> }//end of f_TC_8_1_2_1_1_NR5GC_TestBody