3GPPRAN5 – TTCN Workshop #39                                	 R5w170305

# Sophia Antipolis, France, 19th October 2017

# Agenda item:	4

# Source: 	MCC TF160

Title: 	4Rx_AP_DL: ASP updates

Document for:	Discussion and endorsement

- Introduction:

At RAN5#76, 2 new TBS MAC test cases 7.1.7.1.6a & 7.1.7.1.12a were added by R5-174572 for the Test Work Item LTE DL 4 Rx antenna ports (AP).

According to the associated TS 36.508 CR R5-175050, for the LTE DL 4Rx AP test cases, the SS shall send to the UE the IE MaxLayersMimo-r10 = fourLayers, to enable the UE to compute the total number of soft channel bits as per TS 36.212. Similarly, the SS shall be given this information by TTCN.

- Discussion:

ASP change needed for 4RX support to provide SS the 4RX specific antenna info:

type record AntennaInfoDedicated_R10andLater_Type { AntennaInfoDedicated_r10 antennaInfo, AntennaInfoUL_r10 antennaInfoUL optional, AntennaInfoDedicated_v10i0 antennaInfoDedicated_v10i0 optional, AntennaInfoDedicated_v1250 antennaInfoDedicated_v1250 optional, AntennaInfoDedicated_v1430 AntennaInfoDedicated_v1430 optional };

Note: Though not required in the context of LTE DL 4 Rx AP, it is also suggested to add AntennaInfoDedicated_v1430 in the record for completeness of AntennaInfoDedicated_R10andLater_Type definition.