3GPPRAN5 - TTCN Workshop #46	R5w190202

# Sophia Antipolis, France, 2nd July 2019

Agenda Item:	10 (5G)

Source:	MCC TF160

Title:	Guidelines on 5GS test execution

Document for:	Endorsement

# 1	Introduction

Currently the execution guidelines are not documented in 38.523-3, that clause provides the test case execution limitations when running test cases on bands not having 4 frequencies available.

# 2	Proposal

It is proposed to add the clause 11 presented in TS 38.523-3.

That proposal is based on TS 38.508-1-g00 (June-19) and TS 38.523-1-g00 (June19); only delivered test cases are handled.

Currently, there is no limitation for the delivered ENDC and I-RAT test cases.

NOTE: 	The use of frequencies is not optimised in TS 38.523-1 for TC 6.1.1.8 and 9.1.5.1.2. A discussion is ongoing with the prose author, depending on the outcome of the discussion and RAN5 prose CRs agreement, these TC might be deleted from the list.

# 11	Guidelines on test execution

## 11.1	Introduction

Clause 11 provides the guidelines on test executions.

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

## 11.2	EN-DC

None

## 11.3	NR/5GC

This clause provides the guidelines for the NR/5GC test cases.

A test case using more than one radio frequency, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.13, 6.1.2.14, 6.1.2.20,

8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.5, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9,

9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.8.

A test case using more than three radio frequencies, i.e. using the radio frequency NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n12,

Band n34,

Band n41,

Band n70.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.5, 6.1.1.7, 6.1.1.8,

9.1.5.1.2.