3GPP TSG-RAN5 Meeting #56 	R5-123508r1

# Qingdao, China, August 13-17, 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | CRNum | rev | - | Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 6.6.2.1A SEM for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA-UEConTest |  |  |  |  |  |  | Date: |  | 2012-08-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Table 6.6.2.1A.3-1 in 36.521-1 v10.2.0 is not correct, mismatching Table 6.6.2.1A-1 in the latest 36.101 v10.7.0. Initial Conditions and Test Requirements for the test case have not been added yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 6.6.2.1A.3-1 in Minimum conformance requirements was corrected; - Editor’s notes were updated with missing and completed information. - Initial Conditions were introduced. - Test Requirements were introduced. - Test Procedure, Message Contents and Test Tolerance were left FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA work plan will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | 1st revision R5-123508 R5-123508r1 -Editorial corrections of table fonts and symbol subscripts have been made; -Table numbers have been updated related to R5-123212r1; -One of the test points in Test Configuration Table has been corrected; -Test requirements are modified. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.6.2.1	Spectrum Emission Mask

6.6.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.6.2.1.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 8 and forward.

6.6.2.1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1.3-1 the spurious requirements in clause 6.6.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.3-1 for the specified channel bandwidth.

Table 6.6.2.1.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

6.6.2.1.4	Test description

6.6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for SEM testing | Mod’n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 5 | 5 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 4) | 50 (Note 4) |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 4) | 75 (Note 4) |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 4) | 100 (Note 4) |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 6.2.3.3. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories 2-5 |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1.4.3.

6.6.2.1.4.2	Test procedure

- SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control “up” commands in the uplink scheduling information to the UE until the UE transmits at PUMAX  level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1.5-1 or 6.6.2.1.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

6.6.2.1.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-1 as appropriate,

and

the power of any UE emission shall fullfil requirements in Table.6.6.2.1.5-1 or Table.6.6.2.1.5-2, as applicable.

Table 6.6.2.1.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.1A	Spectrum emission mask for CA

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

- The Test Procedure for this test is undefined

- Message Contents for this test are undefined

- The Test Tolerances applicable to this test are undefined

6.6.2.1A.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support CA.

6.6.2.1A.3	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to one E-UTRA band, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.3-1.

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth (Table 5.4.2A-1) For intra-band contiguous carrier aggregation the bandwidth class C, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.1A.1.3-1 for the specified channel bandwidth.

Table 6.6.2.1A.1.3-1: General E-UTRA CA spectrum emission mask for Bandwidth Class C

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22.5 | -22.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | 1 MHz |
| 5-29.9 | -13 | -13 | -13 | 1 MHz |
| 29.9-30 | -25 | -13 | -13 | 1 MHz |
| 30-34.9 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  | -25 | -13 | 1 MHz |
| 35-39.8 |  |  | -13 | 1 MHz |
| 39.8-44.8 |  |  | -25 | 1 MHz |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.1A.

6.6.2.1A.4	Test description

6.6.2.1A.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause [4.3.1] for different CA bandwidth classes. |  |  |  | C: Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 75 | 75 | N/A for this test | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 130 | P_75@0 | S_55@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | 16QAM | 130 | P_75@0 | S_55@0 | - | - |
| 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 15 | P_5@0 | P_5@35 | S_5@0 | - |
| 75 | 75 |  | QPSK | 75 | P_25@0 | P_20@35 | S_15@0 | S_15@34 |
| 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 15 | P_5@0 | P_5@50 | S_5@0 | - |
| 100 | 50 |  | QPSK | 75 | P_25@0 | P_20@34 | S_15@0 | S_15@34 |
| 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 130 | P_100@0 | S_30@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | 16QAM | 130 | P_100@0 | S_30@0 | - | - |
| 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 20 | P_5@0 | P_5@50 | S_5@0 | S_5@50 |
| 100 | 100 |  | QPSK | 100 | P_25@0 | P_25@50 | S_25@0 | S_25@50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure [A.3].

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in [State 3A-RF-CA] according to TS 36.508 [7] clause [5.2A.2]. Message contents are defined in clause 6.6.2.1A.1.4.3.

6.6.2.1A.4.2	Test procedure

FFS.

6.6.2.1A.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause [FFS].

6.6.2.1A.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in subclause 5.4.2A, derived in step [FFS], shall fulfil requirements in Table 6.2.3A.5-1 as appropriate,

and

the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2, as applicable.

Table 6.6.2.1A.1.5-1: General E-UTRA spectrum emission mask for CA, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22.5+TT | -22.5+TT | -24+TT | 30 kHz |
| 1-5 | -10+TT | -10+TT | -10+TT | 1 MHz |
| 5-29.9 | -13+TT | -13+TT | -13+TT | 1 MHz |
| 29.9-30 | -25+TT |  |  | 1 MHz |
| 30-34.9 |  | -25+TT |  | 1 MHz |
| 34.9-35 |  |  |  | 1 MHz |
| 35-39.8 |  |  |  | 1 MHz |
| 39.8-44.8 |  |  | -25+TT | 1 MHz |

Table 6.6.2.1A.1.5-2: General E-UTRA spectrum emission mask for CA, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22.5+TT | -22.5+TT | -24+TT | 30 kHz |
| 1-5 | -10+TT | -10+TT | -10+TT | 1 MHz |
| 5-29.9 | -13+TT | -13+TT | -13+TT | 1 MHz |
| 29.9-30 | -25+TT |  |  | 1 MHz |
| 30-34.9 |  | -25+TT |  | 1 MHz |
| 34.9-35 |  |  |  | 1 MHz |
| 35-39.8 |  |  |  | 1 MHz |
| 39.8-44.8 |  |  | -25+TT | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<< Unchanged sections omitted >>