3GPP TSG-RAN WG5 Meeting #69 	R5-155251r1

# Anaheim, USA, 16th – 20th Nov 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2179 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test case 6.3.5A.1.3 Power Control Absolute Power Tolerance for CA(intra-band non-contiguous DL CA and UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  |  | Date: |  | 2015-10-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | -Currently, initial conditons,test description,test procedure and test requirement of test case 6.3.5A.1.3 are still missing. - Annex F for 2UL CA test cases is still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Initial conditons,test description,test procedure and test requirement are added. - Annex F for 2UL CA test cases is added Update in r1: Corrected definition of 6.3.5A.1.3.3 to “The minimum conformance requirement are defined in clause 6.3.5A.1.0”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI would not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR …CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

6.3.5A.1.3	Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA)

Editor's notes: This test is incomplete.

-The message contents are not yet defined

6.3.5A.1.3.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms.

6.3.5A.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.3.5A.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3.5A.1.0

6.3.5A.1.3.4	Test description

6.3.5A.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.1.3.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.1.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Mid frequency, Maximum Wgap(low + high frequency) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 25 | 25 | 5 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 3 | 100 | 100 | 5 | N/A | N/A | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3.5A.1.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5A.1.3.4.3. Any PDCCH DCI format 0 sent to the UE during the configuration should have TPC command 0dB.

6.3.5A.1.3.4.2	Test procedure

Same test procdure as in clause 6.3.5A.1.1.4.2.

-	Instead of Table 6.3.5A.1.1.4.1-1 use Table 6.3.5A.1.3.4.1-1.

-	Instead of clause 6.3.5A.1.1.4.3 use clause 6.3.5A.1.3.4.3.

6.3.5A.1.3.4.3	Message contents

TBD

6.3.5A.1.3.5	Test requirement

The absolute power control tolerance per component carrier measured in step (5) of the test procedure is not to exceed the values specified in Table 6.3.5A.1.3.5-1 and 6.3.5A.1.3.5-2. The test requirement tables are originated from tables 6.3.5.1.5-1 and 6.3.5.1.5-2.

Table 6.3.5A.1.3.5-1: Absolute power tolerance for inter-band CA: test point 1

|  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Expected Measured power Normal conditions | -14.8 dBm | -10.8 dBm | -8.6 dBm | -5.6 dBm | -3.9 dBm | -2.6 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB |
| Expected Measured power Extreme conditions | -14.8 dBm | -10.8 dBm | -8.6 dBm | -5.6 dBm | -3.9 dBm | -2.6 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 |  |  |  |  |  |  |

Table 6.3.5A.1.3.5-2: Absolute power tolerance for inter-band CA: test point 2

|  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Expected Measured power Normal conditions | -2.8 dBm | 1.2 dBm | 3.4 dBm | 6.4 dBm | 8.2 dBm | 9.4 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB | ± 10.0dB ± 10.4dB |
| Expected Measured power Extreme conditions | -2.8 dBm | 1.2 dBm | 3.4 dBm | 6.4 dBm | 8.2 dBm | 9.4 dBm |
| Power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB | ± 13.0dB ± 13.4dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 |  |  |  |  |  |  |