3GPP TSG-RAN WG5 Meeting #69 	R5-155252r2

# Anaheim, USA, 16th – 20th Nov 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2180 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to LTE_CA_Rel12_2UL test case 6.3.5A.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12_2UL-UEConTest |  |  |  |  |  |  | Date: |  | 2015-10-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Currently, initial conditons,test description,test procedure and test requirement of test case 6.3.5A.2.2 are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial conditons,test description,test procedure and test requirement are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE_CA_Rel12_2UL work plan will not be fullfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Update in r1: Inserted subclause 6.3.5A.2.0 minimum conformance requirement.Corrected definition of 6.3.5A.2.1.3 and 6.3.5A.2.2.3 to “The minimum conformance requirement are defined in clause 6.3.5A.2.0”. Update in r2: Missing clauses affected added. Correction of some format issues and details in clause 6.3.5A.2.0 and clause 6.3.5A.2.2 |  |  |  |  |  |  |  |  |

<< Skip unchanged sections >>

6.3.5A.2	Power Control Relative power tolerance for CA

6.3.5A.2.0	Minimum conformance requirement

The requirements apply when the power of the target and reference sub-frames on each component carrier exceed -20 dBm and the total power is limited by PUMAX as defined in TS 36.101 [2] subclause 6.2.5A. For the purpose of these requirements, the power in each component carrier is specified over only the transmitted resource blocks .

For intra-band contiguous carrier aggregation bandwidth class C, the UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.5A.2.0-1

b)	For SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.5A.2.0-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subframes

c)	For RACH on the primary component carrier, the requirements given in Table 6.3.5A.2.0-1 for PRACH

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

For a), b) and c) above, two exceptions are allowed for each component carrier for a power per carrier ranging from -20 dBm to PUMAX,c as defined in TS 36.101 [2] subclause 6.2.5. For these exceptions the power tolerance limit is ±6.0 dB in Table 6.3.5A.2.0-1.

For inter-band carrier aggregation, the UE transmitter shall have the capability of changing the output power in both assigned component carrier in the uplink with a step sizes of P between subframes on the two respective component carrier as follows:

a)	The requirements for all combinations of PUSCH and PUCCH transitions per component carrier is given in Table 6.3.5A.2.0-1, when the average transmit power per PRB for the transmission on the assigned carriers are aligned to within ±[2] dB in the reference sub-frame and the target subframe after the transition;

b)	For SRS the requirements for combinations of PUSCH/PUCCH and SRS transitions between sub-frames given in Table 6.3.5A.2.0-1 apply per component carrier when the target and reference subframes are configured for either simultaneous SRS or simultaneous PUSCH and with the average transmit power per PRB for the transmissions on the assigned carrier aligned to within ±[2] dB in the reference sub-frame and the target subframe after the transition;

c)	 For RACH the requirements apply for the primary cell and are given in Table 6.3.5A.2.0-1.

Table 6.3.5A.2.0-1: Relative Power Tolerance for Transmission (normal conditions)

| Power step P (Up or down) [dB] | All combinations of PUSCH and PUCCH transitions [dB] | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames [dB] | PRACH [dB] |
| --- | --- | --- | --- |
| ΔP < 2 | ±2.5 (Note 3) | ±3.0 | ±2.5 |
| 2 ≤ ΔP < 3 | ±3.0 | ±4.0 | ±3.0 |
| 3 ≤ ΔP < 4 | ±3.5 | ±5.0 | ±3.5 |
| 4 ≤ ΔP ≤ 10 | ±4.0 | ±6.0 | ±4.0 |
| 10 ≤ ΔP < 15 | ±5.0 | ±8.0 | ±5.0 |
| 15 ≤ ΔP | ±6.0 | ±9.0 | ±6.0 |
| Note 1: For extreme conditions an additional ± 2.0 dB relaxation is allowed Note 2: For operating bands under Note 2 in Table 6.2.2.3-1, the relative power tolerance is relaxed by increasing the upper limit by 1.5 dB if the transmission bandwidth of the reference sub-frames is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high and the target sub-frame is not confined within any one of these frequency ranges; if the transmission bandwidth of the target sub-frame is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high and the reference sub-frame is not confined within any one of these frequency ranges, then the tolerance is relaxed by reducing the lower limit by 1.5 dB. Note 3: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods for TDD: for a power step ΔP ≤ 1 dB, the relative power tolerance for transmission is ±1.0 dB. |  |  |  |

The power step (ΔP) is defined as the difference in the calculated setting of the UE Transmit power between the target and reference sub-frames with the power setting according to Clause 5.1 of TS 36.213 [10]. The error is the difference between ΔP and the power change measured at the UE antenna port with the power of the cell-specific reference signals kept constant. The error shall be less than the relative power tolerance specified in Table 6.3.5A.2.0-1.

For sub-frames not containing an SRS symbol, the power change is defined as the relative power difference between the mean power of the original reference sub-frame and the mean power of the target subframe not including transient durations. The mean power of successive sub-frames shall be calculated according to TS 36.101 [2] Figure 6.3.4.3-1 and TS 36.101 [2] Figure 6.3.4.1-1 if there is a transmission gap between the reference and target sub-frames.

The normative reference for this requirement is TS 36.101 [2] clause 6.3.5A.2.

6.3.5A.2.1	Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA)

Editor's notes: This test is incomplete. The following aspects are either missing or not yet determined:

- The uncertainties, Test Tolerances and setting margins shown as [FFS]dB in the Test Procedure are undefined.

- The way to test the scenario when both carriers change power simultaneously

- The Test coverage defined by the Test Configuration Table is to be reviewed.

6.3.5A.2.1.1	Test purpose

To verify the ability of the UE transmitter to change the output power in both assigned component carrier in the uplink with a defined power step sizes between sub-frames on the two respective component carrier.

6.3.5A.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.3.5A.2.1.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 6.3.5A.2.0.

6.3.5A.2.1.4	       Test description

6.3.5A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause [4.3.1] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | C: Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | [FFS] |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 75 | 75 | N/A for this test | QPSK | [FFS] | [FFS] | [FFS] | - | - |
| 100 | 50 |  | QPSK | [FFS] | [FFS] | [FFS] | - | - |
| 100 | 100 |  | QPSK | [FFS] | [FFS] | [FFS] | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to table 6.3.5A.2.1.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5A.2.1.4.3.

6.3.5A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clause 8.3.3.2).

4.	The procedure is separated in various subtests to verify different aspects of relative power control. The power changes of the subtests are shown by diagrams in the Test Procedure.

5.	Sub test: SCC power increase

5.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3.5A.2.1.4.2-1. The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. In the target subframe the SCC allocation is increased, and the corresponding power increase is measured.

Table 6.3.5A.2.1.4.2-1: Power settings and RB allocations for SCC power increase, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCRefSet, 0, dBm/NRB alloc | -17 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

5.2.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3.5A.2.1.5-1. If the result meets the normal Test requirement, continue to step 5.3. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the PCC exception counter by 1 and continue to step 5.3, Otherwise, fail the UE for this subtest.

5.3.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3.5A.2.1.5-1. If the result meets the normal Test requirement, continue to step 5.4. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the SCC exception counter by 1 and continue to step 5.4, Otherwise, fail the UE for this subtest.

5.4.	Calculate the Total uplink power across both CCs in dBm as 10log10((PCCTargetMeas, n in mW) + (SCCTargetMeas, n in mW)). If (PUMAX - Total uplink power) > [FFS]dB, continue to step 5.5. Otherwise, go to step 5.6.

5.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3.5A.2.1.4.2-2, Repeat steps 5.2 to 5.4.

Table 6.3.5A.2.1.4.2-2: Power settings and RB allocations for SCC power increase, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n +(SCCTargetMeas, n –SCCRefMeas, n) +[FFS]dB) |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

5.6.	If the PCC exception counter is ≤ 2 and the SCC exception counter is ≤ 2, and the requirements specified in Table 6.3.5A.2.1.5-1 are all met, pass the UE for this subtest.

Figure 6.3.5A.2.1.4.2-1: SCC power increase

6.	Sub test: SCC power decrease

6.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3.5A.2.1.4.2-3.The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. In the target subframe the SCC allocation is decreased, and the corresponding power decrease is measured.

Table 6.3.5A.2.1.4.2-3: Power settings and RB allocations for SCC power decrease, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCRefSet, 0, dBm/NRB alloc | PUMAX -5 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

6.2.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3.5A.2.1.5-2. If the result meets the normal Test requirement, continue to step 6.3. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the PCC exception counter by 1 and continue to step 6.3, Otherwise, fail the UE for this subtest.

6.3.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3.5A.2.1.5-2. If the result meets the normal Test requirement, continue to step 6.4. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the SCC exception counter by 1 and continue to step 6.4, Otherwise, fail the UE for this subtest.

6.4.	If the uplink (power for each CC – (-20dBm)) is >  [FFS]dB, continue to step 6.5. Otherwise, go to step 6.6.

6.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3.5A.2.1.4.2-4, Repeat steps 6.2 to 6.4.

Table 6.3.5A.2.1.4.2-4: Power settings and RB allocations for SCC power decrease, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n +(SCCTargetMeas, n –SCCRefMeas, n) -[FFS]dB) |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

6.6.	If the PCC exception counter is ≤ 2 and the SCC exception counter is ≤ 2, and the requirements specified in Table 6.3.5A.2.1.5-2 are all met, pass the UE for this subtest.

Figure 6.3.5A.2.1.4.2-2: SCC power decrease

7.	Sub test: PCC and SCC power increase together

7.1.	[Placeholder for Test Procedure steps]

6.3.5A.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

6.3.5A.2.1.5	Test requirement

For intra-band contiguous carrier aggregation bandwidth class C, the relative power control tolerance per component carrier measured in steps 5, 6 , 7 and 8  of the test procedure should satisfy the applicable test requirements specified in Tables 6.3.5A.2.1.5-1 to 6.3.5A.2.1.5-[FFS] for normal conditions.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each subtest, and are counted during the Test Procedure.

Table 6.3.5A.2.1.5-1: Test requirements for SCC power increase

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -1dB -TT | +1dB +TT |
|  | Exception | dB | -6dB –TT | +6dB +TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | +5dB -TT | +13dB +TT |
|  | Exception | dB | +3dB –TT | +15dB +TT |

Table 6.3.5A.2.1.5-2: Test requirements for SCC power decrease

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -1dB -TT | +1dB +TT |
|  | Exception | dB | -6dB –TT | +6dB +TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | -13dB -TT | -5dB +TT |
|  | Exception | dB | -15dB –TT | -3dB +TT |

Table 6.3.5A.2.1.5-3: Test requirements for PCC and SCC power increase together

[FFS]

6.3.5A.2.2	Power Control Relative power tolerance for CA (inter-band DL CA and UL CA)

Editor's notes: This test is incomplete. The following aspects are either missing or not yet determined:

- The uncertainties, Test Tolerances and setting margins shown as [FFS]dB in the Test Procedure are undefined.

- The way to test the scenario when both carriers change power simultaneously

- The Test coverage defined by the Test Configuration Table is to be reviewed.

6.3.5A.2.2.1	Test purpose

To verify the ability of the UE transmitter to change the output power in both assigned component carrier in the uplink with a defined power step sizes between sub-frames on the two respective component carrier.

6.3.5A.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

6.3.5A.2.2.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 6.3.5A.2.0

6.3.5A.2.2.4	       Test description

6.3.5A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.5A.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | A: Mid range PCC-SCC: CC1-CC2 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | FFS |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_5@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 50 | N/A for this test | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 50 | 25 |  | QPSK | 20 | P_12@0 | S_8@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , only the first of those is tested, according to the order on the Test Configuration Table list. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to table 6.3.5A.2.2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5A.2.2.4.3

6.3.5A.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133[4], clause 8.3.3.2).

4.	The procedure is separated in various subtests to verify different aspects of relative power control. The power changes of the subtests are shown by diagrams in the Test Procedure.

5.	Sub test: SCC power increase

5.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3.5A.2.2.4.2-1. The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. In the target subframe the SCC allocation is increased, and the corresponding power increase is measured.

Table 6.3.5A.2.2.4.2-1: Power settings and RB allocations for SCC power increase, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCRefSet, 0, dBm/NRB alloc | -17 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

5.2.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3.5A.2.2.5-1. If the result meets the normal Test requirement, continue to step 5.3. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the PCC exception counter by 1 and continue to step 5.3, Otherwise, fail the UE for this subtest.

5.3.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3.5A.2.2.5-1. If the result meets the normal Test requirement, continue to step 5.4. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the SCC exception counter by 1 and continue to step 5.4, Otherwise, fail the UE for this subtest.

5.4.	Calculate the Total uplink power across both CCs in dBm as 10log10((PCCTargetMeas, n in mW) + (SCCTargetMeas, n in mW)). If (PUMAX - Total uplink power) > [FFS]dB, continue to step 5.5. Otherwise, go to step 5.6.

5.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3.5A.2.2.4.2-2, Repeat steps 5.2 to 5.4.

Table 6.3.5A.2.2.4.2-2: Power settings and RB allocations for SCC power increase, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n +(SCCTargetMeas, n –SCCRefMeas, n) +[FFS]dB) |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

5.6.	If the PCC exception counter is ≤ 2 and the SCC exception counter is ≤ 2, and the requirements specified in Table 6.3.5A.2.2.5-1 are all met, pass the UE for this subtest.

Figure 6.3.5A.2.2.4.2-1: SCC power increase

6.	Sub test: SCC power decrease

6.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table A.2.2.2.1-1 (FDD) or Table A.2.3.2.1-1 (TDD) on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3.5A.2.2.4.2-3.The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. In the target subframe the SCC allocation is decreased, and the corresponding power decrease is measured.

Table 6.3.5A.2.2.4.2-3: Power settings and RB allocations for SCC power decrease, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCRefSet, 0, dBm/NRB alloc | PUMAX -5 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

6.2.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3.5A.2.2.5-2. If the result meets the normal Test requirement, continue to step 6.3. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the PCC exception counter by 1 and continue to step 6.3, Otherwise, fail the UE for this subtest.

6.3.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3.5A.2.2.5-2. If the result meets the normal Test requirement, continue to step 6.4. If the result does not meet the normal Test requirement but meets the exception Test requirement, increment the SCC exception counter by 1 and continue to step 6.4, Otherwise, fail the UE for this subtest.

6.4.	If the uplink (power for each CC – (-20dBm)) is >  [FFS]dB, continue to step 6.5. Otherwise, go to step 6.6.

6.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3.5A.2.2.4.2-4, Repeat steps 6.2 to 6.4.

Table 6.3.5A.2.2.4.2-4: Power settings and RB allocations for SCC power decrease, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n +(SCCTargetMeas, n –SCCRefMeas, n) -[FFS]dB) |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

6.6.	If the PCC exception counter is ≤ 2 and the SCC exception counter is ≤ 2, and the requirements specified in Table 6.3.5A.2.2.5-2 are all met, pass the UE for this subtest.

Figure 6.3.5A.2.2.4.2-2: SCC power decrease

7.	Sub test: PCC and SCC power increase together

7.1.	[Placeholder for Test Procedure steps]

6.3.5A.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

6.3.5A.2.2.5	Test requirement

For intra-band contiguous carrier aggregation bandwidth class C, the relative power control tolerance per component carrier measured in steps 5, 6 , 7 and 8  of the test procedure should satisfy the applicable test requirements specified in Tables 6.3.5A.2.2.5-1 to 6.3.5A.2.2.5-[FFS] for normal conditions.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each subtest, and are counted during the Test Procedure.

Table 6.3.5A.2.2.5-1: Test requirements for SCC power increase

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -1dB -TT | +1dB +TT |
|  | Exception | dB | -6dB –TT | +6dB +TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | +5dB -TT | +13dB +TT |
|  | Exception | dB | +3dB –TT | +15dB +TT |

Table 6.3.5A.2.2.5-2: Test requirements for SCC power decrease

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -1dB -TT | +1dB +TT |
|  | Exception | dB | -6dB –TT | +6dB +TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | -13dB -TT | -5dB +TT |
|  | Exception | dB | -15dB –TT | -3dB +TT |

Table 6.3.5A.2.2.5-3: Test requirements for PCC and SCC power increase together

[FFS]