3GPP TSG-RAN WG5 Meeting #76	   R5-173846r2

Berlin, Germany, 21st – 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1157 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Generic Test Procedure for NG eCall setup and MSD update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2017-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Generic Test Procedure for NG eCall setup and MSD update |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Clause defined for the addition of Generic Test Procedure for eCall over IMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCall over IMS workplan test cases will have to include this procedure in the test Prose, better to define a generic Procedure to avoid redundancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.XX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN Impact Please Note: RAN5 Secretary to assign Cxx. Also, A.2.x and A.2.Y to be updated based on R5-173992. Revision1: - Added ACK in the generic procedure. - Renumbered test steps. Revision2: - Corrected ACK in the generic procedure. |  |  |  |  |  |  |  |  |

<Start of new section>

# C.XX	Generic Test Procedure for NG eCall setup and MSD Update

Test procedure:

- SS waits for UE to send an INVITE request with initial MSD.

2)	SS responds to the INVITE request with a 200 OK response.

3)   UE responds to the 200 OK with ACK.

4)	SS sends a SIP INFO to request UE for a MSD update.

5)	UE responds to the MSD update request with 200 OK response.

6)	SS waits for UE to send a SIP INFO with updated MSD.

7)	The SS responds to the SIP INFO with a 200 OK response.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with initial MSD. |
| 2 |  |  | 200 OK | SS responds INVITE with 200 OK. |
| 3 |  |  | ACK | UE responds ACK for 200 OK. |
| 4 |  |  | SIP INFO | SS sends SIP INFO to request MSD update. |
| 5 |  |  | 200 OK | UE responds SIP INFO with 200 OK. |
| 6 |  |  | SIP INFO | UE sends SIP INFO with MSD update. |
| 7 |  |  | 200 OK | SS responds SIP INFO with 200 OK. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following inclusion along with the applicable message body of default message:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec (AMR/8000 and/or AMR-WB/16000) shall be present in the media attributes, optionally including channel number "/1". |

200 OK for INVITE (Step 2)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following inclusion along with the applicable message body of default message:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS:37 - b=RS:0 - b=RR:0 Attributes for media: - a=rtpmap: (payload type) AMR/8000/1 or AMR-WB/16000/1 [Note 1] [Note2] - a=fmtp: (format) mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. Note 2: If UE included AMR-WB/16000 in step 1, SS uses AMR-WB/16000/1. Otherwise SS uses AMR/8000/1. |

INFO (Step 4)

Use the default message “INFO for MT” in annex A.2.Y.

INFO (Step 6)

Use the default message “INFO for MO” in annex A.2.X.

<End of new section>