3GPP TSG RAN WG6 #2  	R6-160192

# Reno, Nevada, USA, November 14-18, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.003 | CR | 0142 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Alternative Mappings for Higher Coverage Classes with 2 PDCHs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2016-11-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of alternative blind physical layer mappings for dedicated channels (EC-PDTCH and EC-PACCH) on DL/UL with 2 PDCH resources has been agreed as part of the Rel-14 work item on Radio Interface Enhancements for EC-GSM-IoT in RP-161806. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alternative mappings for blind physical layer transmissions are included for EC- PDTCH and EC-PACCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Alternative coverage class mappings with 2 PDCHs will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1b.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064, TS 45.001, TS 45.002, TS 45.005, TS 45.008, TS 44.018, TS 44.060 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010, TS 51.021 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

For Information only

## 5.1b	Extended Coverage Packet data traffic channel (EC-PDTCH)

### 5.1b.1	General

Twelve coding schemes are specified for the Extended Coverage Packet data traffic channels. The coding schemes MCS-1 to MCS-9 are identical to those defined for PDTCH in subclause 5.1. The coding schemes MCS-1/16, MCS-1/8 and MCS-1/4 are based on MCS-1 defined in subclause 5.1.

The following additions apply:

-	Blind physical layer transmissions are employed.

-	For downlink, multiple sequences of USF bits are delivered to the encoder. Each precoded USF sequence is mapped onto one instance of the repeated block.

The following restrictions apply:

-	Reduced Transmission Time Interval (RTTI) shall not be used on EC-PDTCH.

-	A PAN field shall not be included in the message delivered to the encoder.

-	For data coding, a reduced set of puncturing schemes is used on an EC-PDTCH, compared to a PDTCH. Only the puncturing schemes listed in table 0 shall be used.

Table 0: Allowed puncturing schemes on EC-PDTCH

| MCS | Allowed puncturing schemes in subclause 5.1 |
| --- | --- |
| MCS-1/16 MCS-1/8 MCS-1/4 MCS-1 MCS-2 | P1 |
| MCS-3 MCS-4 | P1 or P2 |
| Other MCSs | No restriction |

### 5.1b.2	Downlink MCS-1/M

#### 5.1b.2.1	Block constitution

The message delivered to the encoder has a fixed size of 206 information bits {a(0),a(1),...,a(205)}. In addition, M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8 and 4 for MCS-1/16, MCS-1/8 and MCS-1/4, respectively.

#### 5.1b.2.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions, the block is encoded as specified for MCS-1 in subclause 5.1.5.1. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission k is defined as

d(k)=u(m,k)		for 		k=0,1,2;

d(k)=a(k-3)		for 		k=3,…,208.

Denote the resulting bursts from encoding of blind physical layer transmissions m as R(m,B,j) = e(B,j) for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

1st modification

#### 5.1b.2.3	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assigned

B’ = B + 4(m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

For Information only

### 5.1b.3	Uplink MCS-1/M

#### 5.1b.3.1	Block constitution

The message delivered to the encoder has a fixed size of 209 information bits {a(0),a(1),...,a(208)}.

#### 5.1b.3.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions the block is encoded as specified for MCS-1 in subclause 5.1.5.2. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission m is defined as

d(k)=a(k)	for k=0,…,208.

Denote the resulting bursts from encoding of blind physical layer transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.3.3	Mapping onto PDCHs

The mapping is done as specified for MCS-1/M DL in subclause 5.1b.2.3.

End of modifications