3GPP TSG RAN WG6 #4 	R6-170215

# Hangzhou, P.R. China, May 15-19, 2017                                         Revision of R6-170122

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | Draft | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capability support for uplink coverage class CC5 for UL MCL improvement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-05-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The introduction of the new uplink coverage class CC5 to achieve the required UL MCL improvement, follows the agreement of the WID in RP-161806. The support of this feature needs to be indicated by the MS to the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A capability support bit in the MS Radio Access Capability IE is introduced for a Rel-14 MS to indicate the support for uplink coverage class CC5 for EC-RACH, EC-PDTCH and EC-PACCH channels. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support for uplink coverage class CC5 cannot be indicated by the MS to the network and the network may assign a suboptimum coverage class on uplink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.5.12a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 44.018 CR 1061 TS 44.060 CR 1639 TS 45.001 CR 0097 TS 45.002 CR 0206 TS 45.003 CR 0145 TS 45.005 CR 0600 TS 45.008 CR 0649 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR XXXX TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The present CR is based on the RAN6#3 submission in R6-170122. |  |  |  |  |  |  |  |  |

First Modification

#### 10.5.5.12a	MS Radio Access capability

The purpose of the MS Radio Access capability information element is to provide the radio part of the network with information concerning radio aspects of the mobile station. The contents might affect the manner in which the network handles the operation of the mobile station.

The MS Radio Access capability is a type 4 information element, with a maximum length of 52 octets.

The MS Radio Access capability information element is coded as shown in figure 10.5.128a/3GPP TS 24.008 and table 10.5.146/3GPP TS 24.008.

For the indication of the radio access capabilities the following conditions shall apply:

-	Among the three Access Type Technologies GSM 900-P, GSM 900-E and GSM 900-R only one shall be present.

-	Due to shared radio frequency channel numbers between GSM 1800 and GSM 1900, the mobile station should provide the relevant radio access capability for either GSM 1800 band OR GSM 1900 band, not both.

-	The MS shall indicate its supported Access Technology Types during a single MM procedure.

-	If the alternative coding by using the Additional access technologies struct is chosen by the mobile station, the mobile station shall indicate its radio access capability for the serving BCCH frequency band in the first included Access capabilities struct, if this information element is not sent in response to an Access Technologies Request from the network or if none of the requested Access Technology Types is supported by the MS. Otherwise, the mobile station shall include the radio access capabilities for the frequency bands it supports in the order of priority requested by the network (see 3GPP TS 44.060 [76]).

-	The first Access Technology Type shall not be set to "1111".

For error handling the following shall apply:

	If a received Access Technology Type is unknown to the receiver, it shall ignore all the corresponding fields.

	If within a known Access Technology Type a receiver recognizes an unknown field it shall ignore it.

	For more details about error handling of MS radio access capability see 3GPP TS 48.018 [86].

NOTE: 	The requirements for the support of the A5 algorithms in the MS are specified in 3GPP TS 43.020 [13].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MS Radio Access Capability IEI |  |  |  |  |  |  |  | octet 1 |
| Length of MS Radio Access Capability contents |  |  |  |  |  |  |  | octet 2 |
| MS RA capability value part |  |  |  |  |  |  |  | octet 3-52 |

Figure 10.5.128a/3GPP TS 24.008 MS Radio Access Capability information element

Table 10.5.146/3GPP TS 24.008: MS Radio Access Capability Information Element

< MS RA capability value part > ::=

< MS RA capability value part struct >

<spare bits>**; -- may be used for future enhancements

<MS RA capability value part struct >::=  --recursive structure allows any number of Access technologies

{	{	< Access Technology Type: bit (4) > exclude 1111

< Access capabilities : <Access capabilities struct> > }

| 	{	< Access Technology Type: bit (4) == 1111 >	-- structure adding Access technologies with same capabilities

< Length : bit (7) > 		-- length in bits of list of Additional access technologies and spare bits

< bit (val (Length))

& {

{ 1 < Additional access technologies: < Additional access technologies struct > > } ** 0

<spare bits>**

} >

}

}

{ 0 | 1 <MS RA capability  value part struct> } ;

< Additional access technologies struct > ::=

< Access Technology Type : bit (4) >

< GMSK Power Class : bit (3) >

< 8PSK Power Class : bit (2) > ;

< Access capabilities struct > ::=

< Length : bit (7) > -- length in bits of Content and spare bits

< bit (val (Length))

& {

<Access capabilities : <Content>>

<spare bits>**

} > ; -- expands to the indicated length

< Content > ::=

< RF Power Capability : bit (3) >
	{ 0 | 1 <A5 bits : <A5 bits> > } 	-- zero means that the same values apply for parameters as in the immediately preceding Access capabilities field within this IE
	< ES IND : bit >

< PS : bit >

< VGCS : bit >

< VBS : bit >

{ 0 | 1 < Multislot capability : Multislot capability struct > } -- zero means that the same values for multislot parameters as given in an earlier Access capabilities field within this IE apply also here

-- Additions in release 99

{ 0 | 1 < 8PSK Power Capability : bit(2) >}

< COMPACT Interference Measurement Capability : bit >

< Revision Level Indicator : bit >

< UMTS FDD Radio Access Technology Capability : bit > 				-- 3G RAT

< UMTS 3.84 Mcps TDD Radio Access Technology Capability : bit > 	-- 3G RAT

< CDMA 2000 Radio Access Technology Capability : bit > 				-- 3G RAT

-- Additions in release 4

< UMTS 1.28 Mcps TDD Radio Access Technology Capability: bit >	-- 3G RAT

< GERAN Feature Package 1 : bit >

{ 0 | 1 < Extended DTM GPRS Multi Slot Class : bit(2) >

< Extended DTM EGPRS Multi Slot Class : bit(2) > }

< Modulation based multislot class support : bit >

-- Additions in release 5

{ 0 | 1 < High Multislot Capability : bit(2) > }

0	-- The value '1' was allocated in an earlier version of the protocol and shall not be used.

< GMSK Multislot Power Profile : bit (2) >

< 8-PSK Multislot Power Profile : bit (2) >

-- Additions in release 6

< Multiple TBF Capability : bit >

< Downlink Advanced Receiver Performance : bit(2) >

< Extended RLC/MAC Control Message Segmentation Capability : bit >

< DTM Enhancements Capability : bit >
	{ 0 | 1	< DTM GPRS High Multi Slot Class : bit(3) >

{ 0 | 1 < DTM EGPRS High Multi Slot Class : bit(3) > } }

< PS Handover Capability : bit >

-- Additions in release 7

< DTM Handover Capability : bit >

{ 0 | 1 < Multislot Capability Reduction for Downlink Dual Carrier: bit (3) >

< Downlink Dual Carrier for DTM Capability : bit> }

< Flexible Timeslot Assignment : bit >

< GAN PS Handover Capability : bit >

< RLC Non-persistent Mode : bit >

< Reduced Latency Capability : bit >

< Uplink EGPRS2 : bit(2) >

< Downlink EGPRS2 : bit(2) >

-- Additions in release 8

< E-UTRA FDD support : bit >

< E-UTRA TDD support : bit >

< GERAN to E-UTRA support in GERAN packet transfer mode: bit(2) >

< Priority-based reselection support : bit >

-- Additions in release 9

< Enhanced Flexible Timeslot Assignment : Enhanced Flexible Timeslot Assignment struct>

< Indication of Upper Layer PDU Start Capability for RLC UM : bit >

< EMST Capability : bit >

< MTTI Capability : bit >

< UTRA CSG Cells Reporting : bit >

< E-UTRA CSG Cells Reporting : bit >

-- Additions in release 10

< DTR Capability : bit >

< EMSR Capability : bit >

< Fast Downlink Frequency Switching Capability : bit >

< TIGHTER Capability : bit(2) >

-- Additions in release 11

< FANR Capability : bit >

< IPA Capability : bit>

< GERAN Network Sharing support : bit>

< E-UTRA Wideband RSRQ measurements support : bit>

-- Additions in release 12
	< UTRA Multiple Frequency Band Indicators support : bit >

< E-UTRA Multiple Frequency Band Indicators support : bit >

{ 0			-- DLMC not supported

| 1 < DLMC Capability : DLMC Capability struct > }

< Extended TSC Set Capability support : bit >

-- Late addition of a release 11 feature

< Extended EARFCN value range: bit>

-- Additions in release 13

< (EC-)PCH monitoring support: bit(2)>

-- Additions in release 14

< EC uplink coverage enhancement support : bit >;

-- error: struct too short, assume features do not exist

-- error: struct too long, ignore data and jump to next Access technology

Table 10.5.146/3GPP TS 24.008 (continued): MS Radio Access Capability IE

< Multislot capability struct > ::= { 0 ¦ 1 < HSCSD multislot class : bit (5) > } { 0 ¦ 1 < GPRS multislot class : bit (5) > < GPRS Extended Dynamic Allocation Capability : bit > } { 0 ¦ 1 < SMS_VALUE : bit (4) > < SM_VALUE : bit (4) > } -- Additions in release 99 { 0 ¦ 1 < ECSD multislot class : bit (5) > } { 0 ¦ 1 < EGPRS multislot class : bit (5) > < EGPRS Extended Dynamic Allocation Capability : bit > } { 0 ¦ 1 < DTM GPRS Multi Slot Class: bit(2)> <Single Slot DTM : bit> {0 ¦ 1 <DTM EGPRS Multi Slot Class : bit(2)> } } ; -- error: struct too short, assume features do not exist <A5 bits> ::= < A5/1 : bit> <A5/2 : bit> <A5/3 : bit> <A5/4 : bit> <A5/5 : bit> <A5/6 : bit> <A5/7 : bit>; -- bits for circuit mode ciphering algorithms. These fields are not used by the network and may be excluded by the MS. < Enhanced Flexible Timeslot Assignment struct > ::= { 0 ¦ 1 < Alternative EFTA Multislot Class : bit(4) > < EFTA Multislot Capability Reduction for Downlink Dual Carrier: bit (3) > }; < DLMC Capability struct > ::= { 0 ¦ 1 < DLMC - Non-contiguous intra-band reception : bit(2) > < DLMC - Inter-band reception : bit(1) > } < DLMC - Maximum Bandwidth : bit(2) > < DLMC - Maximum Number of Downlink Timeslots : bit(6) > < DLMC - Maximum Number of Downlink Carriers : bit(3) > ; Access Technology Type This field indicates the access technology type to be associated with the following access capabilities. Bits 4 3 2 1 0 0 0 0 GSM P 0 0 0 1 GSM E --note that GSM E covers GSM P 0 0 1 0 GSM R --note that GSM R covers GSM E and GSM P 0 0 1 1 GSM 1800 0 1 0 0 GSM 1900 0 1 0 1 GSM 450 0 1 1 0 GSM 480 0 1 1 1 GSM 850 1 0 0 0 GSM 750 1 0 0 1 GSM T 380 1 0 1 0 GSM T 410 1 0 1 1 -- This value was allocated in an earlier version of the protocol and shall not be used. 1 1 0 0 GSM 710 1 1 0 1 GSM T 810 1 1 1 1 Indicates the presence of a list of Additional access technologies All other values are treated as unknown by the receiver. A MS which does not support any GSM access technology type shall set this field to '0000'. RF Power Capability, GMSK Power Class (3 bit field) This field contains the binary coding of the power class used for GMSK associated with the indicated Access Technology Type (see 3GPP TS 45.005 [33]). A MS which does not support any GSM access technology type shall set this field to '000'. 8PSK Power Capability (2 bit field) If 8-PSK modulation is supported for uplink, this field indicates the radio capability for 8-PSK modulation. The following coding is used (see 3GPP TS 45.005 [33]): Bits 2 1 0 0 Reserved 0 1 Power class E1 1 0 Power class E2 1 1 Power class E3 The presence of this field also indicates 8PSK modulation capability in the uplink. 8PSK Power Class (2 bit field) This field indicates the radio capability for 8-PSK modulation. The following coding is used (see 3GPP TS 45.005 [33]): Bits 2 1 0 0 8PSK modulation not supported for uplink 0 1 Power class E1 1 0 Power class E2 1 1 Power class E3 Additional access technologies struct This structure contains the GMSK Power Class and 8PSK Power Class for an additional Access Technology. All other capabilities for this indicated Access Technology are the same as the capabilities indicated by the preceding Access capabilities struct. A5/1 0 encryption algorithm A5/1 not available 1 encryption algorithm A5/1 available A5/2 The MS shall set this bit to ‘0’. The network shall accept any received value. 0 encryption algorithm A5/2 not available 1 Not used. This value was allocated in earlier versions of the protocol. A5/3 0 encryption algorithm A5/3 not available 1 encryption algorithm A5/3 available A5/4 0 encryption algorithm A5/4 not available 1 encryption algorithm A5/4 available A5/5 0 encryption algorithm A5/5 not available 1 encryption algorithm A5/5 available A5/6 0 encryption algorithm A5/6 not available 1 encryption algorithm A5/6 available A5/7 0 encryption algorithm A5/7 not available 1 encryption algorithm A5/7 available ES IND – (Controlled early Classmark Sending) 0 "controlled early Classmark Sending" option is not implemented 1 "controlled early Classmark Sending" option is implemented

Table 10.5.146/3GPP TS 24.008 (concluded): MS Radio Access Capability IE

PS – (Pseudo Synchronisation) 0 PS capability not present 1 PS capability present VGCS – (Voice Group Call Service) 0 no VGCS capability or no notifications wanted 1 VGCS capability and notifications wanted. VBS – (Voice Broadcast Service) 0 no VBS capability or no notifications wanted 1 VBS capability and notifications wanted HSCSD Multi Slot Class The Multi Slot Class field is coded as the binary representation of the multislot class defined in 3GPP TS 45.002 [32]. This field is not used by the network and may be excluded by the MS. Range 1 to 18, all other values are reserved. GPRS Multi Slot Class The GPRS Multi Slot Class field is coded as the binary representation of the multislot class defined in 3GPP TS 45.002 [32]. ECSD Multi Slot Class The presence of this field indicates ECSD capability. Whether the MS is capable of 8-PSK modulation in uplink is indicated by the presence of 8-PSK Power Capability field. The Multi Slot Class field is coded as the binary representation of the multislot class defined in 3GPP TS 45.002 [32]. This field is not used by the network and may be excluded by the MS. Range 1 to 18, all other values are reserved. EGPRS Multi Slot Class The presence of this field indicates EGPRS capability. Whether the MS is capable of 8-PSK modulation in uplink is indicated by the presence of 8-PSK Power Capability field. The EGPRS Multi Slot Class field is coded as the binary representation of the multislot class defined in 3GPP TS 45.002 [32]. The same multislot capability is applicable also for EGPRS2 if supported. The same multislot capability is applicable also for Downlink Multi Carrier configuration if supported. GPRS Extended Dynamic Allocation Capability 0 Extended Dynamic Allocation Capability for GPRS is not implemented 1 Extended Dynamic Allocation Capability for GPRS is implemented If a multislot class type 1 MS indicates in the GPRS Multi Slot Class field the support of a multislot class for which three or more uplink timeslots can be assigned, Extended Dynamic Allocation for GPRS shall be implemented in the mobile station. EGPRS Extended Dynamic Allocation Capability 0 Extended Dynamic Allocation Capability for EGPRS and EGPRS2 (if supported) is not implemented 1 Extended Dynamic Allocation Capability for EGPRS and EGPRS2 (if supported) is implemented If a multislot class type 1 MS indicates in the EGPRS Multi Slot Class field the support of a multislot class for which three or more uplink timeslots can be assigned, Extended Dynamic Allocation for EGPRS and EGPRS2 (if supported) shall be implemented in the mobile station. SMS_VALUE (Switch-Measure-Switch) (4 bit field) The SMS field indicates the time needed for the mobile station to switch from one radio channel to another, perform a neighbor cell power measurement, and the switch from that radio channel to another radio channel. This field is not used by the network and may be excluded by the MS. Bits 4 3 2 1 0 0 0 0 1/4 timeslot (~144 microseconds) 0 0 0 1 2/4 timeslot (~288 microseconds) 0 0 1 0 3/4 timeslot (~433 microseconds) . . . 1 1 1 1 16/4 timeslot (~2307 microseconds) (SM_VALUE) Switch-Measure (4 bit field) The SM field indicates the time needed for the mobile station to switch from one radio channel to another and perform a neighbour cell power measurement. This field is not used by the network and may be excluded by the MS. Bits 4 3 2 1 0 0 0 0 1/4 timeslot (~144 microseconds) 0 0 0 1 2/4 timeslot (~288 microseconds) 0 0 1 0 3/4 timeslot (~433 microseconds) . . . 1 1 1 1 16/4 timeslot (~2307 microseconds)

DTM GPRS Multi Slot Class (2 bit field) This field indicates the DTM GPRS multislot capabilities of the MS. It is coded as follows: Bits 2 1 0 0 Unused. If received, the network shall interpret this as ‘01’ 0 1 Multislot class 5 supported 1 0 Multislot class 9 supported 1 1 Multislot class 11 supported This field shall contain one of the following values if the DTM GPRS High Multi Slot Class field is present: Multislot class 9 if DTM GPRS High Multi Slot Class is set to indicate Class 31/36 or Class 41; Multislot class 11 if DTM GPRS High Multi Slot Class is set to indicate Classes 32/37, 33/38 or Classes 42, 43, 44. Single Slot DTM (1 bit field) This field indicates whether the MS supports single slot DTM operation (see 3GPP TS 43.055 [87]). Bit 0 Single Slot DTM not supported 1 Single Slot DTM supported An MS indicating support for Extended DTM GPRS multislot class or Extended DTM EGPRS multislot class shall set this bit to ‘1’. The network may ignore the bit in this case. DTM EGPRS Multi Slot Class (2 bit field) This field indicates the DTM EGPRS multislot capabilities of the MS. This field shall be included only if the mobile station supports EGPRS DTM. This field is coded as the DTM GPRS multislot Class field. This field shall contain one of the following values if the DTM EGPRS High Multi Slot Class field is present: Multislot class 9 if DTM EGPRS High Multi Slot Class is set to indicate Class 31/36 or Class 41; Multislot class 11 if DTM EGPRS High Multi Slot Class is set to indicate Classes 32/37, 33/38 or Classes 42, 43, 44. The same multislot capability is applicable also for EGPRS2 if supported. COMPACT Interference Measurement Capability (1 bit field) 0 COMPACT Interference Measurement Capability is not implemented 1 COMPACT Interference Measurement Capability is implemented Revision Level Indicator (1 bit field) Bit 0 The ME is Release ’98 or older 1 The ME is Release ’99 onwards UMTS FDD Radio Access Technology Capability (1 bit field) Bit 0 UMTS FDD not supported 1 UMTS FDD supported UMTS 3.84 Mcps TDD Radio Access Technology Capability (1 bit field) Bit 0 UMTS 3.84 Mcps TDD not supported 1 UMTS 3.84 Mcps TDD supported CDMA 2000 Radio Access Technology Capability (1 bit field) Bit 0 CDMA 2000 not supported 1 CDMA 2000 supported UMTS 1.28 Mcps TDD Radio Access Technology Capability (1 bit field) Bit 0 UMTS 1.28 Mcps TDD not supported 1 UMTS 1.28 Mcps TDD supported GERAN Feature Package 1 (1 bit field) The support of interworking towards E-UTRA is indicated separately in the "GERAN to E-UTRA support in GERAN packet transfer mode" field. This field indicates whether the MS supports the GERAN Feature Package 1 (see 3GPP TS 44.060 [76]). It is coded as follows: 0 GERAN feature package 1 not supported. 1 GERAN feature package 1 supported. Extended DTM GPRS Multi Slot Class (2 bit field) This field indicates the extended DTM GPRS capabilities of the MS and shall be interpreted in conjunction with the DTM GPRS Multi Slot Class field. It is coded as follows, where ‘DGMSC’ denotes the DTM GPRS multislot class field: DGMSC Bit 2 1 Bit 2 1 0 0 0 0 Unused. If received, it shall be interpreted as ‘01 00’ 0 0 0 1 Unused. If received, it shall be interpreted as ‘01 00’ 0 0 1 0 Unused. If received, it shall be interpreted as ‘01 00’ 0 0 1 1 Unused. If received, it shall be interpreted as ‘01 00’ 0 1 0 0 Multislot class 5 supported 0 1 0 1 Multislot class 6 supported 0 1 1 0 Unused. If received, it shall be interpreted as ‘01 00’ 0 1 1 1 Unused. If received, it shall be interpreted as ‘01 00’ 1 0 0 0 Multislot class 9 supported 1 0 0 1 Multislot class 10 supported 1 0 1 0 Unused. If received, it shall be interpreted as ’10 00’ 1 0 1 1 Unused. If received, it shall be interpreted as ’10 00’ 1 1 0 0 Multislot class 11 supported 1 1 0 1 Unused. If received, it shall be interpreted as ’11 00’ 1 1 1 0 Unused. If received, it shall be interpreted as ’11 00’ 1 1 1 1 Unused. If received, it shall be interpreted as ’11 00’ The presence of this field indicates that the MS supports combined fullrate and halfrate GPRS channels in the downlink. When this field is not present, the MS supports the multislot class indicated by the DTM GPRS Multi Slot Class field. If this field is included, it shall contain one of the following values if the DTM GPRS High Multi Slot Class field is present: Multislot class 10 if DTM GPRS High Multi Slot Class is set to indicate Class 31/36 or Class 41; Multislot class 11 if DTM GPRS High Multi Slot Class is set to indicate Classes 32/37, 33/38 or Classes 42, 43, 44. Extended DTM EGPRS Multislot Class (2 bit field) This field is not considered when the DTM EGPRS Multislot Class field is not included. This field indicates the extended DTM EGPRS multislot capabilities of the MS and shall be interpreted in conjunction with the DTM EGPRS Multislot Class field. This field is coded as the Extended DTM GPRS Multislot Class field. The presence of this field indicates that the MS supports combined fullrate and halfrate GPRS channels in the downlink. When this field is not present, the MS supports the multislot class indicated by the DTM EGPRS Multi Slot Class field. If this field is included, it shall contain one of the following values if the DTM EGPRS High Multi Slot Class field is present: Multislot class 10 if DTM EGPRS High Multi Slot Class is set to indicate Class 31/36 or Class 41; Multislot class 11 if DTM EGPRS High Multi Slot Class is set to indicate Classes 32/37, 33/38 or Classes 42, 43, 44. Modulation based multislot class support (1 bit field) Bit 0 "Modulation based multislot class" not supported 1 "Modulation based multislot class" supported High Multislot Capability (2 bit field) The High Multislot Capability is individually combined with each multislot class field sent by the MS (the possible multislot class fields are: GPRS multislot class, EGPRS multislot class) to extend the related multislot class to multislot classes 30 to 45, see 3GPP TS 45.002 [32]. The same capability is applicable also to EGPRS2 if supported. The same capability is applicable also to Downlink Multi Carrier configuration if supported. For each multislot class, the following mapping is done: Bits 2 1 coded multislot class field actual multislot class 0 0 8 30 0 0 10, 23, 28, 29 39 0 0 11, 20, 25 32 0 0 12, 21, 22, 26, 27 33 0 0 Any other Multislot Class field value 0 1 8 35 0 1 10, 19, 24 36 0 1 11, 23, 28, 29 45 0 1 12, 21, 22, 26, 27 38 0 1 Any other Multislot Class field value 1 0 8 40 1 0 10, 19, 24 41 1 0 11, 20, 25 42 1 0 12, 23, 28, 29 44 1 0 Any other Multislot Class field value 1 1 12, 21, 22, 26, 27 43 1 1 11, 20, 25 37 1 1 10, 19, 24 31 1 1 9, 23, 28, 29 34 1 1 Any other Multislot Class field value GMSK Multislot Power Profile (2 bit field) For detailed definitions, see the Mobile Station Classmark 3 information element. 8-PSK Multislot Power Profile (2 bit field) For detailed definitions, see the Mobile Station Classmark 3 information element. Multiple TBF Capability (1 bit field) Bit 0 Multiple TBF procedures in A/Gb mode not supported 1 Multiple TBF procedures in A/Gb mode supported Downlink Advanced Receiver Performance (2 bit field) This field indicates Downlink Advanced Receiver Performance capabilities of the MS (see 3GPP TS 45.005 [33]). Bits 2 1 0 0 Downlink Advanced Receiver Performance not supported 0 1 Downlink Advanced Receiver Performance – phase I supported 1 0 Downlink Advanced Receiver Performance – phase II supported The value ‘11’ shall not be used by the MS. If the value ‘11’ is received by the network, it shall be interpreted as ‘10’ . Extended RLC/MAC Control Message Segmentation capability (1 bit field) Bit 0 Extended RLC/MAC control message segmentation not supported 1 Extended RLC/MAC control message segmentation supported DTM Enhancements Capability (1 bit field) This field indicates whether the mobile station supports enhanced DTM CS establishment and enhanced DTM CS release or not. It is coded as follows: Bit 0 The mobile station does not support enhanced DTM CS establishment and enhanced DTM CS release procedures. 1 The mobile station supports enhanced DTM CS establishment and enhanced DTM CS release procedures. DTM GPRS High Multi Slot Class (3 bit field) This field indicates the DTM GPRS multislot capabilities of the MS. It is coded as follows: Bit 3 2 1 0 0 0 Unused. If received, the network shall interpret this as ‘0 0 1’ 0 0 1 Multislot class 31 or 36 supported 0 1 0 Multislot class 32 or 37 supported 0 1 1 Multislot class 33 or 38 supported 1 0 0 Multislot class 41 supported 1 0 1 Multislot class 42 supported 1 1 0 Multislot class 43 supported 1 1 1 Multislot class 44 supported The presence of this field indicates that the MS supports the DTM extension to high multislot classes. When this field is not present, the MS supports the DTM multislot class indicated by the DTM GPRS Multi Slot Class field. The values '0 0 1', '0 1 0' and '0 1 1' shall be interpreted as indicating DTM GPRS multislot class 36, 37 or 38 respectively in case the MS indicates support for one of the GPRS multislot classes 35 to 39; in all other cases those codepoints shall be interpreted as indicating DTM GPRS multislot class 31, 32 or 33 respectively. This field shall be ignored if the High Multislot Capability field is not present. DTM EGPRS High Multi Slot Class (3 bit field) This field indicates the DTM EGPRS multislot capabilities of the MS. This field may be included only if the mobile station supports EGPRS DTM. This field is coded as the DTM GPRS High Multi Slot Class field. When this field is not present, the MS supports the DTM multislot class indicated by the DTM EGPRS Multi Slot Class field. The values '0 0 1', '0 1 0' and '0 1 1' shall be interpreted as indicating DTM EGPRS multislot class 36, 37 or 38 respectively in case the MS indicates support for one of the EGPRS multislot classes 35 to 39; in all other cases those codepoints shall be interpreted as indicating DTM EGPRS multislot class 31, 32 or 33 respectively. This field shall be ignored if the High Multislot Capability field is not present. The same multislot capability is applicable also for EGPRS2 if supported. PS Handover Capability (1 bit field) This field indicates whether the mobile station supports PS Handover. The PS Handover Capability applies to all RATs and modes indicated as supported in this information element, except for E-UTRA, where the support is indicated separately in the "GERAN to E-UTRA support in GERAN packet transfer mode" field. Bit 0 The mobile station does not support PS Handover. 1 The mobile station supports PS Handover DTM Handover Capability (1 bit field) This field indicates whether the mobile station supports DTM Handover. The DTM Handover Capability applies to all RATs and modes indicated as supported in this information element. It is coded as follows: Bit 0 The mobile station does not support DTM Handover. 1 The mobile station supports DTM Handover Multislot Capability Reduction for Downlink Dual Carrier (3 bit field) This field indicates the receive multislot capability reduction of a dual carrier capable mobile station applicable to EGPRS and EGPRS2 support when EFTA is not used (see 3GPP TS 45.002 [32]). This reduction applies to the maximum number of downlink timeslots for dual carrier operation derived from the (DTM) EGPRS (high) multislot class. The field is coded as follows: Bit 3 2 1 0 0 0 No reduction 0 0 1 The MS supports 1 timeslot fewer than the maximum number of receive timeslots 0 1 0 The MS supports 2 timeslots fewer than the maximum number of receive timeslots 0 1 1 The MS supports 3 timeslots fewer than the maximum number of receive timeslots 1 0 0 The MS supports 4 timeslots fewer than the maximum number of receive timeslots 1 0 1 The MS supports 5 timeslots fewer than the maximum number of receive timeslots 1 1 0 The MS supports 6 timeslots fewer than the maximum number of receive timeslots 1 1 1 Reserved for future use The presence of this field also indicates that the mobile station supports dual carrier in the downlink for EGPRS. Downlink Dual Carrier for DTM Capability (1 bit field) This field indicates whether the mobile station supports DTM during downlink dual carrier operation. Bit 0 The mobile station does not support DTM during downlink dual carrier operation 1 The mobile station supports DTM during downlink dual carrier operation If the mobile station supports DTM during downlink dual carrier operation as indicated by this field, the Multislot Capability Reduction for Downlink Dual Carrier field provided in the MS Radio Access Capability IE is applicable to EGPRS DTM support as well. Flexible Timeslot Assignment (1 bit field) This field indicates whether the mobile station supports Flexible Timeslot Assignment (see 3GPP TS 45.002 [32]). 0 The mobile station does not support Flexible Timeslot Assignment 1 The mobile station supports Flexible Timeslot Assignment GAN PS Handover Capability (1 bit field) This field indicates whether or not the mobile station supports PS Handover from GERAN/UTRAN cell to a GAN cell. The field is coded as follows: Bit 0 The mobile station does not support PS Handover from GERAN/UTRAN cell to a GAN cell 1 The mobile station supports PS Handover from GERAN/UTRAN cell to a GAN cell RLC Non-persistent Mode Capability (1 bit field) This field indicates whether the mobile station supports RLC Non-persistent Mode (see 3GPP TS 44.060 [76]). 0 The mobile station does not support RLC Non-persistent Mode 1 The mobile station supports RLC Non-persistent Mode Reduced Latency Capability (1 bit field) This field indicates whether the mobile station supports Reduced TTI configurations and Fast Ack/Nack Reporting (see 3GPP TS 44.060 [76]) in packet transfer mode for both EGPRS and, if supported, EGPRS2. 0 The mobile station does not support Reduced TTI configurations and Fast Ack/Nack Reporting 1 The mobile station supports Reduced TTI configurations and Fast Ack/Nack Reporting A mobile station whose multislot class does not allow the support of Reduced TTI configurations in packet transfer mode due to a limited number of uplink or downlink timeslots (see 3GPP TS 45.002 [32]) shall set the Reduced Latency Capability field to '0'. Uplink EGPRS2 (2 bit field) This field indicates whether the mobile station supports EGPRS2-A or EGPRS2-A and EGPRS2-B in the uplink. Bit 2 1 0 0 The mobile station does not support either EGPRS2-A or EGPRS2-B in the uplink 0 1 The mobile station supports EGPRS2-A in the uplink 1 0 The mobile station supports both EGPRS2-A and EGPRS2-B in the uplink 1 1 This value is not used in this release/version of the specifications. If received it shall be interpreted as ‘10’ Downlink EGPRS2 (2 bit field) This field indicates whether the mobile station supports EGPRS2-A or EGPRS2-A and EGPRS2-B in the downlink. Bit 2 1 0 0 The mobile station does not support either EGPRS2-A or EGPRS2-B in the downlink 0 1 The mobile station supports EGPRS2-A in the downlink 1 0 The mobile station supports both EGPRS2-A and EGPRS2-B in the downlink 1 1 This value is not used in this release/version of the specifications. If received it shall be interpreted as ‘10’ E-UTRA FDD support (1 bit field) Bit 0 E-UTRA FDD not supported 1 E-UTRA FDD supported E-UTRA TDD support (1 bit field) Bit 0 E-UTRA TDD not supported 1 E-UTRA TDD supported GERAN to E-UTRA support in GERAN packet transfer mode (2 bit field) This field indicates the capabilities supported by the mobile station in packet transfer mode for GERAN to E-UTRA interworking. If both "E-UTRA FDD support" and "E-UTRA TDD support" bits are set to '0', the bit field shall be set to '0 0'. If one or both of "E-UTRA FDD support" and "E-UTRA TDD support" bits are set to '1', the bit field may be any of the listed values. The bit field is coded as follows: Bit 2 1 0 0 None 0 1 E-UTRAN Neighbour Cell measurements and MS autonomous cell reselection to E-UTRAN supported 1 0 CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN supported in addition to capabilities indicated by '01' 1 1 PS Handover to E-UTRAN supported in addition to capabilities indicated by '01' and '10' Priority-based reselection support (1 bit field) This field indicates whether the mobile station supports priority-based cell reselection. 0 Priority-based cell reselection not supported 1 Priority-based cell reselection supported Alternative EFTA multislot class (4 bit field) The presence of the Alternative EFTA multislot class field indicates that the mobile station supports Enhanced Flexible Timeslot Assignment, EFTA, (see 3GPP TS 45.002 [32]). This field shall be ignored if the High Multislot Capability field is not present. The Alternative EFTA multislot class field is used together with the (DTM) EGPRS (high) multislot class to determine the mobile stations capabilities when using Enhanced Flexible Timeslot Assignment, EFTA, and is coded as follows: Bit 4 3 2 1 0 0 0 0 No Alternative EFTA multislot class is indicated. Use (DTM) EGPRS (high) multislot class only. 0 0 0 1 Alternative EFTA multislot class 1 0 0 1 0 Alternative EFTA multislot class 2 0 0 1 1 Alternative EFTA multislot class 3 0 1 0 0 to 1 1 1 1 Unused. If received, these values shall be interpreted as ’0000’ EFTA Multislot Capability Reduction for Downlink Dual Carrier (3 bit field) This field indicates the receive multislot capability reduction of a dual carrier capable mobile station applicable to EGPRS and EGPRS2 support when Enhanced Flexible Timeslot Assignment is used (see 3GPP TS 45.002 [32]). This reduction applies to the maximum number of downlink timeslots for dual carrier operation derived from the Alternative EFTA Multislot Class field. The coding of this field is the same as the coding of the Multislot Capability Reduction for Downlink Dual Carrier field. This field shall be ignored if a mobile station does not support Downlink Dual Carrier. Indication of Upper Layer PDU Start Capability for RLC UM (1 bit field) This field indicates whether the mobile station supports "Indication of Upper Layer PDU Start for RLC UM” (see 3GPP TS 44.060 [76]) for RLC unacknowledged mode of operation. The field is coded as follows: 0 The mobile station does not support "Indication of Upper Layer PDU Start for RLC UM” 1 The mobile station supports "Indication of Upper Layer PDU Start for RLC UM” EMST Capability (1 bit field) This field indicates whether the mobile station supports Enhanced Multiplexing for Single TBF (EMST) (see 3GPP TS 44.060 [76]). 0 The mobile station does not support EMST 1 The mobile station supports EMST MTTI Capability (1 bit field) This field indicates whether the mobile station supports multiple TTI (MTTI) configurations (see 3GPP TS 44.060 [76]) Bit 0 MTTI configurations not supported 1 MTTI configurations supported UTRA CSG Cells Reporting (1 bit field) This field indicates whether the mobile station supports reporting of measurements and routing parameters (see 3GPP TS 44.060 [76]) for UTRAN CSG cells in packet transfer mode. This capability shall apply to each UTRA radio access mode supported by the mobile. Bit 0 Reporting of UTRAN CSG cells in packet transfer mode not supported 1 Reporting of UTRAN CSG cells in packet transfer mode supported E-UTRA CSG Cells Reporting (1 bit field) This field indicates whether the mobile station supports reporting of measurements and routing parameters (see 3GPP TS 44.060 [76]) for E-UTRAN CSG cells in packet transfer mode. This capability shall apply to each E-UTRA radio access mode supported by the mobile. Bit 0 Reporting of E-UTRAN CSG cells in packet transfer mode not supported 1 Reporting of E-UTRAN CSG cells in packet transfer mode supported DTR Capability (1 bit field) This field indicates whether the mobile station supports Dynamic Timeslot Reduction (DTR), see 3GPP TS 44.060 [76]. 0 The mobile station does not support DTR 1 The mobile station supports DTR EMSR Capability (1 bit field) This field indicates whether the mobile station supports Enhanced Multiplexing for Single RLC Entity (EMSR), see 3GPP TS 44.060 [76]. 0 The mobile station does not support EMSR 1 The mobile station supports EMSR Fast Downlink Frequency Switching Capability (1 bit field) This field indicates whether the mobile station supports fast downlink frequency switching between two consecutive TDMA frames (see 3GPP TS 45.002 [32]). 0 Fast downlink frequency switching not supported 1 Fast downlink frequency switching supported TIGHTER Capability (2 bit field) This field indicates Tightened Link Level Performance support in the MS (see 3GPP TS 45.005 [33]). The tightened performance applies to the traffic channels and signalling channels specified in 3GPP TS 45.005 [33]. Bits 2 1 0 0 TIGHTER not supported 0 1 TIGHTER supported for speech and signalling channels only 1 0 TIGHTER supported for speech and signalling channels and for GPRS and EGPRS, but not for EGPRS2 1 1 TIGHTER supported for speech and signalling channels and for GPRS, EGPRS and EGPRS2 FANR Capability (1 bit field) This field indicates whether the mobile station supports Fast Ack/Nack Reporting (see 3GPP TS 44.060 [76]) in packet transfer mode for EGPRS and, if supported, EGPRS2. If the mobile station indicates support for Reduced TTI configurations and Fast Ack/Nack Reporting using the Reduced Latency Capability Indicator then the network shall ignore FANR Capability information. 0 The mobile station does not support Fast Ack/Nack Reporting 1 The mobile station supports Fast Ack/Nack Reporting IPA Capability (1 bit field) This field indicates whether the mobile station supports Immediate Packet Assignment (IPA), see 3GPP TS 44.018 [84]. Bit 0 The mobile station does not support IPA 1 The mobile station supports IPA GERAN Network Sharing support (1 bit field) This field indicates whether the mobile station supports GERAN network sharing. A mobile station supporting GERAN network sharing shall also support the extended EARFCN value range in GERAN and indicate this in the respective bit. Bit 0 The mobile station does not support GERAN network sharing 1 The mobile station supports GERAN network sharing E-UTRA Wideband RSRQ measurements support (1 bit field) This field indicates whether the mobile station supports E-UTRA wideband RSRQ measurements (see 3GPP TS 45.008 [151]). Bit 0 The mobile station does not support E-UTRA wideband RSRQ measurements 1 The mobile station supports E-UTRA wideband RSRQ measurements UTRA Multiple Frequency Band Indicators support (1 bit field) This field indicates whether the mobile station supports multiple radio frequency bands in UTRAN (see 3GPP TS 25.331 [23c]) and whether it understands signalling of overlapping UTRA frequency bands (see 3GPP TS 44.060 [76]). Bit 0 The mobile station does not support Multiple Frequency Band Indicators in UTRAN 1 The mobile station supports Multiple Frequency Band Indicators in UTRAN E-UTRA Multiple Frequency Band Indicators support (1 bit field) This field indicates whether the mobile station supports multiple radio frequency bands in E-UTRAN (see 3GPP TS 36.331 [129]) and whether it understands signalling of overlapping E-UTRA frequency bands (see 3GPP TS 44.060 [76]). Bit 0 The mobile station does not support Multiple Frequency Band Indicators in E-UTRAN 1 The mobile station supports Multiple Frequency Band Indicators in E-UTRAN DLMC - Non-contiguous intra-band reception (2 bit field) This field indicates the intra-band non-contiguous reception mode (see 3GPP TS 45.005 [33]) supported by the mobile station in Downlink Multicarrier configurations. The absence of this field indicates that a mobile station does not support intra-band non-contiguous reception. Bit 2 1 0 0 Not supported 0 1 Supported in band E-GSM or GSM850 1 0 Supported in band DCS1800 or PCS1900 1 1 Supported in band E-GSM, or GSM850, or DCS1800 or PCS1900 DLMC - Inter-band reception (1 bit field) This field indicates the inter-band reception mode (see 3GPP TS 45.005 [33]) supported by the mobile station in Downlink Multicarrier configuration. The absence of this field indicates that a mobile station does not support inter-band reception. Bit 0 Not supported 1 Supported in band combination (E-GSM, DCS1800), or band combination (GSM850, PCS1900). DLMC - Maximum Bandwidth (2 bit field) This field indicates the maximum bandwidth supported by a mobile station. Bit 2 1 0 0 Bandwidth = 5 MHz 0 1 Bandwidth = 10 MHz 1 0 Bandwidth = 15 MHz 1 1 Bandwidth = 20 MHz DLMC - Maximum Number of Downlink Timeslots (6 bit field) This field indicates the maximum number of downlink timeslots supported by a mobile station in Downlink Multi Carrier configuration. Bit 6 5 4 3 2 1 0 0 0 0 0 0 6 TS supported 0 0 0 0 0 1 8 TS supported 0 0 0 0 1 0 10 TS supported 0 0 0 0 1 1 12 TS supported … … 1 1 1 1 0 1 128 TS supported 1 1 1 1 1 0 reserved 1 1 1 1 1 1 reserved DLMC - Maximum Number of Downlink Carriers (3 bit field) This field indicates the maximum number of downlink carriers supported by a mobile station in Downlink Multi Carrier configuration. Bit 3 2 1 0 0 0 2 carriers supported 0 0 1 4 carriers supported 0 1 0 6 carriers supported 0 1 1 8 carriers supported 1 0 0 10 carriers supported 1 0 1 12 carriers supported 1 1 0 14 carriers supported 1 1 1 16 carriers supported Extended TSC Set Capability support (1 bit field) This field indicates whether the mobile station supports the extended TSC sets when operating in the PS or CS domain (see 3GPP TS 45.002 [32]). Bit 0 The mobile station does not support Extended TSC sets 1 The mobile station supports Extended TSC sets Extended EARFCN value range (1 bit field) This field indicates whether the mobile station supports the extended EARFCN value range in GERAN (see 3GPP TS 44.060 [76]). Bit 0 The mobile station does not support extended EARFCN value range 1 The mobile station supports extended EARFCN value range (EC-)PCH monitoring support (2 bit field) This field indicates if the mobile station is capable of monitoring the PCH channel, the EC-PCH channel or both for paging based reachability. Bit 2 1 0 0 PCH supported 0 1 EC-PCH supported 1 0 PCH and EC-PCH supported 1 1 reserved EC-PCH support also indicates EC-GSM-IoT capability, see 3GPP TS 43.064 [159]. EC uplink coverage enhancement support (1 bit field) This field indicates if the mobile station supports uplink coverage class CC5. Bit 0 The mobile station does not support uplink coverage class CC5 1 The mobile station supports uplink coverage class CC5

End of Modification