3GPP TSG-SA3 Meeting #116	S3-242078

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | 0024 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test Name: missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test names. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test names. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1, 4.2.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******Start of changes*****

#### 4.2.7.1 UP	Security enforcement configuration for TSC service

Requirement Name: UP security enforcement configuration

Requirement Reference: TS 33.501 [2], clause L.3, TS 23.501 [5], clause 5.10.3.

Requirement Description:

"After the 5GS TSC-enabled UE is authenticated and data connection is set up, any data received from a TSC bridge or another 5GS TSC-enabled UE shall be transported between DS-TT (in the UE) and NW-TT (in the UPF) in a protected way using the mechanisms for UP security as described in clause 6.6.

The UP security enforcement information shall be set to "required" for data transferred from gNB to a 5GS TSC-enabled UE. This is also applicable to the gPTP messages sent in the user plane."

as specified in TS 33.501 [2], clause L.3.

"The SMF determines at PDU session establishment a User Plane Security Enforcement information for the user plane of a PDU session based on:

-	subscribed User Plane Security Policy which is part of SM subscription information received from UDM; and

-	User Plane Security Policy locally configured per (DNN, S-NSSAI) in the SMF that is used when the UDM does not provide User Plane Security Policy information.

-	The maximum supported data rate per UE for integrity protection for the DRBs, provided by the UE in the Integrity protection maximum data rate IE during PDU Session Establishment. The UE supporting NR as primary RAT, i.e. NG-RAN access via Standalone NR, shall set the Integrity protection maximum data rate IE for Uplink and Downlink to full rate at PDU Session Establishment as defined in TS 24.501 [47]."

as specified in TS 23.501 [5], clause 5.10.3.

Threat References: TR 33.926 [4].

NOTE:	The test case below only applies to the UDMs which support the setting and providing of User Plane Security Policy for dedicated TSC service.

Test Case:

Test Name: TC_UP_SECURITY_ENFORCEMENT_CONFIGURATION

Purpose:

Verify that UP security enforcement information is set to "required" for dedicated TSC service.

Pre-Conditions:

Test environment with SMF. The SMF may be simulated.

A dedicated DNN/S-NSSAI combination is defined to identify the TSC service.

The security policy is configured in the UDM.

Execution Steps

1.	During the PDU session establishment procedure, the SMF sends a Nudm_SDM_Get Request message to the UDM under test with a dedicated DNN/S-NSSAI combination.

2.	The UDM under test sends the Nudm_SDM_Get Response back to the SMF with UP security enforcement information.

Expected Results:

The confidentiality and integrity protection requirements of the UP security enforcement information are set to "required".

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

******Start of 2nd changes*****

#### 4.2.8.1	UP security policy configuration for 5G LAN service

Requirement Name: UP security enforcement configuration

Requirement Reference: TS 33.501 [2], clause K.3, TS 23.501 [5], clause 5.10.3.

Requirement Description: "To reduce incremental complexity added by security, all PDU sessions associated with a specific 5G LAN group should have the same UP security policy. When generating the policy enforcement information, and to avoid the redundant double protection, the SMF may consider information by a DN-AAA about DN protection mechanisms already applied."

as specified in TS 33.501 [2], clause K.3.

"The SMF determines at PDU session establishment a User Plane Security Enforcement information for the user plane of a PDU session based on:

-	subscribed User Plane Security Policy which is part of SM subscription information received from UDM; and

-	User Plane Security Policy locally configured per (DNN, S-NSSAI) in the SMF that is used when the UDM does not provide User Plane Security Policy information.

-	The maximum supported data rate per UE for integrity protection for the DRBs, provided by the UE in the Integrity protection maximum data rate IE during PDU Session Establishment. The UE supporting NR as primary RAT, i.e. NG-RAN access via Standalone NR, shall set the Integrity protection maximum data rate IE for Uplink and Downlink to full rate at PDU Session Establishment as defined in TS 24.501 [47]."

as specified in TS 23.501 [5], clause 5.10.3.

Threat References: TR 33.926 [4].

NOTE 1:	The test case below only applies to the UDMs which support the setting and providing of User Plane Security Policy for 5G LAN service.

Test Case:

Test Name: TC_UP_SECURITY_ENFORCEMENT_CONFIGURATION_FOR_5G_LAN

Purpose:

Verify that UP security policy is set to the same for all the 5G LAN UEs.

Pre-Conditions:

Test environment with SMF. The SMF may be simulated.

A dedicated DNN/S-NSSAI combination is defined to identify the 5G LAN service.

The security policy of the 5G LAN service is configured in the UDM.

Execution Steps

1.	During the PDU session establishment procedure initiated by the UE1, the SMF1 sends a Nudm_SDM_Get Request message to the UDM under test with a dedicated DNN/S-NSSAI combination, and SUPI1.

2.	The UDM under test sends the Nudm_SDM_Get Response back to the SMF1 with UP security policy1.

3.	During the PDU session establishment procedure initiated by the UE2, the SMF2 sends a Nudm_SDM_Get Request message to the UDM under test with a dedicated DNN/S-NSSAI combination, and SUPI2.

4.	The UDM under test sends the Nudm_SDM_Get Response back to the SMF2 with UP security policy2.

NOTE 2:	SMF1 and SMF2 could be the same network function.

Expected Results:

The confidentiality and integrity protection requirements of the UP security policy1 and UP security policy2 are the same.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

******End of changes*****