3GPP TSG-SA3 Meeting #82	S3-160308

# Dubrovnik, Croatia, 1st Feb 2016 - 5th Feb 2016						revision of S3-160007,0223

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0564 | rev | 2 | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding NB-IoT keys and processes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | VODAFONE Group Plc, Nokia Networks, Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT |  |  |  |  |  | Date: |  |  | 2016-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Addition of security mechanisms for user data via the MME for Cellular (Narrowband) Internet of Things |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 3.1 - addition of user plane and Data via MME definitions Section 3.2 – addtion of NASDEV abbrieviation Section 4 – addition of note to cover user data sent via MME Section 5.1.3.1 – addition of user data sent via MME ciphering requirements Section 5.1.4.1 – addition of user data sent via MME integrity requirements Section 6.5 – addition of Editor's Note on COUNT handling for data sent via MME and minor changes Section 8 - Addition of Editor's Note on COUNT handling for data sent via MME and minor changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user data via MME cannot be secured on a network level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4, 5.1.3.1, 5.1.4.1, 6.5, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[…]

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 33.102 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Security Management Entity: entity which receives the top-level keys in an access network from the HSS. For E-UTRAN access networks, the role of the ASME is assumed by the MME

Activation of security context: the process of taking into use a security context.

Authentication data: Data that is part of a security context or of authentication vectors.

Chaining of KeNB: derivation of a new KeNB from another KeNB (i.e., at cell handover)

Current EPS security context: The security context which has been activated most recently. Note that a current EPS security context originating from either a mapped or native EPS security context may exist simultaneously with a native non-current EPS security context.

ECM-CONNECTED state: This is as defined in TS 23.401 [2]. The term ECM-CONNECTED state corresponds to the term EMM-CONNECTED mode used in TS 24.301 [9].

ECM-IDLE state: As defined in TS 23.401 [2]. The term ECM-IDLE state corresponds to the term EMM-IDLE mode used in TS 24.301 [9].

EPS-Authentication Vector: KASME, RAND, AUTN, XRES

EPS security context: A state that is established locally at the UE and a serving network domain. At both ends "EPS security context data" is stored, that consists of the EPS NAS security context, and the EPS AS security context.

NOTE 1:	An EPS security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

EPS AS security context: the cryptographic keys at AS level with their identifiers, the Next Hop parameter NH, the Next Hop Chaining Counter parameter NCC used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, counters used for replay protection and SCG Counter used as freshness input into S-KeNB derivations. Note that the EPS AS security context only exists when cryptographically protected radio bearers are established and is otherwise void.

NOTE 2: NH and NCC need to be stored also at the MME during connected mode.

EPS AS Secondary Cell security context: This context consists of the cryptographic keys for SeNB (KUPenc), the identifier of the selected AS SC level cryptographic algorithm and counters used for replay protection.

EPS NAS security context: This context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. In particular, separate pairs of NAS COUNT values are used for each EPS NAS security contexts, respectively. The distinction between native and mapped EPS security contexts also applies to EPS NAS security contexts. The EPS NAS security context is called "full" if it additionally contains the keys KNASint and KNASenc and the identifiers of the selected NAS integrity and encryption algorithms.

Full native EPS security context: A native EPS security context for which the EPS NAS security context is full according to the above definition. A full native EPS security context is either in state "current" or state "non-current".

Forward security: In the context of KeNB key derivation, forward security refers to the property that, for an eNB with knowledge of a KeNB, shared with a UE, it shall be computationally infeasible to predict any future KeNB, that will be used between the same UE and another eNB. More specifically, n hop forward security refers to the property that an eNB is unable to compute keys that will be used between a UE and another eNB to which the UE is connected after n or more handovers (n=1 or 2).

Legacy security context: A security context which has been established according to TS 33.102 [4].

Mapped security context: Security context created by converting the current security context in the source system to a security context for the target system in inter-system mobility, e.g., UMTS keys created from EPS keys. The EPS NAS security context of a mapped security context is full and current.

Native EPS security context: An EPS security context whose KASME was created by a run of EPS AKA.

Non-current EPS security context: A native EPS security context that is not the current one. A non-current EPS security context may be stored along with a current EPS security context in the UE and the MME. A non-current EPS security context does not contain an EPS AS security context. A non-current EPS security context is either of type "full native" or of type "partial native".

Partial native EPS security context: A partial native EPS security context consists of KASME with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context. A partial native EPS security context is created by an EPS AKA, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

Re-derivation of NAS keys: derivation of new NAS keys from the same KASME but including different algorithms (and no freshness parameter)

Refresh of KeNB: derivation of a new KeNB from the same KASME and including a freshness parameter

Re-keying of KeNB: derivation of a new KeNB from a new KASME in ECM-CONNECTED (i.e., . to activate a partial native EPS security context, or to re-activate a non-current full EPS security context)

Re-keying of NAS keys: derivation of new NAS keys from a new KASME

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE. This includes capabilities for EPS AS and NAS, and includes capabilities for UTRAN and GERAN if these access types are supported by the UE.

UE EPS security capabilities: The UE security capabilities for EPS AS and NAS.

User plane: Within the context of TS 33.401, this means the data path between UE and Serving Gateway that does NOT go via the MME.

(User) Data via MME: User Data sent to or from the UE that uses an RRC connection established using the Control Plane CIoT EPS optimisation specified in TS 23.401[2].

IOPS-capable eNB: an eNB that has the capability of IOPS mode operation, which provides local IP connectivity and Public Safety services to IOPS-enabled UEs via a Local EPC when the eNB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNBs in IOPS mode of operation use, instead of the Macro EPC, in order to support Public Safety services.

Macro EPC: the EPC which serves an eNB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNB(s)), local IP connectivity and Public Safety services to IOPS-enabled UEs in the absence of normal EPS.IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

||	Concatenation

Bitwise Exclusive Or (XOR) operation

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LSB	Least Significant Bit

LSM	Limited Service Mode

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

XRES	Expected Response

[…]

# 4	Overview of Security Architecture

Figure 4-1 gives an overview of the complete security architecture.

Figure 4-1: Overview of the security architecture

Five security feature groups are defined. Each of these feature groups meets certain threats and accomplishes certain security objectives:

-	Network access security (I): the set of security features that provide users with secure access to services, and which in particular protect against attacks on the (radio) access link.

-	Network domain security (II): the set of security features that enable nodes to securely exchange signalling data, user data (between AN and SN and within AN), and protect against attacks on the wireline network.

-	User domain security (III): the set of security features that secure access to mobile stations.

-	Application domain security (IV): the set of security features that enable applications in the user and in the provider domain to securely exchange messages.

-	Visibility and configurability of security (V): the set of features that enables the user to inform himself whether a security feature is in operation or not and whether the use and provision of services should depend on the security feature.

NOTE 1:	Relay nodes are not explicitly shown in Figure 4-1. They combine the functionalities of ME and AN in a way described in 3GPP TS 36.300 [30]. The present document describes how to apply security features to relay nodes.

NOTE 2:	Release 13 introduces an option for some uplink and downlink user data to be sent via the MME. This is referred to as “data via MME” and within the context of TS 33.401 the abbreviation NASDVM is used.

### […]

#### 5.1.3.1	Ciphering requirements

Ciphering may be provided to RRC-signalling to prevent UE tracking based on cell level measurement reports, handover message mapping, or cell level identity chaining. RRC signalling confidentiality is an operator option.

All S1 and X2 messages carried between RN and DeNB shall be confidentiality-protected.

NOTE 0:	Encryption is subject to national regulation.

Synchronization of the input parameters for ciphering shall be ensured for the protocols involved in the ciphering.

The NAS signalling may be confidentiality protected. NAS signalling confidentiality is an operator option.

NOTE 1:	RRC and NAS signalling confidentiality protection is recommended to be used.

When authentication of the credentials on the UICC during Emergency Calling in Limited Service Mode, as defined in the TS 23.401 [2], can not be successfully performed, the confidentiality protection of the RRC and NAS signaling, and user plane shall be omitted (see clause 15). This shall be accomplished by the network by selecting EEA0 for confidentiality protection of NAS, RRC and user plane.

User Plane confidentiality protection over the access stratum shall be done at PDCP layer and is an operator option.

NOTE 2:	User plane confidentiality protection is recommended to be used.

NOTE 3:	Confidentiality protection for RRC and UP is applied at the PDCP layer, and no layers below PDCP are confidentiality protected. Confidentiality protection for NAS is provided by the NAS protocol.

User data sent via MME may be confidentiality protected.

NOTE 4:	Confidentiality protection of user data sent via MME  is recommended to be used.

[…]

### 5.1.4	User data and signalling data integrity

#### 5.1.4.1	Integrity requirements

Synchronization of the input parameters for integrity protection shall be ensured for the protocols involved in the integrity protection.

Integrity protection, and replay protection, shall be provided to NAS and RRC-signalling.

All NAS signaling messages except those explicitly listed in TS 24.301 [9] as exceptions shall be integrity-protected. All RRC signaling messages except those explicitly listed in TS 36.331 [21] as exceptions shall be integrity-protected.

When authentication of the credentials on the UICC during Emergency Calling in Limited Service Mode, as defined in the TS 23.401 [2], can not be successfully performed, the integrity and replay protection of the RRC and NAS signaling shall be omitted (see clause 15). This shall be accomplished by the network by selecting EIA0 for integrity protection of NAS and RRC. EIA0 shall only be used for unauthenticated emergency calls.

User plane packets between the eNB and the UE shall not be integrity protected on the Uu interface. User plane packets between the RN and the UE shall not be integrity protected. All user plane packets carrying S1 and X2 messages between RN and DeNB shall be integrity-protected. Integrity protection for all other user plane packets between RN and DeNB may be supported.

All user data packets sent via the MME shall be integrity protected.

[…]

## 6.5	Handling of NAS COUNTs

Each separate KASME has a distinct pair of NAS COUNTs, one NAS COUNT for uplink and one NAS COUNT for downlink, associated with it.

Editor's Note: There will be precisely one pair of NAS COUNTs for NAS signalling. But it is ffs whether there will be one or more separate pairs of NAS COUNTs for NAS data via MME, e.g. one pair of NAS COUNTs per bearer.  This depends on CT1 decisions.

It is essential that the NAS COUNTs for a particular KASME are not reset to the start values (that is the NAS COUNTs only have their start value when a new KASME is created). This prevents the security issue of using the same NAS COUNTs with the same NAS keys, e.g. key stream re-use, in the case a UE moves back and forth between two MMEs and the same NAS keys are re-derived.

The NAS COUNTs shall only be set to the start value in the following cases:

-	for a partial native EPS NAS security context created by a successful AKA run,

NOTE:	The NAS COUNTs are not actually needed at the UE for a native context until it has successfully received the first NAS Security Mode Command for that security context. The NAS COUNTs are not needed at the MME until it sends the first NAS Security Mode Command for that security context. Before the MME sends the first NAS Security Mode Command for a given partial native security context, the MME sets the NAS COUNTs for the security context to 0. After the NAS SMC message is sent for that partial native security context the NAS COUNTs for that partial native context are increased for each following sent NAS message as specified in TS 24.301.

-	or for an EPS NAS security context created through a context mapping during a handover from UTRAN/GERAN to E-UTRAN,

-	or for an EPS NAS security context created through a context mapping during idle mode mobility from UTRAN/GERAN to E-UTRAN.

The NAS COUNTs shall not be reset during idle mode mobility or handover for an already existing native EPS NAS security context.

The start value of NAS COUNT shall be zero (0).

[…]

# 8	Security mechanisms for non-access stratum signalling and data via MME

## 8.0	General

The statements relating to UEs in clause 8 apply also to RNs regarding the security between a relay node and its MME.

Clause 8 also applies to the security procedures for data sent via the MME.

## 8.1	NAS integrity mechanisms

Integrity protection for NAS signalling messages shall be provided as part of the NAS protocol.

### 8.1.1	NAS input parameters and mechanism

Input parameters to the NAS 128-bit integrity algorithms as described in Annex B are an 128-bit integrity key KNASint as KEY, an 5-bit bearer identity BEARER which shall equal the constant value 0x00, the direction of transmission DIRECTION, and a bearer specific, time and direction dependent 32-bit input COUNT which is constructed as follows:

COUNT :=  0x00 || NAS OVERFLOW || NAS SQN

Where

-	the leftmost 8 bits are padding bits including all zeros.

-	NAS OVERFLOW is a 16-bit value which is incremented each time the NAS SQN is incremented from the maximum value.

-	NAS SQN is the 8-bit sequence number carried within each NAS message.

NOTE:	The BEARER identity is not necessary since there is only one NAS signalling connection per pair of MME and UE, but is included as a constant value so that the input parameters for AS and NAS will be the same, which simplifies specification and implementation work.

Editor's Note: If other 3GPP WGs decide that there will be one or more separate pairs of NAS COUNTs for NAS data via MME, cf. Editor's Note in clause 6.5, then there will have to be one separate value of BEARER each per separate pair of NAS COUNTs, and each BEARER will have to be different from the constant 0x00, which would then apply to NAS signalling only. But still only one pair of NAS keys, KNASenc and KNASint , as defined in clause 6.2 would be used. The rest of the confidentiality and integrity procedures in clause 8 and in Annex B is described in such a way that the procedures would not be affected by the introduction of new NAS COUNTs and BEARERs. But at least the first paragraph of 8.1.1 and the NOTE preceding this Editor's Note would have to be modified.

The use and mode of operation of the 128-bit integrity algorithms are specified in Annex B.

The supervision of failed NAS integrity checks shall be performed both in the ME and the MME. In case of failed integrity check (i.e. faulty or missing NAS-MAC) is detected after the start of NAS integrity protection, the concerned message shall be discarded except for some NAS messages specified in TS 24.301 [9]. For those exceptions the MME shall take the actions specified in TS 24.301 [9] when receiving a NAS message with faulty or missing NAS-MAC. Discarding NAS messages can happen on the MME side or on the ME side.

### 8.1.2	NAS integrity activation

NAS integrity shall be activated using the NAS SMC procedure or after a handover to E-UTRAN from UTRAN/GERAN. Replay protection shall be activated when integrity protection is activated (except for when the selected integrity protection algorithm is EIA0, see Annex B). Replay protection shall ensure that the receiver only accepts each particular incoming NAS COUNT value once using the same NAS security context. Once NAS integrity has been activated, NAS messages without integrity protection shall not be accepted by the UE or MME. Before NAS integrity has been activated, NAS messages without integrity protection shall only be accepted by the UE or MME in certain cases where it is not possible to apply integrity protection as specified in TS 24.301 [9]. While some NAS messages such as reject messages need to be accepted by the UE without integrity protection, the MME shall only send a reject message that causes the CSG list on the UE to be modified after the start of NAS security. The UE shall discard any message modifying the CSG list if it is not integrity protected.

NAS integrity stays activated until the EPS security context is deleted in either the UE or MME. In particular the NAS service request shall always be integrity protected and the NAS attach request message shall be integrity protected if the EPS security context is not deleted while UE is in EMM-DEREGISTERED. The length of the NAS-MAC is 32 bit. The full NAS-MAC shall be appended to all integrity protected messages except for the NAS service request. Only the 16 least significant bits of the 32 bit NAS-MAC shall be appended to the NAS service request message.

The use and mode of operation of the 128-EIA algorithms are specified in Annex B.

## 8.2	NAS confidentiality mechanisms

The input parameters for the NAS 128-bit ciphering algorithms shall be the same as the ones used for NAS integrity protection as described in clause 8.1, with the exception that a different key, KNASenc , is used as KEY, and there is an additional input parameter, namely the length of the key stream to be generated by the encryption algorithms.

The use and mode of operation of the 128-bit ciphering algorithms are specified in Annex B.