3GPP TSG-SA5 Meeting #129e 	S5-201154

# e-meeting, 24 February – 4 March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0199 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Serving Network Function ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In deployments topologies with specific SMF Service Areas, a PDU session can be served by SMF + I-SMF in non-roaming: in this case, the "Serving Network Function ID" is the I-SMF. In Home routed scenario, based on TS 23.502 clause 4.23.1 and 4.23.2, a PDU session is served by V-SMF serving specific SMF Service Areas: in this case, the "Serving Network Function ID" is the V-SMF. Per TS 32.255 Table 6.2.1.2.1: Structure of PDU Session Charging Information, the "Serving Network Function ID" needs to hold "SMF" value for: - V-SMF in roaming Home Routed - I-SMF when PDU session is served by SMF + I-SMF. However, the description for ServingNetworkFunctionID attribute allows V-SMF and I-SMF values for nodeFunctionality, which is not aligned TS 32.255. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update servingNetworkFunctionInformation attribute description for alignment with TS 32.255. Remove "I-SMF" enumeration in NodeFunctionality data type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V-SMF change in deployments topologies with specific SMF Service Areas cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2.12, 6.1.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.2.12	Type ServingNetworkFunctionID

Table 6.1.6.2.2.12-1: Definition of type ServingNetworkFunctionID

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servingNetworkFunctionInformation | NFIdentification | M | 0..1 | Serving Network Function information: i.e. AMF, I-SMF, SGW, or V-SMF. For I-SMF and V-SMF, the NFIdentification.nodeFunctionality shall be have the value SMF. |  |
| aMFId | AmfId | OC | 0..1 | AMF identifier |  |

Next change

##### 6.1.6.3.4	Enumeration: NodeFunctionality

Table 6.1.6.3.4-1: Enumeration NodeFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SMF | This field identifies that NF is a SMF. |  |
| AMF | This field identifies that NF is a AMF. |  |
| SMSF | This field identifies that NF service consumer is a SMSF. |  |
| SGW | This field identifies that node is an SGW, only applicable for interworking with EPC. |  |

End of changes