| 3GPP TS 28.105 V1.0.0 (2022-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	 Concepts and overview	8

4.1	Overview	8

5	 AI/ML management functionality and service framework	8

5.1	Functionality and service framework for AI/ML model training	8

6	AI/ML management use cases and requirements	9

6.1	General	9

6.2	AI/ML model training	9

6.2.1	Description	9

6.2.2	Use cases	10

6.2.2.1	AI/ML model training requested by consumer	10

6.2.2.2	AI/ML model training initiated by producer	10

6.2.3	Requirements for AI/ML model training	11

7	Information model definitions for AI/ML management	11

7.1	Imported and associated information entities	11

7.1.1	Imported information entities and local labels	11

7.1.2	Associated information entities and local labels	11

7.2	Class diagram	11

7.2.1	Relationships	11

7.2.2	Inheritance	12

7.3	Class definitions	13

7.3.1	AIMLTrainingRequests	13

7.3.1.1	Definition	13

7.3.1.2	Attributes	13

7.3.1.3	Attribute constraints	13

7.3.1.4	Notifications	13

7.3.2	AIMLTrainingRequest	13

7.3.2.1	Definition	13

7.3.2.2	Attributes	13

7.3.2.3	Attribute constraints	13

7.3.2.4	Notifications	13

7.3.3	AIMLTrainingReports	14

7.3.3.1	Definition	14

7.3.3.2	Attributes	14

7.3.3.3	Attribute constraints	14

7.3.3.4	Notifications	14

7.3.4	AIMLTrainingReport	14

7.3.4.1	Definition	14

7.3.4.2	Attributes	14

7.3.4.3	Attribute constraints	14

7.3.4.4	Notifications	14

7.4	Attribute definitions	15

7.4.1	Attribute properties	15

7.4.2	Constraints	16

7.5	Common notifications	16

7.5.1	Configuration notifications	16

8	Service components	16

8.1	Service components for AI/ML model training MnS	16

Annex X (informative): Change history	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document specifies the Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e.g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see TS 23.288 [3]).

This document also describes the functionality and service framework for AI/ML management.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[4]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SBMA	Service Based Management Architecture

# 4		Concepts and overview

## 4.1	Overview

Artificial Intelligence / Machine Learning (AI/ML) capabilities are used in various domains in 5GS, including management and orchestration (e.g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see 23.288 [3]).

The AI/ML-enabled function in the 5GS uses the AI/ML model for inference.

To enable and facilitate the AI/ML, the AI/ML model and AI/ML-enabled function need to be managed.

The present document specifies the AI/ML management related capabilities and services, which include:

-	AL/ML model training.

# 5		AI/ML management functionality and service framework

## 5.1	Functionality and service framework for AI/ML model training

An entity playing the role of AI/ML Model Training MnS producer, may consume various data for AI/ML model training purpose.

As illustrated in Figure 5.1-1 the AI/ML model training capability is provided via AI/ML Model Training MnS in the context of SBMA to the authorized consumer(s) by AI/ML Model Training MnS producer.

Figure 5.1-1: Functional overview and service framework for AI/ML model training

The internal business logic related to AI/ML model training leverages the current and historical data related to the following to monitor the networks and/or services where are relevant to the AI/ML model, prepare the data for model training, trigger and conduct the model training:

-	Performance Measurements (PM) as per TS 28.552 [3], TS 32.425 [4] and Key Performance Indicators (KPIs) as per TS 28.554 [5].

-	Trace/MDT/RLF/RCEF data, as per TS 32.422 [6] and TS 32.423 [7].

-	QoE and service experience data as per TS 28.405 [8] and TS 28.406 [9].

-	Analytics data offered by NWDAF as per TS 23.288 [3].

-	Alarm information and notifications as per TS 28.532 [10].

-	CM information and notifications.

-	MDA reports from MDA MnS producers as per TS 28.104 [2].

-	Management data from non-3GPP systems.

-	Other data that can be used for training.

# 6	AI/ML management use cases and requirements

## 6.1	General

The use cases and requirements for AI/ML management are specified in the following sub-clauses.

## 6.2	AI/ML model training

### 6.2.1	Description

In operational environment before the AI/ML model is deployed the AI/ML-enabled function (i.e., Inference Function) to conduct inference, it needs to be trained (e.g., by an external entity of the Inference function).

The AI/ML model is trained by the AI/ML Model Training (AIMLMT) MnS producer, and the training can be triggered by the request(s) from one or more AIMLMT MnS consumer(s), or initiated by the AIMLMT MnS producer (e.g., as result of model evaluation).

### 6.2.2	Use cases

#### 6.2.2.1	AI/ML model training requested by consumer

The AI/ML model training capabilities are provided by an AIMLMT MnS producer to one or more consumer(s).

Figure 6.6.2.1-1: AI/ML model training requested by AIMLMT MnS consumer

The AI/ML model training may be triggered by the request(s) from one or more AIMLMT MnS consumer(s). To trigger an AI/ML model training, the AIMLMT MnS consumer requests the AIMLMT MnS producer to train the AI/ML model. In the AI/ML model training request, the consumer should specify the inference type which indicates the function or purpose of the model, e.g., CoverageProblemAnalysis. The AIMLMT MnS producer can perform the model training according to the designated inference type. The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training. To obtain the valid AI/ML model, consumers may also designate their requirements for model performance (e.g., accuracy, etc) in the training request.

The AIMLMT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the AIMLMT MnS producer decides when to start the AI/ML model training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained model, the AIMLMT MnS producer may examine the consumer provided training data and select none, some or all of them. In addition, the AIMLMT MnS producer may select some other training data that are available.

-	trains the AI/ML model using the selected training data , and

-	provides the training result (including the location of the trained model, etc) to the AIMLMT MnS consumer (s).

#### 6.2.2.2	AI/ML model training initiated by producer

The AI/ML model training may be initiated by the AIMLMT MnS producer, for instance as result of evaluation of performance of the AI/ML model, based on the feedback received from the consumer, or when new training data describing the new network status/events are available.

When the AIMLMT MnS producer decides to start the AI/ML model training, the producer

-	selects the training data,

-	trains the AI/ML model using the selected training data, and

-	provides the training result (including the location of the trained model, etc) to the AIMLMT MnS consumer(s) who have subscribed to receive the AI/ML model training results.

### 6.2.3	Requirements for AI/ML model training

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_Training-FUN-1 | The AIMLMT MnS producer shall have a capability allowing the consumer to request AI/ML model training. | AI/ML model training requested by consumer (clause 6.2.2.1) |
| REQ- ML_Training-FUN-2 | The AIMLMT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for AI/ML model training. | AI/ML model training requested by consumer (clause 6.2.2.1) |
| REQ- ML_Training-FUN-3 | The AIMLMT MnS producer shall have a capability allowing the consumer to specify the inference type of the model to be trained. | AI/ML model training requested by consumer (clause 6.2.2.1) |
| REQ- ML_Training-FUN-4 | The AIMLMT MnS producer shall have a capability to provide the training result (including the location of the trained model) to the consumer. | AI/ML model training requested by consumer (clause 6.2.2.1), and AI/ML model training initiated by producer (clause 6.2.2.2) |

# 7	Information model definitions for AI/ML management

## 7.1	Imported and associated information entities

### 7.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [11], IOC, Top | Top |

### 7.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## 7.2	Class diagram

### 7.2.1	Relationships

This clause depicts the set of classes (e.g., IOCs) that encapsulates the information relevant to AI/ML model training. For the UML semantics, see 3GPP TS 32.156 [12].

Figure 7.2.1-1: NRM fragment for AI/ML model training

Editor’s note: the following aspects of the class diagram are to be revisited:

-	whether the collection IOCs (AIMLTrainingRequests, AIMLTrainingReports) are necessary;

-	the most suitable way to represent the collection of IOCs if they are necessary.

-	the cardinality and direction of the relationship between AIMLTrainingRequest and AIMLTrainingReport.

### 7.2.2	Inheritance

Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

## 7.3	Class definitions

### 7.3.1	AIMLTrainingRequests

#### 7.3.1.1	Definition

The IOC AIMLTrainingRequests represents the container of the AIMLTrainingRequest IOC(s).

#### 7.3.1.2	Attributes

No additional attributes other than the ones inherited from the parent class.

#### 7.3.1.3	Attribute constraints

None.

#### 7.3.1.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC, without exceptions or additions.

### 7.3.2	AIMLTrainingRequest

#### 7.3.2.1	Definition

The IOC AIMLTrainingRequest represents the AI/ML model training request that is created by the MnS consumer.

Editor’s note: the behaviour of MnS producer after receiving the AI/ML model training request is TBD.

The AIMLTrainingRequest MOI is contained under one AIMLTrainingRequests MOI.

#### 7.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLModelId | CM | T | T | F | T |
| inferenceType | CM | T | T | F | T |
| candidateTraingDataSource | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

Editor’s note: it is FFS for the attributes allowing consumer to choose the configurations of the AI/ML model.

#### 7.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| aIMLModelId Support Qualifier | Condition: The AI/ML model training request is not for the initial training, as the aIMLModelId is not available before initial AI/ML model is generated. |
| inferenceType Support Qualifier | Condition: The AI/ML model training request is for the initial training, where the aIMLModelId is not available before initial AI/ML model is generated. |

#### 7.3.2.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC, without exceptions or additions.

### 7.3.3	AIMLTrainingReports

#### 7.3.3.1	Definition

The IOC AIMLTrainingReports represents the container of the AIMLTrainingReport IOC(s).

#### 7.3.3.2	Attributes

No additional attributes other than the ones inherited from the parent class.

#### 7.3.3.3	Attribute constraints

None.

#### 7.3.3.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC, without exceptions or additions.

### 7.3.4	AIMLTrainingReport

#### 7.3.4.1	Definition

The IOC AIMLTrainingReport represents the AI/ML model training report that is provided by the MnS producer.

The AIMLTrainingReport MOI is contained under one AIMLTrainingReports MOI.

#### 7.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLModelId | M | T | F | F | T |
| aIMLModelPackageAddress | M | T | F | F | T |
| inferenceType | M | T | F | F | T |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| confidenceIndication | O | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |

#### 7.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the MnS consumer (via AIMLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was not initial training (i.e., the model has been trained before). |

#### 7.3.4.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC, without exceptions or additions.

## 7.4	Attribute definitions

### 7.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLModelId | It identifies the AI/ML model. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aIMLModelPackageAddress | It provides the address where the AI/ML model package is located. The AI/ML model package may contain the AI/ML model (e.g., software image or file) and the model descriptor. The model descriptor may contain more detailed information about the model, such as version, resource requirements, etc. The model descriptor is vendor specific and not specified in the present document. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| candidateTraingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the AI/ML model supports. allowedValues: the values of the MDA type (see TS 28.104 [c]), Analytics ID(s) of NWDAF (see TS 23.288 [d]), RAN-intelligence inference function name(s), and vendor’s specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the AI/ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists the consumer provided training data which have been used for the AI/ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related AIMLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the AIMLTrainingReport MOI that represents the reports for the last training of the AI/ML model. allowedValues: DN. | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the AI/ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 7.4.2	Constraints

| Name | Affected attribute(s) | Definition |
| --- | --- | --- |

## 7.5	Common notifications

### 7.5.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [10], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

# 8	Service components

## 8.1	Service components for AI/ML model training MnS

The components for AI/ML model training MnS are listed in table 8.1-1:

Table 8.1-1: Components for AI/ML model training MnS

| Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- |
| The operations and notifications for generic provisioning management service (see clause 11.1.1 of TS 28.532[10]). | AIMLTrainingRequests IOC; AIMLTrainingRequest IOC; AIMLTrainingReports IOC; AIMLTrainingReport IOC. | None |

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-01 | SA5#141e | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2022-01 | SA5#141e | S5-221616 | - | - | - | Add scope | 0.1.0 |
| 2022-01 | SA5#141e | S5-221617 | - | - | - | Add overview | 0.1.0 |
| 2022-01 | SA5#141e | S5-221618 | - | - | - | Add service framework for AI-ML model training | 0.1.0 |
| 2022-01 | SA5#141e | S5-221335 | - | - | - | Move in ML model training part from TS 28.104 | 0.1.0 |
| 2022-01 | SA5#141e | S5-221614 | - | - | - | Add the requirements for ML model training for MDA | 0.1.0 |
| 2022-02 | SA5#141e | S5-221620 | - | - | - | Add NRMs for AI-ML model training | 0.1.0 |
| 2022-03 | SA#95e | SP-220128 |  |  |  | Presented for information | 1.0.0 |