3GPP TSG-SA4 Meeting #120-e	S4-221109

# Electronic Meeting, Telco,  August 17-26, 2022	revision of S4-220944

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.532 | CR | 0002 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [EVEX] TS 26.532 Bug fixes regarding updating data collection and reporting configurations for data collection clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Existing introductory descriptions in clauses 4.2.4.3, 4.2.5.3 and 4.3.2.3 on updating and renewing data collection and reporting configuration information for/by different types of data collection clients are a bit unclear and inaccurate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to the following clauses of TS 26.532: - Clause 4.2.4.3.1: Modification of description in the 1st paragraph of that clause. - Clause 4.2.4.3.2: Minor typo correction. - Clause 4.2.5.3.1: Modification of description in the 1st paragraph of that clause (in similar manner to above change). - Clause 4.3.2.3.1: Deletion of the 1st paragraph which is identical to/redundant with the 2nd paragraph. Modification of description in the 2nd paragraph of that clause (in similar manner to proposed changes to clauses 4.2.4.3.1 and 4.2.5.3.1). - Clause 4.3.2.3.2: Minor typo correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imprecise/incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4.3.1, 4.2.4.3.2, 4.2.5.3.1 and 4.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.2.4.3	Updating and renewing data collection and reporting configuration

##### 4.2.4.3.1	Introduction

The data collection and reporting configuration may change as a result of subscriptions to events exposed by the Data Collection AF, for example, as a result of narrower or wider filters applied to an event subscription. There are two ways for a modified data collection and reporting configuration to be acquired by the Indirect Data Collection Client:

1.	The Indirect Data Collection Client invokes the Ndcaf_DataReporting_RetrieveSession service operation (see clause 4.2.4.3.2).

2.	The Data Collection AF supplies a DataReportingSession in response to a data report submitted by the Indirect Data Collection Client (see clause 4.2.4.3.3).

##### 4.2.4.3.2	Indirect Data Collection Client retrieves up-to-date configuration

This operation is typically performed when the validUntil property of the current DataReportingSession stored in the Indirect Data Collection Client is about to expire.

Figure 4.2.4.3.2-1: Indirect Data Collection Client retrieves up-to-date DataReportingSession

The steps in this procedure are as follows:

- The Indirect Data Collection Client requests the DataReportingSession for the current session by using the Ndcaf_DataReporting _RetrieveSession service operation (see clauses 7.2.3.2 and 7.2.3.3.1).

- The Data Collection AF provides the latest DataReportingSession in the message body of a 200 OK response. The validUntil property of the provided DataReportingSession should be a time in the future. In addition, the Data Collection AF may change properties reportForDomains and reportingCondition.

NEXT CHANGE

#### 4.2.5.3	Updating and renewing data collection and reporting configuration

##### 4.2.5.3.1	Introduction

The data collection and reporting configuration may change as a result of subscriptions to events exposed by the Data Collection AF, for example, as a result of narrower or wider filters applied to an event subscription. There are two ways for a modified data collection and reporting configuration to be acquired by the Application Server:

1.	The Application Server invokes the Ndcaf_DataReporting_RetrieveSession service operation (see clause 4.2.5.3.2).

2.	The Data Collection AF supplies a DataReportingSession in response to a data report submitted by the Application Server (see clause 4.2.5.3.3).

NEXT CHANGE

##### 4.3.2.3.1	Introduction

The data collection and reporting configuration may change as a result of subscriptions to events exposed by the Data Collection AF, for example, as a result of narrower or wider filters applied to an event subscription. There are two ways the data collection and reporting configuration can be updated or renewed by the Direct Data Collection Client:

1.	The Direct Data Collection Client invokes the Ndcaf_DataReporting_RetrieveSession service operation (see clause 4.3.2.3.2).

2.	The Data Collection AF supplies a DataReportingSession in response to a data report submitted by the Direct Data Collection Client (see clause 4.3.2.3.3).

##### 4.3.2.3.2	Direct Data Collection Client retrieves up-to-date configuration

This operation is typically performed when the validUntil property of the current DataReportingSession stored in the Direct Data Collection Client is about to expire.

Figure 4.3.2.3.2-1: Direct Data Collection Client retrieves up-to-date DataReportingSession

The steps in this procedure are as follows:

1.	The Direct Data Collection Client requests the DataReportingSession for the current session by using the Ndcaf_DataReporting_RetrieveSession service operation (see clauses 7.2.3.2 and 7.2.3.3.1).

2.	The Data Collection AF provides the latest DataReportingSession in the message body of a 200 OK response. The validUntil property of the provided DataReportingSession should be a time in the future. In addition, the Data Collection AF may change properties reportForDomains and reportingCondition.

END OF CHANGES