3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206022

# E-Meeting, 17th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3501 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 Alignment of SBI-based SMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Orange, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SMS_SBI |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to LS (SP-191362 and CP-193301), CT4 took responsibility for the work on service-based support for SMS in 5GS (Rel-17). In order to fulfil SBI-based SMS, CT4 has produced TS 23.540 (stage 2), TS 29.577 (stage 3), TS 29.578 (stage 3) and TS 29.579 (stage 3), and added new services/service operations of UDM/SMSF/NEF to corresponding TSs. In SA/CT#96, it is proposed that TS23.501/TS23.502 needs to keep align with SBI-based SMS in TS23.540, i.e. introduce the new NF functions, services and service operations brought by SBI-based SMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - References update; - Monitoring Events update for SBI-based SMS; - UDM/NEF/SMSF Services update for SBI-based SMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and misaligned specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.15.3.1, 5.2.3, 5.2.6, 5.2.9, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[w]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

# * * * * next change * * * *

#### 4.15.3.1	Monitoring Events

The Monitoring Events feature is intended for monitoring of specific events in 3GPP system and making such monitoring events information reported via the NEF. It is comprised of means that allow NFs in 5GS for configuring the specific events, the event detection and the event reporting to the requested party.

To support monitoring features in roaming scenarios, a roaming agreement needs to be made between the HPLMN and the VPLMN. If the AMF/SMF in the VPLMN determine that normalisation of an event report is required, the AMF/SMF normalises the event report before sending it to the NEF.

The set of capabilities required for monitoring shall be accessible via NEF to NFs in 5GS. Monitoring Events via the UDM, the AMF, the SMF, the NSACF and the GMLC enables NEF to configure a given Monitor Event at UDM, AMF, SMF, NSACF or GMLC and reporting of the event via UDM and/or AMF, SMF, NSACF or GMLC. Depending on the specific monitoring event or information, it is the AMF, GMLC, NSACF or the UDM that is aware of the monitoring event or information and makes it reported via the NEF.

The following table enumerates the monitoring events and their detection criteria:

Table 4.15.3.1-1: List of events for monitoring capability

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Loss of Connectivity | Network detects that the UE is no longer reachable for either signalling or user plane communication (see NOTE 4). The AF may provide a Maximum Detection Time, which indicates the maximum period of time without any communication with the UE after which the AF is to be informed that the UE is considered to be unreachable (see NOTE 7). | AMF |
| UE reachability | Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g. Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE. The AF may provide the following parameters: 1) Maximum Latency; 2) Maximum Response Time; 3) Suggested number of downlink packets. (see NOTE 5 and NOTE 7). This event requires the Reachability Filter set to UE reachable for DL traffic" (see clause 5.2.2.3.1-1). For the usage of this event, see clauses 4.2.5.2 and 4.2.5.3. When requesting UE reachability monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF, UDM |
| Location Reporting | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (one-time reporting, maximum number of reports, maximum duration of reporting, periodicity, etc. as specified in clause 4.15.1). It reports either the Current Location or the Last Known Location of a UE. When AMF is the detecting NF: One-time and Continuous Location Reporting are supported. For Continuous Location Reporting the serving node(s) sends a notification every time it becomes aware of a location change, with the granularity depending on the accepted accuracy of location (see NOTE 1). For One-time Reporting with immediate reporting flag set, AMF reports the Last Known Location immediately. When AMF is the detecting NF: If the immediate reporting flag is not set, the AMF reports the UE Current Location (In case the AMF does not have the UE current location in the granularity as requested by the location report, the AMF retrieves the information via NG-RAN Location reporting procedure as defined in clause 4.10). When GMLC is the detecting NF: Immediate and Deferred Location Reporting is supported. For Deferred Location Reporting the event types UE availability, Area, Periodic Location and Motion are supported. | AMF, GMLC |
| Change of SUPI-PEI association | This event is detected when the association between PEI and subscription (SUPI) changes (USIM change). | UDM |
| Roaming status | This event is detected based on the UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and notification is sent when that status changes. (see NOTE 2). If the UE is registered via both 3GPP and N3GPP Access Type, then both instances of Roaming status are included. | UDM |
| Communication failure | This event is detected when RAN or NAS level failure is detected based on connection release and it identifies RAN/NAS release code. | AMF |
| Availability after Downlink Data Notification failure | This event is detected when the UE becomes reachable again after downlink data delivery failure. When requesting Availability after Downlink Data Notification failure monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF |
| PDU Session Status | This event is detected when PDU session is established or released. (see NOTE 6) | SMF |
| Number of UEs present in a geographical area | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (Level of aggregation, Sampling ratio, see clause 4.15.1). It indicates the number of UEs that are in the geographical area described by the AF. The AF may ask for the UEs that the system knows by its normal operation to be within the area (Last Known Location) or the AF may request the system to also actively look for the UEs within the area (Current Location). | AMF |
| CN Type change | The event is detected when the UE moves between EPC and 5GC. It indicates the current CN type for a UE or a group of UEs when detecting that the UE switches between being served by a MME and an AMF or when accepting the event subscription. (see NOTE 3) | UDM |
| Downlink data delivery status | It indicates the downlink data delivery status in the core network. Events are reported at the first occurrence of packets being buffered, transmitted or discarded, including: - Downlink data in extended buffering, including: - First data packet buffered event - Estimated buffering time, as per clause 4.2.3.3 - First downlink data transmitted event - First downlink data discarded event | SMF |
| UE reachability for SMS delivery | This event is detected when an SMSF is registered for a UE and the UE is reachable as determined by the AMF and the UDM. This enables the UE to receive an SMS. See clauses 4.2.5.2 and 4.2.5.3. | UDM |
| UE memory available for SMS | This event is detected when the UE memory is available for SMS as determined by the AMF and the UDM. This enables to trigger Alert for SMS. See clauses 5.1.8 of TS 23.540 [w] | UDM |
| Number of registered UEs or established PDU Sessions | It indicates the current number of registered UEs or established PDU Sessions for a network slice that is subject to NSAC. For One-time Reporting with Immediate Reporting Flag set, NSACF reports the number of registered UEs or established PDU Sessions immediately. | NSACF |
| Area Of Interest | It indicates change of the UE presence in the Area Of Interest. | AMF, GMLC |
| NOTE 1: Location granularity for event request, or event report, or both could be at cell level (Cell ID) or TA level. The granularity can also be expressed by other formats such as geodetic uncertainty shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) which can be mapped by NEF to AMF specific granularity levels. NOTE 2: Roaming status means whether the UE is in HPLMN or VPLMN based on the most recently received registration state in the UDM. NOTE 3: CN type of CN Type change event is defined in clause 5.17.5.1 of TS 23.501 [2]. NOTE 4: In the case of UDM service operation information flow, the UDM should set the subscribed periodic registration timer to a smaller value than the value of Maximum Detection Time, since the value of the mobile reachable timer is larger than the value of the periodic registration timer. NOTE 5: Maximum Latency, Maximum Response Time and Suggested number of downlink packets are defined in clause 4.15.6.3a. NOTE 6: The NEF makes a mapping between the 5GS internal event "PDU Session Status" and the T8 API event "PDN Connectivity Status". NOTE 7: The preferred method for provisioning Network Configuration Parameters is External Parameter Provisioning specified in clause 4.15.6.3a. Provisioning event specific parameters as part of Monitoring Request is expected to be used only by the AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a. |  |  |

# * * * * next change * * * *

### 5.2.3	UDM Services

#### 5.2.3.1	General

The following table illustrates the UDM Services and Service Operations.

Table 5.2.3.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data | Get | Request/Response | AMF, SMF, SMSF, NEF, 5G DDNMF, TSCTSF |
| Management | Subscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF |
| (SDM) | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, NEF, GMLC, 5G DDNMF |
|  | ModifySubscription | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF |
|  | Info | Request/Response | AMF, NEF |
| UE Context | Registration | Request/Response | AMF, SMF, SMSF |
| Management | DeregistrationNotification | Subscribe/Notify | AMF |
| (UECM) | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF, SMSF, GMLC, NWDAF, SMF |
|  | Update | Request/Response | AMF, SMF |
|  | PCscfRestoration | Subscribe/Notify | AMF, SMF |
|  | SendRoutingInfoForSM | Request/Response | SMS-GMSC |
| UE | Get | Request/Response | AUSF |
| Authentication | ResultConfirmation | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF (NOTE), NWDAF, SMS-GMSC |
|  | Unsubscribe |  | NEF (NOTE), NWDAF, SMS-GMSC |
|  | Notify |  | NEF (NOTE), NWDAF, SMS-GMSC |
|  | ModifySubscription |  | NEF (NOTE), NWDAF, SMS-GMSC |
| Parameter | Update | Request/Response | NEF, AMF |
| Provision | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Get | Request/Response | NEF |
| NIDDAuthorisation | Get | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| ServiceSpecificAuthorisation | Create | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| ReportSMDeliveryStatus | Request | Request/Response | SMS-GMSC |
| NOTE: Other NFs are allowed to consume the service based on roaming agreement or operator policy. |  |  |  |

# * * * * next change (new) * * * *

#### 5.2.3.2	Nudm_UECM (UECM) service

##### 5.2.3.2.7	Nudm_UECM_SendRoutingInfoForSM service operation

Service operation name: Nudm_UECM_SendRoutingInfoForSM.

Description: This service operation is used to get SMS Routing Info for MT SMS.

Inputs, Required: GPSI.

Inputs, Optional: None.

Outputs, Required: Result indication, SMS Routing Info.

Outputs, Optional: None.

# * * * * next change * * * *

#### 5.2.3.5	Nudm_EventExposure service

##### 5.2.3.5.1	General

See clause 4.15.3.1.

##### 5.2.3.5.2	Nudm_EventExposure_Subscribe service operation

Service operation name: Nudm_EventExposure_Subscribe

Description: The NF consumer subscribes to receive an event.

NF Consumers: NEF, SMS-GMSC.

Inputs, Required: Target of Event Reporting: UE(s) ID (SUPI or GPSI, Internal Group Identifier or External Group Identifier, or indication that any UE is targeted), Event filter containing the Event Id(s) (see clause 4.15.3.1) and Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Expiry time, DNN, S-NSSAI, traffic descriptor identifying the source of the downlink IP or Ethernet traffic (for Availability after DDN Failure and downlink data delivery status events), MTC Provider Information, list of group member UE(s) whose subscription to event notification(s) are removed or added for a group-based event notification subscription, operation indication (cancellation or addition), Idle Status Indication request (if UE reachability or Availability after DDN failure reporting is requested).

For configuration of monitoring events applicable to both EPC and 5GC, a combined SCEF+NEF indicates that the monitoring event is also applicable to EPC (i.e. the event must be reported both by 5GC and EPC) and may include a SCEF address (i.e. if the event needs to be configured in a serving node in the EPC and the corresponding notification needs to be sent directly to the SCEF).

Outputs, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if corresponding information is available (see clause 4.15.1), Number of UE if the External Group Identifier and Maximum Number of Reports are included in the inputs.

Number of UEs indicates the number of UEs within the group identified by the External Group Identifier. The NEF uses this value to determine whether the monitoring event has been reported for all group member UEs.

##### 5.2.3.5.3	Nudm_EventExposure_Unsubscribe service operation

Service operation name: Nudm_EventExposure_Unsubscribe

Description: the consumer deletes the subscription of an event if already defined in UDM.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

##### 5.2.3.5.4	Nudm_EventExposure_Notify service operation

Service operation name: Nudm_EventExposure_Notify

Description: UDM reports the event to the consumer that has previously subscribed.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event specific parameters list, Event Removal Indication, list of group member UE(s) whose subscription to event notification(s) are removed from a group-based event notification subscription.

Outputs, Required: None.

##### 5.2.3.5.5	Nudm_EventExposure_ModifySubscription service operation

Service operation name: Nudm_EventExposure_ModifySubscription

Description: The NF consumer requests to modify an existing subscription to event notifications.

NF Consumers: NEF, SMS-GMSC.

Inputs, Required: UE(s) ID (SUPI or GPSI, Internal Group Identifier or External Group Identifier, or any UE), Subscription Correlation ID, Modifications to the Event Subscription.

Inputs, Optional:

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * next change (new) * * * *

#### 5.2.3.9	Nudm_ReportSMDeliveryStatus service

##### 5.2.3.9.1	General

Service description: This service is to report the SM-Delivery Status to UDM.

##### 5.2.3.9.2	Nudm_ReportSMDeliveryStatus_Request service operation

Service operation name: Nudm_ReportSMDeliveryStatus_Request

Description: Reports the SM-Delivery Status to UDM.

Inputs, Required: GPSI, SM-Delivery status.

Inputs, Optional: None.

Outputs, Required: Report SM-Delivery status result.

Outputs, Optional: None.

# * * * * next change * * * *

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |

# * * * * next change (new) * * * *

#### 5.2.6.29	Nnef_SMService service

##### 5.2.6.29.1	General

Service description: This service can be used for SBI-based MO SM transmit through NEF for MSISDN-less MO SMS.

##### 5.2.6.29.2	Nnef_SMService_MoForwardSm service operation

Service operation name: Nnef_SMService_MoForwardSm.

Description: Transmit MO SMS message from consumer NF to NEF.

Inputs, Required: SMS payload, Application port ID, SUPI, destination SME address (long/short code of the AF).

Inputs, Optional: None.

Outputs, Required: SMS message transmission result.

Outputs, Optional: None.

# * * * * next change * * * *

### 5.2.9	SMSF Services

#### 5.2.9.1	General

The following table illustrates the SMSF Services.

Table 5.2.9.1-1: List of SMSF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmsf_SMService | Activate | Request/Response | AMF |
|  | Deactivate | Request/Response | AMF |
|  | UplinkSMS | Request/Response | AMF |
|  | MtForwardSm | Request/Response | SMS-GMSC, IP-SM-GW, SMS Router |

#### 5.2.9.2	Nsmsf_SMService service

##### 5.2.9.2.1	General

This service allows AMF to authorize SMS and activate SMS for the served user on SMSF. This service allows downlink SMS message transmit from consumer NF to SMSF

##### 5.2.9.2.2	Nsmsf_SMService_Activate service operation

Service operation name: Nsmsf_SMService_Activate.

Description: Authorize whether the specified UE is allowed to activate SMS service, or add connectivity for SMS over new Access Type.

Concurrent use: None.

Inputs, Required: SUPI, NF ID, RAT Type.

Inputs, Optional: GPSI, Time Zone, one or more Access Type(s), GUAMI, backup AMF(s) (if NF Type is AMF). Backup AMF(s) sent only once by the AMF to the SMSF in its first interaction with the SMSF, UE's Routing Indicator optionally with Home Network Public Key identifier or UDM Group ID for the UE.

Outputs, Required: SMS service activation result.

Outputs, Optional: None.

##### 5.2.9.2.3	Nsmsf_SMService_Deactivate service operation

Service operation name: Nsmsf_SMService_Deactivate.

Description: Remove SMS service authorization from SMSF for a given service user, or with Access Type included, remove connectivity for SMS over the affected Access Type.

Concurrent use: None.

Inputs, Required: SUPI.

Inputs, Optional: Access Type.

Outputs, Required: SMS service deactivation result.

Outputs, Optional: None.

##### 5.2.9.2.4	Nsmsf_SMService_UplinkSMS service operation

Service operation name: Nsmsf_SMService_UplinkSMS.

Description: transmit uplink SMS message from consumer NF to SMSF.

Concurrent use: None.

Inputs, Required: SUPI, SMS payload.

Inputs, Optional: GPSI, UE Time Zone, UE Location Information (ULI).

Outputs, Required: SMS message transmission result.

Outputs, Optional: None.

##### 5.2.9.2.5	Nsmsf_SMService_MtForwardSm service operation

Service operation name: Nsmsf_SMService_MtForwardSm.

Description: Transmit downlink SMS message from consumer NF to SMSF.

Inputs, Required: GPSI, SMS payload.

Inputs, Optional: None.

Outputs, Required: SMS message transmission result.

Outputs, Optional: None.

# * * * * end of change * * * *