3GPP TSG-SA WG6 Meeting #50-e	S6-222019

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0054 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Priority Type IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Priority Type IE is included by the message sender and is intended for the message receiver. The Priority Type IE is not used by the MSGin5G Service with the exception of a Priority Type with value “High”. If the value is “High” then the MSGin5G Service shall not wait for possible message aggregation. Note that the meaning and the values of the Priority Type IE is left for implementation, but at least the values “High”, “Medium”, and “Low shall be present. In the CR it is assumed that “left for implementation” means left for implementation by the MSGin5G Service user, not necessarily only the MSGin5G Service provider and therefore the Priority Type IE needs to be delivered to the message receiver. Delivery of the Priority Type IE to the receiver is included in some procedures, but is missing in some other procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the Priority Type IE in all procedures where MSGin5G messages are delivered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Indicating the priority has restricted and unpredictable (by the service user) usefulness and may lead to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3, 8.11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * *

### 8.3.3	MSGin5G outbound messages from the MSGin5G Server

Figure 8.3.3-1 shows the procedure for the MSGin5G Server that forwards an MSGin5G message.

Figure 8.3.3-1: MSGin5G message towards UE

Figure 8.3.3-2 shows the same procedure (step 1 only), however for the MSGin5G Server that sends the message to an Application Server by application request.

Figure 8.3.3-2: message towards an Application Server

Figure 8.3.3-3 shows the procedure for the MSGin5G Server that sends an MSGin5G message to a Legacy 3GPP Message Gateway, a Non-3GPP Message Gateway, or a Broadcast Message Gateway.

Figure 8.3.3-3: MSGin5G message towards a Message Gateway

The following procedure applies to the above figures 8.3.3-1, 8.3.3-2 and 8.3.3-3 with the exception that step 2 only applies to figure 8.3.3-1.

1.	The MSGin5G Server sends the MSGin5G message request and includes the IEs as listed in table 8.3.3-1.

Table 8.3.3-1: MSGin5G message request from MSGin5G Server

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the originating MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the originating Application Server. This IE is copied from the associated inbound message. |
| Recipient UE Service ID/AS Service ID (see NOTE1, NOTE 2) | O | The service identity of the receiving entity. This IE is present in case the recipient is an individual UE or Application Server For Point-to-Point messaging, Application-to-Point messaging/Point-to-Application messaging, this IE is copied from the associated inbound message. For Group messaging, this IE can be a Recipient UE Service ID only. This IE is fetched from the participant information of the recipient in the group profile. For message delivery based on Messaging Topic subscription, this IE is the UE Service ID/AS Service ID of the Messaging Topic subscriber. |
| Broadcast Area ID (see NOTE 1) | O | The identifier of the Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. This IE is copied from the associated inbound message. |
| Application ID | O | Identifies the application for which the payload is intended. This list of Application ID(s) IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application ID(s) IE may be included when the message is sent to an Application Server or to an Application Client. This list of IEs is copied from the associated inbound message. |
| Message ID | M | Unique identifier of this message. This IE is copied from the associated inbound message request. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is copied from the associated inbound message. |
| Payload | O | Payload of the message. This IE is copied from the associated inbound message. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Group Service ID | O | The service identifier of a Group. This IE is included if message delivery is based on Group messaging. This IE is copied from the Recipient Group Service ID IE in the associated inbound message. |
| Messaging Topic | O | Indicates which Messaging Topic this message is related to. This IE is included if message delivery is based on a Messaging Topic subscription. This IE is copied from the Messaging Topic IE in the associated inbound message. |
| Segmentation Set Identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages. |
| Last Segment Flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| Priority type | O | Application priority level requested by the message originator for this message. |
| NOTE 1: Only one of these IEs shall be included to represent the type of message request. The MSGin5G client may construct the IEs specified in table 8.11.5-1 based on the IEs above in the received MSGin5G message, e.g. removes the MSGin5G service domain, if the Payload is sent to Application Client. NOTE 2: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. |  |  |

If the received MSGin5G message request is for Group Message, the MSGin5G Server shall, in addition to the Recipient Group Service ID IE, add the Recipient UE Service ID of each individual group member, excluding the message originator.

If the received MSGin5G message request is for a Topic Message, the MSGin5G Server shall, in addition to the Messaging Topic IE, add the Recipient UE Service ID/AS Service ID of each individual Topic subscriber, excluding the message originator.

* * Next Change * * *

### 8.11.4	Constrained device sending message using Gateway UE

The signalling flow for the Application Client-2 on the UE-2 (which is a Constrained UE) to send message using gateway UE functionality on MSGin5G UE-1 is illustrated in figure 8.11.4-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The UE-2 is Constrained UE and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.4-1: Application Client-2 on UE-2 sends message using gateway UE functionality on MSGin5G UE-1

1)	An Application Client-2 on the UE-2 sends a request to send MSGin5G message to the MSGin5G Client-1. The information elements defined in Table 8.11.4-1 are included in the message.

Table 8.11.4-1: Information elements for Request to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Recipient address | O | Indicates details of the recipient. This IE is mandatory to initiate Point-to-Point messaging and Point-to-Application messaging. |
| Recipient Group address | O | Indicates target group. This IE is mandatory to initiate Group messaging. |
| Application ID | O | Identifies the application(s) for which the payload is intended. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |
| Priority type | O | Application priority level requested for this message as specified in Table 8.3.2-1. |

Editor's note:	If Priority type needs to be communicated from application clients is FFS.

2)	Upon receiving the request from the Application Client-2 in UE-2, the MSGin5G Client-1  constructs the related IEs specified in table 8.3.2-1 and sends Point-to-Point Message, Point-to-Application Message or Group Message based on these Ies.

a)	if the size of the received message exceeds the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.5; and

b)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.7.

NOTE 1:	The MSGin5G Client-1 may also reject the request to send MSGin5G message based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client-1 sends response to send MSGin5G message to Application Client-2 on UE-2. The response message includes information elements as specified in Table 8.11.4-2.

Table 8.11.4-2: Information elements for Response to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If the MSGin5G Client-1 has decided to reject the request to send the message or the MSGin5G Client-1 received reject response from MSGin5G Server in step 2, the MSGin5G Client-1 sends failure response to the Application Client-2 and stops performing further steps.

4)	If delivery status is requested while sending the message in step 1, the MSGin5G Client-1 may receive MSGin5G message delivery status report from the MSGin5G Server.

5)	Upon receiving the MSGin5G message delivery status report, the MSGin5G Client-1 sends the message delivery status report to the Application Client-2 on UE-2. The message delivery status report includes information elements as specified in Table 8.11.4-3.

Table 8.11.4-3: Information elements for MSGin5G message delivery status

| Information element | Status | Description |
| --- | --- | --- |
| Delivery status | M | Indicates delivery status |

* * End of Changes * * *