3GPP TSG-SA WG6 Meeting #50-e 	S6-222440

# e-meeting, 22nd – 31st August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0058 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to clauses 8.2.7-8.7.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2.7.1: - It is not allowed per drafting rules to include normative words in a note. Several clauses contain a pre-condition in which a non-MSGin5G UE has registered with the MSGin5G Server, but registration of a non-MSGin5G UE is out of scope. Clause 8.7.1.3: Step 1 in figure 8.7.1.3-1 includes the Application Client but the description of step 1 does not mention what the Application Client does. Clause 8.7.1.3: Including the Delivery Status Report Requested is optional in steps 1a and 5, but mandatory in step 4. Clauses 8.7.1.4, 8.7.1.5, 8.7.3.2, and 8.7.3.3 specify a procedure in which a non-MSGin5G UE initiates sending a message as a reply to an earlier received message. If a non-MSGin5G UE sends a message to a GW then this message does not contain any recipient ID other than the GW ID; hence the GW does not know to which recipient to forward the message. The procedure assumes that the message has to be a reply to a previously sent message. However, this necessarily assumed pre-condition is not included in any of the procedures. Furthermore, in the procedure the MSGin5G Server verifies that the UE is allowed to send a reply message, but the Server cannot determine that a message is a reply to an earlier message. Clause 8.7.1.4 and 8.7.3.1: readability issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.2.7.1: - Change “may” into “can” and removal of the words that refer to a configuration procedure in a clause on a registration procedure. - Modify pre-conditions to state that the GW registers on behalf of non-MSGin5G UEs. Clause 8.7.1.3: Step 1 includes the triggering action for the MSGin5G Client to send the message. Clause 8.7.1.3: The mandatory Delivery Status Report Requested is removed from step 4. Clauses 8.7.1.4, 8.7.1.5, 8.7.3.2, and 8.7.3.3: Include the pre-condition that the message is a reply to an earlier sent message and remove the pre-condition in which the GW allocates a routable ID to an entity that already has a Service ID and does not communicate this anywhere. Furthermore, “reply a message” is changed to “send a message”. Clause 8.7.1.4 and 8.7.3.1: rewording to improve readability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification, possibly leading to wrong implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.7.1.3, 8.7.1.4, 8.7.1.5, 8.7.2.2, 8.7.2.3, 8.7.3.2, 8.7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * Changes * * *

## 8.2.7	MSGin5G UE bulk registration over MSGin5G-6 reference point

When constrained devices UE-2a and UE-2b which support an MSGin5G Client perform registration to use MSGin5G UE-1 to contact with the MSGin5G Server, the MSGin5G UE-1 may decide to use bulk registration procedure specified in this clause based on the registration request sent from MSGin5G Clients in MSGin5G UE-2a and MSGin5G UE-2b if allowed by service policy. The procedure for MSGin5G UE bulk registration is illustrated in figure 8.2.7-1.

Pre-conditions:

1.	The MSGin5G UE-1 has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-2a and MSGin5G UE-2b have discovered and selected MSGin5G UE-1 as the MSGin5G Proxy UE and connected to the serving network via MSGin5G UE-1 successfully.

Editor's note:	how constrained device decides whether to use MSGin5G Relay UE service or MSGin5G proxy UE service and how to discover them is FFS.

3.	UE Service IDs and the MSGin5G Server addresses have been configured on the MSGin5G UE-1, MSGin5G UE-2a and MSGin5G UE-2b. All these MSGin5G UEs are served by a same MSGin5G Server.

4.	Both the MSGin5G UEs and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.7-1: MSGin5G Client bulk registration

1.	The MSGin5G Client-2a sends an MSGin5G UE registration request to the MSGin5G Server as specified in clause 8.2.1, In addition to the Information Elements specified in table 8.2.1-1, the MSGin5G Client-2a may also add the Information Elements specified in table 8.2.7-1 in the MSGin5G UE registration request.

Table 8.2.7-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration urgent degree | O | Indicates whether the registration request is urgent or not. If a MSGin5G UE registration request is indicated as urgent, the MSGin5G UE registration request can not be bulked handled. |
| Registration request expiration time | O | Indicates the maximum processing time of the registration request allowed by the MSGin5G Client-2. |

2.	Upon receiving the MSGin5G UE registration request, the MSGin5G Client-1 checks whether this request can be bulked based on the service policy and the information included in the MSGin5G UE registration request, e.g.

a)	whether the MSGin5G UE registration request is urgent; or

b)	whether bulk registration is supported by the MSGin5G service provider;

NOTE 1:	Whether bulk registration is supported can be configured to the MSGin5G Client-1.

NOTE 2:	The bulk registration conditions are specified by application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE registration request can not be bulked, MSGin5G Client-1 sends the  registration request to MSGin5G Server; or

b)	if the MSGin5G UE registration request can be bulked, MSGin5G Client-1 sends a bulk registration notification to MSGin5G Client-2 and caches/stores the MSGin5G UE registration request until the bulk registration conditions can be fulfilled. The Information Elements specified in table 8.2.7-2 are included in the bulk registration notification.The step 4 and 5 are skipped.

Table 8.2.7-2: bulk registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE-2. |
| Expected registration time | O | The expected time when the bulk registration can be handled and the MSGin5G UE registration Response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE registration request as specified in clause 8.2.1 and sends the corresponding MSGin5G UE registration response to MSGin5G Client-2a.

5.	The MSGin5G Client-1 sends the MSGin5G UE registration response to MSGin5G Client-2a. The MSGin5G Client-2a handles the MSGin5G UE registration response as specified in clause 8.2.1 and skips all the remaining steps in this clause.

6.	MSGin5G Client-2b sends an MSGin5G UE registration request and the MSGin5G UE registration request is handed as in steps 2-5.

7.	The MSGin5G Client-1 finds that the conditions of sending a bulk registration request can be fulfilled, e.g.

a)	the Registration request expiration time in the MSGin5G UE registration request sent from MSGin5G Client-2a or MSGin5G Client-2b is reached;

b)	the periodic bulk registration interval is reached;or

c)	the maximum MSGin5G UE registration request number in a MSGin5G UE bulk registration request is reached.

NOTE 3:	The maximum registration time, periodic bulk registration interval and maximum MSGin5G UE registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Client-1 includes all cached/stored MSGin5G UE registration requests in an MSGin5G UE bulk registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.7-3 are included in the MSGin5G UE bulk registration request.

Table 8.2.7-3: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration indicator | M | Indicates that this request is a MSGin5G UE bulk registration request. |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of MSGin5G UE registration requests which are bulked in this MSGin5G UE bulk registration request |
| List of individual MSGin5G UE registration request | M | Each element in this list contains information as specified in Table 8.2.1-1. |

9.	Upon receiving the MSGin5G UE bulk registration request, the MSGin5G Server splits the MSGin5G UE bulk registration request, executes the authentication, authorization and registration procedures for each MSGin5G Client-2 individually as specified in clause 8.2.1.

Editor's note:	Security aspects of bulk registration depends on SA3 and is FFS

10.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in an MSGin5G UE bulk registration response and sends it to MSGin5G Client-1. The Information Elements specified in table 8.2.7-4 are included in the MSGin5G UE bulk registration response.

Table 8.2.7-4: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration response indicator | M | Indicates that this response is a MSGin5G UE bulk registration response. |
| Number of individual MSGin5G UE registration responses | M | Indicates total number of MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |

11.	Upon receiving the MSGin5G UE bulk registration response, the MSGin5G Client-1 splits the MSGin5G UE bulk registration response into multiple individual MSGin5G UE registration response, each individual MSGin5G UE registration response contains information as specified in Table 8.2.1-4.

12.	MSGin5G Client-1 sends the MSGin5G UE registration Response 2a to MSGin5G Client-2a.

13.	MSGin5G Client-1 sends the MSGin5G UE registration Response 2b to MSGin5G Client-2b.

* * Next Change * * *

#### 8.7.1.3	From MSGin5G UE to Non-3GPP UE

Figure 8.7.1.3-1 shows the message delivery procedure from MSGin5G UE to Non-3GPP UE.

Pre-conditions:

1.	MSGin5G Client in MSGin5G UE has registered with the MSGin5G Server and the Non-3GPP Message Client in Non-3GPP UE has registered with the MSGin5G Server via the Non-3GPP Message Gateway.

2.	Non-3GPP Message Gateway is aware of the non-3GPP message client in Non-3GPP UE and provides the mapping to UE Service ID.

Figure 8.7.1.3-1 Message Delivery from MSGin5G UE to Non-3GPP UE

1.	The Application Client in the MSGin5G UE triggers the MSGin5G Client to send an MSGin5G message request to the MSGin5G Server as specified in 8.3.2 with following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient UE Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

2.	The MSGin5G Server determines that the MSGin5G Client is authorized to send message to the recipient UE.

3.	The MSGin5G Server forwards the MSGin5G message request to the recipient based on the UE Service ID. The Non-3GPP Message Gateway receives the MSGin5G message request on behalf of the Non-3GPP UE as specified in clause 8.6.1.

4.	The Non-3GPP Message Gateway translates the MSGin5G message to the Non-3GPP message with message delivery status report requested, if appropriate, and sends it to the Non-3GPP Message Client. This step is outside the scope of the current specification.

5.	If message delivery status report is requested, the Non-3GPP Message Gateway sends the MSGin5G message delivery status report to the MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server forwards the MSGin5G message delivery status report to the MSGin5G Client as specified in clause 8.3.5.

#### 8.7.1.4	From Legacy 3GPP UE to MSGin5G UE

This procedure is used for message reply from Legacy 3GPP UE (e.g. SMS UE) to MSGin5G UE.

Figure 8.7.1.4-1 shows the message delivery procedure from Legacy 3GPP UE (e.g. SMS UE) to MSGin5G UE.

Pre-conditions:

1.	MSGin5G Client in MSGin5G UE has registered with the MSGin5G Server and the Legacy 3GPP Message Client in Legacy 3GPP UE has registered with the MSGin5G Server via the Legacy 3GPP Message Gateway.

2.	The Legacy 3GPP UE received a message from the MSGin5G UE.

3.	The Legacy 3GPP Message Gateway is aware of the Legacy 3GPP UE and provides the mapping to UE Service ID.

4.	The Legacy 3GPP UE replies to the MSGin5G UE upon receiving the message from the MSGin5G UE.

5.	The Legacy 3GPP Message Gateway implementation supports storing a messaging transaction, i.e. mapping the message originating MSGin5G Service ID and the message delivered to the Legacy 3GPP UE, for an operator configured time period to allow if the Legacy 3GPP UE will send a response to the incoming message.

Figure 8.7.1.4-1: Legacy 3GPPs UE replies to MSGin5G UE

1.	The Legacy 3GPP UE sends a Legacy 3GPPmessage request to the Legacy 3GPP Message Gateway  (e.g. through SMSC if SMS is used according the procedure in 3GPP TS 23.204 [13] or the procedure in clause 4.13.3 of TS 23.502 [7]).

2.	The Legacy 3GPP Message Gateway translates the SMS message to MSGin5G message and sends an MSGin5G message request to the MSGin5G Server as specified in 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient UE Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines that the Legacy 3GPP UE with its UE Service ID is allowed to send a message to the MSGin5G UE.

3.	The MSGin5G Server forwards the MSGin5G message request to the target MSGin5G Client as specified in 8.3.3.

4-6.	If the message delivery status report is requested, the MSGin5G Client in MSGin5G UE sends an MSGin5G message delivery status report to the MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server sends the MSGin5G message delivery status report to the Legacy 3GPP Message Gateway specified in clause 8.3.5, the Legacy 3GPP Message Gateway translates the MSGin5G message delivery status report to a Legacy 3GPP message delivery status report and sends it to the Legacy 3GPP UE.

#### 8.7.1.5	From Non-3GPP UE to MSGin5G UE

This procedure is used for message reply from Non-3GPP UE to MSGin5G UE.

Figure 8.7.1.5-1 shows the message delivery procedure from Non-3GPP UE to MSGin5G UE.

Pre-conditions:

1.	MSGin5G Client in MSGin5G UE has registered with the MSGin5G Server and the Non-3GPP Message Client in Non-3GPP UE has registered with the MSGin5G Server via the Non-3GPP Message Gateway.

2.	The Non-3GPP UE received a message from the MSGin5G UE.

3.	The Non-3GPP Message Gateway is aware of the Non-3GPP message client on the Non-3GPP UE and provides the mapping to UE Service ID.

4.	The Non-3GPP UE replies to the MSGin5G UE upon receiving the message from the MSGin5G UE.

5.	The Non-3GPP Message Gateway implementation supports storing a messaging transaction, i.e. mapping the message originating MSGin5G Service ID and the message delivered to the Non-3GPP UE, for an operator configured time period to allow if the Non-3GPP UE will send a response to the incoming message.

Figure 8.7.1.5-1: Non-3GPP UE replies to MSGin5G UE

1.	The Non-3GPP UE sends a Non-3GPP message request to the Non-3GPP Message Gateway.

2.	The Non-3GPP Message Gateway translates the Non-3GPP message to MSGin5G message with message delivery status report requested and sends an MSGin5G message request to the MSGin5G Server as specified in clause 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient UE Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines that the Non-3GPP UE with its UE Service ID is allowed to send a message to the MSGin5G UE.

3.	The MSGin5G Server forwards the MSGin5G message request to the target MSGin5G Client as specified in clause 8.3.3.

4.	If the message delivery status report is request by the Non-3GPP UE, the MSGin5G Client in MSGin5G UE sends an MSGin5G message delivery status report to the MSGin5G Server specified in clause 8.3.4, the MSGin5G Server sends the MSGin5G message delivery status report to the Non-3GPP Message Gateway specified in clause 8.3.5, the Non-3GPP Message Gateway translates the MSGin5G message delivery status report to a Non-3GPP message delivery status report and sends it to the Non-3GPP UE.

* * Next Change * * *

#### 8.7.2.2	From Application Server to Legacy 3GPP UE

Figure 8.7.2.2-1 shows the message delivery procedure from Application Server to Legacy 3GPP UE.

Pre-conditions:

1.	The Application server has established a secured communication with the MSGin5G Server

2.	The Legacy 3GPP Message Gateway has registered with the MSGin5G Server on behalf of the client in the Legacy 3GPP UE.

3.	Legacy 3GPP Message Gateway is aware of the legacy 3GPP message client (i.e. SMS client) in Legacy 3GPP UE and provides the mapping to UE Service ID.

Figure 8.7.2.2-1Application Server to Legacy 3GPP UE messaging

1.	The Application Server sends an API Request to MSGin5G Server for sending MSGin5G message as specified in clause 8.3.2 with the following clarifications:

a)	The API Request includes Originating AS Service ID, Recipient UE Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

2.	Upon receiving the API Request for MSGin5G message delivery, the MSGin5G Server determines that the Application Server is allowed to send message to the recipient UE

3.	The MSGin5G Server sends MSGin5G message request to the recipient based on the UE Service ID. The Legacy 3GPP Gateway receives the MSGin5G message request on behalf of the Legacy 3GPP UE as specified in clause 8.3.3.

4-7. Same with step 4-7 in clause 8.7.1.2

8.	The Legacy 3GPP Message Gateway sends the MSGin5G message delivery status report to the MSGin5G Server as specified in clause 8.3.4.

#### 8.7.2.3	From Application Server to Non-3GPP UE

Figure 8.7.2.3-1 shows the message delivery procedure from Application Server to Non-3GPP UE.

Pre-conditions:

1.	The Application Server has established a secured communication with the MSGin5G Server

2.	The Non-3GPP Message Gateway has registered with the MSGin5G Server on behalf of the message client in the Non-3GPP UE.

3.	Non-3GPP Message Gateway is aware of the non-3GPP message client in Non-3GPP UE and provides the mapping to UE Service ID.

Figure 8.7.2.3-1 Application Server to Non-3GPP UE messaging

1.	The Application Serve sends an API Request to MSGin5G Server for sending MSGin5G message as specified in 8.3.2 with the following clarifications:

a)	The API Request includes Originating AS Service ID, Recipient UE Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

2.	Upon receiving the API Request for MSGin5G message delivery, the MSGin5G Server determines that the Application Server is allowed to send message to the Non-3GPP UE.

3.	The MSGin5G Server sends MSGin5G message request to the recipient based on the UE Service ID. The Non-3GPP Message Gateway receives the MSGin5G message request on behalf of the Non-3GPP UE as specified in clause 8.3.3.

4.	The Non-3GPP Message Gateway translates the MSGin5G message to the Non-3GPP message and sends it to the Non-3GPP UE. This step is outside the scope of the current specification.

5-6.	If message delivery status report is required, the Non-3GPP Message Gateway sends the MSGin5G message delivery status report to the MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server sends the message delivery status report to the Application Server as specified in clause 8.3.5.

* * Next Change * * *

#### 8.7.3.1	From MSGin5G UE to Application Server

Figure 8.7.3.1-1 shows the message delivery procedure from MSGin5G UE to Application Server.

Pre-conditions:

1.	The Application Server and MSGin5G Client in MSGin5G UE have registered with the MSGin5G Server.

Figure 8.7.3.1-1: Message delivery from MSGin5G UE to Application Server

1.	The MSGin5G Client sends an MSGin5G message request to MSGin5G Server as specified in clause 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient AS Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines the MSGin5G Client is allowed to send message to Application Server.

2.	The MSGin5G Server forwards the MSGin5G message request to Application Server as specified in clause 8.3.3.

3-4.	If Delivery status required is included in the MSGin5G message request, Application Server sends message delivery status report to the MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server sends the delivery status to MSGin5G Client as specified in clause 8.3.5.

#### 8.7.3.2	From Legacy 3GPP UE to Application Server

This procedure is used for message reply from Non-3GPP UE to Application Server.

Figure 8.7.3.2-1 shows the message delivery procedure from Legacy 3GPP UE to Application Server.

Pre-conditions:

1.	The Application server has established a secured communication with the MSGin5G Server.

2.	The Legacy 3GPP Message Gateway has registered with the MSGin5G Server on behalf of the message client in the Legacy 3GPP UE.

3	The Legacy 3GPP UE received a message from the Application Server.

4.	The Legacy 3GPP Message Gateway is aware of the Legacy 3GPP Message client on the Legacy 3GPP UE and provides the mapping between its identifiers and UE Service ID.

5.	The Legacy 3GPP Message Gateway implementation supports storing a messaging transaction, i.e. mapping the message originating MSGin5G Service ID and the message delivered to the  Legacy 3GPP UE, for an operator configured time period to allow if the  Legacy 3GPP UE will send a response to the incoming message.

Figure 8.7.3.2-1: Legacy 3GPP UE replies to Application Server

1.	The Legacy 3GPP UE sends an Legacy 3GPP message request to the Legacy 3GPP Message Gateway (e.g. through SMSC if SMS is used according the procedure in 3GPP TS 23.204 [13] or the procedure in clause 4.13.3 of TS 23.502 [7]).

2.	The Legacy 3GPP Message Gateway translates the Legacy 3GPP message to MSGin5G message, and may include message delivery status report requested in the MSGin5G message. The Legacy 3GPP Message Gateway sends the MSGin5G message request to the MSGin5G Server as specified in clause 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient AS Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, and may include Application ID, Payload and Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines that the Legacy 3GPP UE with its UE Service ID is allowed to send a message to the Application Server.

3.	The MSGin5G Server forwards the MSGin5G message in API request to Application Server as specified in clause 8.3.3.

4-6.	If message delivery status report is requested, the Application Server sends message delivery status report by API request to MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server forwards the message delivery status report to the Legacy 3GPP Message Gateway as specified in clause 8.3.5, the Legacy 3GPP Message Gateway translates the MSGin5G message delivery status report to a Legacy 3GPP message delivery status report and sends it to the Legacy 3GPP UE.

#### 8.7.3.3	From Non-3GPP UE to Application Server

This procedure is used for message reply from Legacy 3GPP UE to Application Server.

Figure 8.7.3.3-1 shows the message delivery procedure from Non-3GPP UE to Application Server.

Pre-conditions:

1.	The Application server has established a secured communication with the MSGin5G Server

2.	The Non-3GPP Message Client in Non-3GPP UE has registered with the MSGin5G Server via the Non-3GPP Message Gateway.

3.	The Non-3GPP UE received a message from the Application Server.

4.	The Non-3GPP Message Gateway is aware of the Non-3GPP message client on the Non-3GPP UE and provides the mapping between its identifiers and UE Service ID.

5.	The Non-3GPP Message Gateway implementation supports storing a messaging transaction, i.e. mapping the message originating MSGin5G Service ID and the message delivered to the Non-3GPP UE, for an operator configured time period to allow if the Non-3GPP UE will send a response to the incoming message.

Figure 8.7.3.3-1: Non-3GPP UE replies to Application Server

1.	The Non-3GPP UE sends a Non-3GPP message request to the Non-3GPP Message Gateway.

2.	The Non-3GPP Message Gateway translates the Non-3GPP Message to MSGin5G message and may include MSGin5G message delivery status report requested in the MSGin5G message.  The Non-3GPP Message Gateway sends an MSGin5G message request to the MSGin5G Server as specified in clause 8.3.2 with the following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient AS Service ID and Message ID information elements in Table 8.3.2-1, and may include Delivery status required, Application ID, Payload, Priority type information elements from Table 8.3.2-1.

b)	Upon receiving the MSGin5G message request, the MSGin5G Server determines that the Non-3GPP UE with its UE Service ID is allowed to send a message to the Application Server.

3.	The MSGin5G Server forwards the MSGin5G message in API request to Application Server as specified in clause 8.3.3.

4-6.	If the delivery status is required, the Application Server sends message delivery status report by API request to MSGin5G Server as specified in clause 8.3.4, the MSGin5G Server sends the message delivery status report to the Non-3GPP Message Gateway as specified in clause 8.3.5, the Non-3GPP Message Gateway translates the MSGin5G message delivery status report to a Non-3GPP message delivery status report and sends it to the Non-3GPP UE.