Annex 1

3GPP 5G

The present Annex includes unwanted emission requirements from NR carriers for NR base stations. The requirements shall apply to BS that support NR, or NR with NB-IoT operation in NR in-band.

A NR base station is characterized by the ability of its receiver and transmitter to process only NR carriers or NR carriers with NB-IoT operation in NR in-band.

This Annex is outlined as follows:

–	Chapter 1 specifies the frequency ranges and operating bands for which the requirements in the present Annex apply.

–	Chapter 2 specifies definitions, symbols and abbreviations.

–	Chapter 3 includes the NR BS unwanted emission requirements for BS type 1-C and BS type 1-H.

–	Chapter 4 includes the NR BS unwanted emission requirements for BS type 1-O and BS type 2-O.

–	Chapter 5 specifies references.

Values specified in the present Annex incorporate test tolerances defined in Recommendation ITU-R M.1545 [4].

# 1	Operating bands

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in table 1-1.

TABLE 1-1

NR frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The unwanted emission limits defined in the present Annex are for NR BS operating at least one of the bands in table 1-2 or table 1-3:

TABLE 1-2

NR operating bands in FR1

| NR operating band number | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n13 | 777 MHz – 787 MHz | 746 MHz – 756 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n24(7) | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n30 | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD(3) |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n67 | N/A | 738 MHz – 758 MHz | SDL |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD(2) |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD(2) |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD(2) |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD(2) |
| n95(1) | 2010 MHz – 2025 MHz | N/A | SUL |
| n96(4) | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD(3) |
| n97(5) | 2300 MHz – 2400 MHz | N/A | SUL |
| n98(5) | 1880 MHz – 1920 MHz | N/A | SUL |
| n99(6) | 1626.5 MHz -1660.5 MHz | N/A | SUL |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |
| n102(4) | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD(3) |
| n104(8) | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |
| (1) This band is applicable in China only. (2) Variable duplex operation does not enable dynamic variable duplex configuration by the network and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. (3) This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [4]. (4) This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. (5) The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. (6) UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. (7) DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. (8) This band is applicable only in countries/regions designating this band for IMT licensed operation in accordance with RCC Recommendation 1/21. |  |  |  |

TABLE 1-3

NR operating bands in FR2

| NR operating band number | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n259 | 39500 MHz – 43500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |
| n262 | 47200 MHz – 48200 MHz | TDD |
| n263 | 57000 MHz – 71000 MHz | TDD |

# 2	Definitions, symbols and abbreviations

## 2.1	Definitions

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [3] used for the formulation of unwanted emission requirements for FR1

antenna connector: connector at the conducted interface of the BS type 1-C

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:		The operating band(s) for a BS is declared by the manufacturer according to the designations in tables 1-2 and 1-3.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:		For requirements based on EIRP, the radiated interface boundary is associated to the far-field region

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted RX requirements are applied

NOTE:		Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted TX requirements are applied.

NOTE:		Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

## 2.2	Symbols

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12 kHz

FDL_low		Lowest frequency of the downlink operating band

FDL_high		Highest frequency of the downlink operating band

FUL_low		Lowest frequency of the uplink operating band

FUL_high		Highest frequency of the uplink operating band

ΔfOBUE		Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

NRB		Transmission bandwidth configuration, expressed in resource blocks

NTXU,counted	The number of active transmitter units as calculated in clause 3.1, that are taken into account for conducted TX output power limit in clause 3.1.2, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 3.1

## 2.3	Abbreviations

ACLR	      Adjacent Channel Leakage Ratio

CA	      Carrier Aggregation

CACLR      Cumulative ACLR

CLTA		Co-Location Test Antenna

DTT	      Digital terrestrial television

EIRP		Effective Isotropic Radiated Power

FR		Frequency Range

GPS	      The Global Positioning System

OTA		Over-The-Air

PHS		Personal Handyphone System

RIB		Radiated Interface Boundary

SCS		Sub-Carrier Spacing

TAB		Transceiver Array Boundary

TRP		Total Radiated Power

## 2.4	Conducted and radiated reference points

The manufacturer can declare the NR BS to be one of the following BS types:

– 	BS type 1-C: Requirements are applied at the BS antenna connector (port A) for a single transmitter or receiver with a full complement of transceivers for the configuration in normal operating conditions. If any external apparatus such as an amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

Figure 2-1

BS type 1-C transmitter interface

Figure 2-2

BS type 1-C receiver interface

– 	BS type 1-H: Requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

Figure 2-3

Radiated and conducted reference points for BS type 1-H

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is requirement specific.

– 	BS type 1-O: Radiated characteristics for BS type 1-O are defined over the air (OTA) where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). The (spatial) characteristics in which the OTA requirements apply are requirement specific.

Figure 2-4

General architecture of BS type 1-O and BS type 2-O

– 	BS type 2-O: OTA requirements defined at the RIB only, same as for the BS type 1-O.

## 2.5	Base station classes

Some requirements apply specifically to a specific BS class. The BS classes for NR are Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations.

BS classes for BS type 1-C and BS type 1-H are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

BS classes for BS type 1-O and BS type 2-O are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

# 3	NR generic unwanted emission characteristics for BS type 1-C and BS type 1-H

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

The values of ΔfOBUE are defined in table 3-1 for the NR operating bands.

TABLE 3-1

Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-C | FDL_high – FDL_low 200 MHz | 10 |
|  | 200 MHz < FDL_high – FDL_low 900 MHz | 40 |
| BS type 1-H | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz FDL_high – FDL_low 900 MHz | 40 |

For band n46, n96 and n102, the values of ΔfOBUE are defined in table 3-2.

TABLE 3-2

Maximum offset of OBUE outside the downlink operating band

| Operating band | ΔfOBUE (MHz) |
| --- | --- |
| n46, n102 | 40 |
| n96 | 50 |

For BS type 1-H, the manufacturer shall declare the minimum number of supported geographical cells (i.e. geographical areas). The minimum number of supported geographical cells (Ncells) relates to the BS setting with the minimum amount of cell splitting supported with transmission on all TAB connectors supporting the operating band. The manufacturer shall also declare TAB connector TX min cell groups. Every TAB connector supporting transmission in an operating band shall map to one TAB connector TX min cell group supporting the same operating band. The mapping of TAB connectors to cells is implementation dependent.

The number of active transmitter units that are considered when calculating the emissions limit (NTXU, counted) for BS type 1-H is calculated as follows:

NTXU, counted = min(NTXU,active, 8·Ncells)

Further:

NTXU,countedpercell = NTXU,counted/Ncells

NTXU,countedpercell is used for scaling the basic limits.

NOTE:		NTXU,active depends on the actual number of active transmitter units and is independent to the declaration of Ncells.

## 3.1	Conducted operating band unwanted emissions

### 3.1.1	Conducted Operating band unwanted emissions for BS type 1-C

The operating band unwanted emissions for BS type 1-C for each antenna connector shall be below the applicable basic limits defined in clauses 3.1.3 – 3.1.8.

For Band n41 and n90 operation in Japan, the operating band unwanted emissions limits shall be applied to the sum of the emission power over all antenna connectors for BS type 1-C.

### 3.1.2	Conducted Operating band unwanted emissions BS type 1-H

The operating band unwanted emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clauses 3.1.3 – 3.1.8, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.1.3	Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, basic limits are specified in table 6.6.4.5.2-1 of TS 38.141-1 [1].

For BS operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n92, n94, basic limits are specified in table 6.6.4.5.2-2 of TS 38.141-1 [1]:

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in table 6.6.4.5.2-3 of TS 38.141-1 [1].

### 3.1.4	Basic limits for Wide Area BS (Category B)

For Category B Operating band unwanted emissions, there are two options for the basic limits that may be applied regionally. Either the basic limits in clause 6.6.4.2.2.1 or clause 6.6.4.2.2.2 shall be applied.

#### 3.1.4.1	Category B requirements (Option 1)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, basic limits are specified in table 6.6.4.5.3.1-1 of TS 38.141-1 [1].

For BS operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n92, n94, basic limits are specified in tables 6.6.4.5.3.1-2 of TS 38.141-1 [1].

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in table 6.6.4.5.3.1-3 of TS 38.141-1 [1].

#### 3.1.4.2	Category B requirements (Option 2)

The limits in this clause are intended for Europe and may be applied regionally for BS operating in Bands n1, n3, n7, n8, n38, n65 or n101.

For a BS operating in Bands n1, n3, n8, n65 or BS type 1-C operating in bands n7, n38 or n101 basic limits are specified in table 6.6.4.5.3.2-1 of TS 38.141-1 [1].

### 3.1.5	Basic limits for Medium Range BS (Category A and B)

For Medium Range BS in NR bands ≤ 3 GHz, basic limits are specified in table 6.6.4.5.4-1 and table 6.6.4.5.4-2 of TS 38.141-1 [1].

For Medium Range BS in NR bands > 3 GHz, basic limits are specified in table 6.6.4.5.4-3 and table 6.6.4.5.4-4 of TS 38.141-1 [1].

### 3.1.6	Basic limits for Local Area BS (Category A and B)

For Local Area BS in NR bands ≤ 3 GHz, basic limits are specified in table 6.6.4.5.5-1 of TS 38.141-1 [1].

For Local Area BS in NR bands > 3 GHz, basic limits are specified in table 6.6.4.5.5-2 of TS 38.141-1 [1].

### 3.1.7	Basic limits for Local Area and Medium Range BS for band n46, n96 and n102 (Category A and B)

For Local Area and Medium Range BS operating in Band n46, basic limits for 10 MHz channel bandwidth are specified in table 6.6.4.5.5A-1 of TS 38.141-1 [1]. For Local Area and Medium Range BS operating in Band n46, n96 and Band n102, basic limits for 20 MHz, 40 MHz, 60 MHz and 80 MHz channel bandwidth are specified in table 6.6.4.5.5A-2 of TS 38.141-1 [1]. For one non-transmitted channel basic limits are specified in table 6.6.4.5.5A-3 of TS 38.141-1 [1], and for two non-transmitted channels basic limits are specified in table 6.6.4.5.5A-4 of TS 38.141-1 [1].

In the case of one or two non-transmitted 20 MHz channels between transmitted channels, when a NR-U channel bandwidth of 60 MHz or 80 MHz have been assigned, the spectrum emission mask for non-transmitted channels specified in table 6.6.4.5.5A-3 and table 6.6.4.5.5A-4 of TS 38.141-1 [1] applies for one and two non-transmitted channels respectively. The relative power of any BS emission shall not exceed the most stringent levels given by table 6.6.4.5.5A-2 and table 6.6.4.5.5A-3 of TS 38.141-1 [1]in the case of non-transmitted channels between transmitted channels.

### 3.1.8	Basic limits for additional requirements

In certain regions the following additional requirements may apply:

– 	For protection of DTT, the requirement in clause 6.6.4.5.6.2 of TS 38.141-1 [1].

–	For operation in Band n48, the requirement in clause 6.6.4.5.6.3 of TS 38.141-1 [1].

–	For operation in Band n53, the requirement in clause 6.6.4.5.6.4 of TS 38.141-1 [1].

–	For protection of GPS, the requirement in clause 6.6.4.5.6.5 of TS 38.141-1 [1].

–	For operation with shared spectrum channel access, the requirement in clause 6.6.4.5.6.6 of TS 38.141-1 [1].

## 3.2	Conducted Adjacent channel leakage ratio (ACLR)

### 3.2.1	General requirements

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The assumed filter for the adjacent channel frequency is defined in table 6.6.3.5.2-1, 6.6.3.5.2-1a, 6.6.3.5.2-3 and 6.6.3.5.2-3a of TS 38.141-1 [1] and a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency.

### 3.2.2	Limits and basic limits

For operation in paired and unpaired spectrum except for band n46, n96 and n102, the ACLR shall be higher than the value specified in table 6.6.3.5.2-1 of TS 38.141-1 [1].

For operation in non-contiguous spectrum or multiple bands except for band n46, n96 and n102, the ACLR shall be higher than the value specified in table 6.6.3.5.2-3 of TS 38.141-1 [1].

For band n46, n96 and n102, the ACLR shall be higher than the value specified in table 6.6.3.5.2-1a of TS 38.141-1 [1].

For operation in non-contiguous spectrum for band n46, n96 and n102, the ACLR shall be higher than the value specified in table 6.6.3.2-3a of TS 38.141-1 [1].

The ACLR absolute basic limit is specified in table 6.6.3.5.2-2 of TS 38.141-1 [1].

### 3.2.3	BS type 1-C

For operation in paired and unpaired spectrum except for band n46, n96 and n102, the ACLR test requirements for BS type 1-C are given in table 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the ACLR limit in table 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1], or the absolute basic limits in table 6.6.3.5.2-2 of TS 38.141-1 [1], whichever is less stringent.

For operation in band n46, n96 and n102, the ACLR test requirements for BS type 1-C are given in table 6.6.3.5.2-1a or 6.6.3.5.2-3a of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the ACLR limit in table 6.6.3.5.2-1a or 6.6.3.5.2-3a of TS 38.141-1 [1], or the absolute basic limits in table 6.6.3.5.2-2 of TS 38.141-1 [1], whichever is less stringent.

For Band n41 and n90 operation in Japan, absolute ACLR limits shall be applied to the sum of the absolute ACLR power over all antenna connectors for BS type 1-C.

### 3.2.4	BS type 1-H

The ACLR absolute basic limits in table 6.6.3.5.2-2 of TS 38.141-1 [1] + X (where X = 10log10(NTXU,countedpercell)) or the ACLR limits in table 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1], whichever is less stringent, shall apply for each TAB connector TX min cell group.

Conformance to the BS type 1-H ACLR limit can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The ratio of the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the assigned channel frequency to the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency shall be greater than or equal to the ACLR limit of the BS. This shall apply for each TAB connector TX min cell group.

Or

2)	The ratio of the filtered mean power at the TAB connector centred on the assigned channel frequency to the filtered mean power at this TAB connector centred on the adjacent channel frequency shall be greater than or equal to the ACLR limit of the BS for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group.

In case the ACLR absolute basic limit of BS type 1-H are applied, the conformance can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency shall be less than or equal to the ACLR absolute basic limit + X (where X = 10log10(NTXU,countedpercell)) of the BS. This shall apply to each TAB connector TX min cell group.

Or

2)	The filtered mean power at each TAB connector centred on the adjacent channel frequency shall be less than or equal to the ACLR absolute basic limit of the BS scaled by X -10log10(n) for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group, where n is the number of TAB connectors in the TAB connector TX min cell group.

## 3.3	Conducted Cumulative adjacent channel leakage ratio (CACLR)

### 3.3.1	General requirements

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.6.3.5.2-4 of TS 38.141-1 [1] and the filters on the assigned channels are defined in table 6.6.3.5.2-6 of TS 38.141-1 [1].

### 3.3.2	Limits and basic limits

For operation in non-contiguous spectrum or multiple bands except for band n46, n96 and n102, the CACLR for NR carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall be higher than the value specified in table 6.6.3.5.2-4 of TS 38.141-1 [1].

For operation in non-contiguous spectrum for band n46, n96 and n102, the CACLR for NR carriers located on either side of the sub-block gap shall be higher than the value specified in table 6.6.3.5.2-4a of TS 38.141-1 [1].

The CACLR absolute basic limit is specified in table 6.6.3.5.2-5 of TS 38.141-1 [1].

### 3.3.3	BS type 1-C

The CACLR test requirements for BS type 1-C are given in table 6.6.3.5.2-4 of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the CACLR limit in table 6.6.3.5.2-4 of TS 38.141-1 [1] or the absolute basic limits in table 6.6.3.5.2-5 of TS 38.141-1 [1], whichever is less stringent.

### 3.3.4	BS type 1-H

The CACLR absolute basic limits in table 6.6.3.5.2-5 of TS 38.141-1 [1] + X, (where X = 10log10(NTXU,countedpercell)) or the CACLR limits in table 6.6.3.5.2-4 of TS 38.141-1 [1], whichever is less stringent, shall apply for each TAB connector TX min cell group.

Conformance to the BS type 1-H CACLR limit can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The ratio of the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the assigned channel frequency to the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency shall be greater than or equal to the CACLR limit of the BS. This shall apply for each TAB connector TX min cell group.

Or

2)	The ratio of the filtered mean power at the TAB connector centred on the assigned channel frequency to the filtered mean power at this TAB connector centred on the adjacent channel frequency shall be greater than or equal to the CACLR limit of the BS for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group.

In case the CACLR absolute basic limit of BS type 1-H are applied, the conformance can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency shall be less than or equal to the CACLR absolute basic limit + X (where X = 10log10(NTXU,countedpercell)) of the BS. This shall apply to each TAB connector TX min cell group.

Or

2)	The filtered mean power at each TAB connector centred on the adjacent channel frequency shall be less than or equal to the CACLR absolute basic limit of the BS scaled by X -10log10(n) for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group, where n is the number of TAB connectors in the TAB connector TX min cell group.

## 3.4	Conducted Transmitter spurious emissions

### 3.4.1 Conducted transmitter spurious emissions for BS type 1-C

The Tx spurious emissions for BS type 1-C for each antenna connector shall not exceed the applicable basic limits defined in clauses 3.4.3-3.4.6.

For Band n41 and n90 operation in Japan, the sum of the spurious emissions over all antenna connectors for BS type 1-C shall not exceed the basic limits defined in clauses 3.4.3-3.4.6.

### 3.4.2 Conducted transmitter spurious emissions for BS type 1-H

The Tx spurious emissions requirements for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 3.4.3-3.4.6, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the BS type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1) 	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2) 	The unwanted emissions power at each TAB connector shall be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.4.3 Basic limits for transmitter spurious emissions

The limits of either table 6.6.5.5.1.1-1 in TS 38.141-1 [1] for Category A or table 6.6.5.5.1.1-2 in TS 38.141-1 [1] for Category B shall apply. The application of either Category A or Category B limits shall be the same as for operating band unwanted emissions in clause 3.1, and as declared by the manufacturer.

### 3.4.4 Protection of the BS receiver of own or different BS

This requirement shall be applied for NR FDD operation in order to prevent the receivers of the BSs being desensitised by emissions from a BS transmitter. It is measured at the transmit antenna connector for BS type 1-C or at the TAB connector for BS type 1-H for any type of BS which has common or separate Tx/Rx antenna connectors / TAB connectors.

The basic limits are provided in table 6.6.5.5.1.2-1 in TS 38.141-1 [1].

### 3.4.5 Additional spurious emission requirements

In certain regions the following additional requirements in TS 38.141-1 [1] may apply:

– 	For BS co-existence with systems operating in other frequency bands, the requirement in table 6.6.5.5.1.3-1.

– 	For protection of PHS, the requirement in table 6.6.5.5.1.3-2.

– 	For BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band n51 and Band n76, the requirement in table 6.6.5.5.1.3-4.

– 	For BS operating in NR Band n50 and n75 within 1492-1517 MHz, and in Band n74 within 1492-1518 MHz, the requirements in table 6.6.5.5.1.3-5.

– 	For BS operating in Band n13 and n14, the requirements in table 6.6.5.5.1.3-6.

– 	For BS operating in Band n30, the requirements in table 6.6.5.5.1.3-7.

– 	For BS operating in Band n48, the requirements in table 6.6.5.5.1.3-8.

– 	For BS operating in Band n26 for protection of 800 MHz public safety operations, the requirements in table 6.6.5.5.1.3-9.

– 	For BS operating in Band n41 and n90 in Japan, the requirements in table 6.6.5.5.1.3-10.

– 	For BS operating in 3.45-3.55 GHz in Band n77 in certain regions, the requirements in table 6.6.5.5.1.3-11.

– 	For BS operating in Band n101 in CEPT countries, the requirements in table 6.6.5.5.1.3-12.

For BS operating in Band n100 in CEPT countries, the requirements in table 6.6.5.5.1.3-13.

### 3.4.6 Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The basic limits are in table 6.6.5.5.1.4-1 in TS 38-141-1 [1] for a BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared BS class. For a multi-band connector, the exclusions and conditions in the Note column of table 6.6.5.5.1.4-1 shall apply for each supported operating band.

## 3.5	Conducted receiver spurious emissions

### 3.5.1	Conducted receiver spurious emissions BS type 1-C

The receiver spurious emissions for BS type 1-C for each antenna connector shall be below the applicable basic limits defined in clause 3.5.3.

For Band n41 and n90 operation in Japan, the sum of RX spurious emissions over all antenna connectors for BS type 1-C shall not exceed basic limits defined in clause 3.5.3.

### 3.5.2	Conducted receiver spurious emissions BS type 1-H

The receiver spurious emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 3.5.3, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.5.3	Basic limits for receiver spurious emissions

Basic limits for receiver spurious emissions are specified in table 7.6.5.1-1 of TS 38.141-1 [1].

# 4	NR generic unwanted emission characteristics for BS type 1-O and BS type 2-O

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 4-1 for BS type 1-O and BS type 2-O for the NR operating bands.

TABLE 4-1

Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low ≤ 4000 MHz | 1500 |

## 4.1	OTA Operating band unwanted emissions

### 4.1.1	OTA Operating band unwanted emissions BS type 1-O

For BS type 1-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to BS channel bandwidths of the outermost carrier. In addition, for a RIB operating in non-contiguous spectrum, the requirements shall apply inside any sub-block gap. In addition, for a multi-band RIB, the requirements shall apply inside any Inter RF Bandwidth gap.

#### 4.1.1.1	Wide Area BS (Category A)

For a RIB operating in Bands n5, n8, n12, n13, n14, n26, n28, n29, n71, n85, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.1-1 of TS 38.141-2 [2].

For a RIB operating in Bands n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n77, n78, n79, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.1-2 to 6.7.4.5.1.1-4 of TS 38.141-2 [2].

#### 4.1.1.2	Wide Area BS Category B (Option 1)

For a RIB operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.2-1 of TS 38.141-2 [2].

For a RIB operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n77, n78, n79, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.2-2 to 6.7.4.5.1.2-4 of TS 38.141-2 [2].

#### 4.1.1.3	Wide Area BS Category B (Option 2)

The limits in this clause are intended for Europe and may be applied regionally for a RIB operating in bands n1, n3, n8 and n65.

For a RIB operating in bands n1, n3, n8, n65 emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.3-1 of TS 38.141-2 [2].

#### 4.1.1.4	Medium Range BS (Category A and Category B)

For Medium Range BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-1 and 6.7.4.5.1.4-4 of TS 38.141-2 [2].

For Medium Range BS class in 3GHz <NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-2 and 6.7.4.5.1.4-5 of TS 38.141-2 [2].

For Medium Range BS class in 4.2GHz <NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-3 and 6.7.4.5.1.4-6 of TS 38.141-2 [2].

#### 4.1.1.5	Local Area BS (Category A and Category B)

For Local Area BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.5-1 of TS 38.141-2 [2].

For Local Area BS class in 3 GHz < NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-2 of TS 38.141-2 [2].

For Local Area BS class in 4.2 GHz < NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-3 of TS 38.141-2 [2].

#### 4.1.1.6	Additional requirements

In certain regions the following additional requirements may apply:

– 	For protection of DTT, the requirement in clause 6.7.4.5.1.6.2 of TS 38.141-1 [1].

– 	For Protection of GPS, the requirement in clause 6.7.4.5.1.6.3 of TS 38.141-1 [1].

– 	For operation in Bands n50, n51, n74, n75, n76, the requirement in clause 6.7.4.5.1.6.4 of TS 38.141-1 [1].

### 4.1.2	OTA Operating band unwanted emissions BS type 2-O

For BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to the frequencies starting from the edge of the contiguous transmission bandwidth. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any sub-block gap.

#### 4.1.2.1	OTA operating band unwanted emission limits (Category A)

The power of unwanted emission shall not exceed the limits in table 6.7.4.5.2.2-1, 6.7.4.5.2.2-2 or 6.7.4.5.2.2-3 of TS 38.141-2 [2].

#### 4.1.2.2	OTA operating band unwanted emission limits (Category B)

The power of unwanted emission shall not exceed the limits in table 6.7.4.5.2.3-1, 6.7.4.5.2.3-2 or 6.7.4.5.2.3-3 of TS 38.141-2 [2].

#### 4.1.2.3	Additional OTA operating band unwanted emission limits

The following additional requirement shall apply:

– 	For BS operating in the frequency range 24.25 – 27.5 GHz, for protection of Earth Exploration Satellite Service, the requirement in table 6.7.4.5.2.4.1-1 of TS 38.141-2 [2].

## 4.2	OTA Adjacent channel leakage ratio (ACLR)

### 4.2.1	BS type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/2a of TS 38.141-2 [2] or the OTA ACLR absolute limits in table 6.7.3.5.1-2 of TS 38.141-2 [2] shall apply, whichever is less stringent.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-3 of TS 38.141-2 [2] and the filters on the assigned channels are defined in table 6.7.3.5.1-4 of TS 38.141-2 [2].

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-1of TS 38.141-2 [2].

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1-2 of TS 38.141-2 [2].

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-2a of TS 38.141-2 [2].

### 4.2.2	BS type 2-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.2-1/3 of TS 38.141-2 [2] or the OTA ACLR absolute limits in table 6.7.3.5.2-2 of TS 38.141-2 [2] shall apply, whichever is less stringent.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.2-4 of TS 38.141-2 [2] and the filters on the assigned channels are defined in table 6.7.3.5.2-5 of TS 38.141-2 [2].

The OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-1 of TS 38.141-2 [2].

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.2-2 of TS 38.141-2 [2].

For operation in non-contiguous spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-3 of TS 38.141-2 [2].

## 4.3	OTA Cumulative adjacent channel leakage ratio (CACLR)

### 4.3.1	BS type 1-O

The OTA CACLR limits in table 6.7.3.5.1-3 of TS 38.141-2 [2] or the OTA CACLR absolute limits in table 6.7.3.5.1-3a of TS 38.141-2 [2] shall apply, whichever is less stringent.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-3 of TS 38.141-2 [2] and the filters on the assigned channels are defined in table 6.7.3.5.1-4 of TS 38.141-2 [2].

The OTA CACLR measurement result shall not less than the OTA CACLR limit specified in table 6.7.3.5.1-3 of TS 38.141-2 [2].

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.1-3a of TS 38.141-2 [2].

### 4.3.2	BS type 2-O

The OTA CACLR limits in table 6.7.3.5.2-4 of TS 38.141-2 [2] or the OTA CACLR absolute limits in table 6.7.3.5.2-4a of TS 38.141-2 [2] shall apply, whichever is less stringent.

The CACLR in a sub-block gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.2-4 of TS 38.141-2 [2] and the filters on the assigned channels are defined in table 6.7.3.5.2-5 of TS 38.141-2 [2].

For operation in non-contiguous spectrum, the CACLR for carriers located on either side of the sub-block gap shall be less than the value specified in table 6.7.3.5.2-4 of TS 38.141-2 [2].

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.2-4a of TS 38.141-2 [2].

## 4.4	OTA Transmitter spurious emissions

### 4.4.1 OTA transmitter spurious emissions BS type 1-O

For Category A BS the TRP of any spurious emission shall not exceed the limits in table 6.7.5.2.5.1-1 in TS 38.141-2 [2]. For Category B BS the TRP of any spurious emission shall not exceed the limits in table 6.7.5.2.5.1-2 in TS 38.141-2 [2].

### 4.4.2 OTA transmitter spurious emissions BS type 2-O

For Category A BS the power of any spurious emission shall not exceed the limits in table 6.7.5.2.5.2.2-1 in TS 38.141-2 [2]. For Category B BS the power of any spurious emission shall not exceed the limits in table 6.7.5.2.5.2.2-2 in TS 38.141-2 [2].

### 4.4.3 Protection of the BS receiver of own or different BS

This requirement shall be applied for NR FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a BS type 1-O.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.104 [6]. The power levels are specified at the CLTA output, as described in 4.12.2 of TS 38.141-2 [2].

The total power of any spurious emission from both polarizations of the CLTA connector output shall not exceed the limits in table 6.7.5.3.5.1-1 in TS 38.141-2 [2].

### 4.4.4 Additional spurious emissions requirements

In certain regions the following additional requirements in TS 38.141-2 [2] may apply:

For BS co-existence with systems operating in other frequency bands, the requirement in table 6.7.5.4.5-1.

For protection of PHS, the requirement in table 6.7.5.4.5-2

For BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band n51 and Band n76, the requirement in table 6.7.5.4.5-3.

For BS operating in NR Band n50 and n75 within 1492-1517 MHz, and in Band n74 within 1492-1518 MHz, the requirements in table 6.7.5.4.5-4.

For BS operating in Band n13 and n14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations, the requirements in table 6.7.5.4.5-5.

For BS operating in Band n30, the requirements in table 6.7.5.4.5-6.

For BS operating in Band n26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations, the requirements in table 6.7.5.4.5-7.

For BS operating in Band n41 and n90 operation in Japan, the requirements in table 6.7.5.4.5-8.

The following additional requirement shall apply:

For BS operating in the frequency range 24.25 – 27.5 GHz, for protection of Earth Exploration Satellite Service, the requirements in table 6.7.5.4.5.2-1 of TS 38.141-2 [2].

### 4.4.5 Co-location requirements

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE:		For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.104 [6]. The power levels are specified at the CLTA output, as described in 4.12.2 of TS 38.141-2 [2].

The output of the CLTA of any spurious emission shall not exceed the test limit in table 6.7.5.5.5.1-1 in TS 38.141-2 [2].

For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.5.5.5.1-1 apply for each supported operating band.

## 4.5	OTA receiver spurious emissions

### 4.5.1	OTA receiver spurious emissions BS type 1-O

The receiver spurious emissions for BS type 1-O at RIB interface shall be below the applicable limits defined in table 7.7.5.1-1 of TS 38.141-2 [2].

### 4.5.2	OTA receiver spurious emissions BS type 2-O

The receiver spurious emissions for BS type 2-O at RIB interface shall be below the applicable limits defined in table 7.7.5.2-1 of TS 38.141-2 [2], with the step frequencies defined in table 7.7.5.2-2 of TS 38.141-2 [2]. In addition, the requirement to protect Earth Exploration Satellite Service shall apply for BS operating in the frequency range 24.25 – 27.5 GHz, as defined in table 7.7.5.2-3 of TS 38.141-2 [2].

# 5	References

[1]	3GPP TS 38.141-1 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing (Release 17)”.

[2]	3GPP TS 38.141-2 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing (Release 17)”.

[3]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TS 37.213 v17.2.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Physical layer procedures for shared spectrum channel access (Release 17)”.

[6]	3GPP TS 38.104 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 17)”.

Attachment 1 
to Annex 1

Definition of test tolerance

Test tolerance

With reference to Recommendation ITU-R M.1545 [4], “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figure 2 and Figure 3 of Annex 1 of Recommendation ITU-R M.1545. In case the core specification value is equal to the test limit (Figure 3 of Annex 1 of Recommendation ITU-R M.1545) the “test tolerances” are equal to 0 dB.