3GPP TSG SA WG 1 Meeting #104 	S1-233103

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	Qualcomm

pCR Title:	Ambient IoT Functional Requirements

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri , oawoniyi@qti.qualcomm.com

Abstract: This pCR proposes updates to the TS

1. Introduction

- Propose Updates to KPI tables in 6.X1, 6.X2 and 6.X3

2. Reason for Change

- Propose changes to capture the KPI table in the TR with some updates:

  - Update max. range for data collection smart city use cases to 500m not 700m as revised in use cases 5.24 and 5.25 in the TR

  - Include the max. device density (250 devices/100 m2) for the indoor tracking use case 5.8 based on the proposed numbers in the TR

  - Include the max. device density (10 device/100 m2) for the outdoor tracking use case 5.8 based on the proposed numbers in the TR

  - Correct a typo in max. device density for “Outdoor actuator control for medium coverage”, the device density is derived from uses cases 5.11 and 5.25 where the device density is < 20 devices/100 m2

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.XXX

* * * First Change * * * *

# 6	Performance service requirements of Ambient IoT

## 6.X1	Performance service requirements for sensor data collection

Table 6.3-1 KPIs for sensor data collection

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Room envirionment monitoring (e.g. domicile, machinery) | 1-20 s | 99 % | NA | <1 kbit/s | < 100 bits | NA | [10-30] m | NA | Stationary | NA | NA | NA | NA |  |
|  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m² | 30 - 200 m | 6000 m²~30,000 m² | Quasi-stationary | 15 min to half an hour | NA | NA | NA |  |
| Outdoor | Smart grid | 1 s | 99% | NA | < 1 kbit/s | Typically, < 100 bytes | < 10,000 /km² | Typically, 50 m-200 m | [several km² up to 100 000 km²] | Stationary | 5-15 min | NA | NA | several 10 m |  |
|  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km² | [300 m - 500 m] | 430000 m² | Up to 3 km/h | 15 min | NA | NA | NA |  |
|  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km² | [300 m - 500 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.X2	Performance service requirements for tracking

Table 6.4-1 KPIs for tracking

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 k bits | 25/100 m² - 250/100 m2 | 10 m indoor | 200 m² | Static | 1 per hour | 1 s | 90% | 1-3 m, 90% availability |  |
| Outdoor | Outdoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | ≤10 per 100 m² | 500 m | Up to the whole PLMN | up to [10 km/h] | 1 per hour | 1 s | 99% | several 10 m |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.X3	Performance service requirements for actuator control

Table 6.5-1 KPIs for actuator control

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor actuator control | Several seconds | 99% | NA | 2 kbit/s | <100 Bytes | less than 1.5 million/km² | 50- m indoors | <250 m² for home, and 15,800 square meters for supermarket | stationary | 20 min to 2 h | NA | NA | 3 m to 5 m indoor |  |
| Outdoor | Outdoor actuator control for large coverage | hundreds ms level | 99% | N/A | NA | 128 bit (DL) | NA | [500] m outdoors | NA | Static | NA | NA | NA | NA |  |
|  | Outdoor actuator control for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <20 per 100 m² | 200 m | City wide including rural areas | Static | NA | NA | NA | 3 m to 5 m indoor |  |
| NOTE 1: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *