3GPP TSG-SA WG1 Meeting #109 	S1-250059

Athens, Greece, 17-21 February 2025

Source:	OTD_US

pCR Title:	Positioning for Regulatory Services in 6G TR

Draft Spec:	3GPP TR 22.870v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	selvam@trideaworks.com

Abstract: In clause 8.4.1, the requirements related to emergency services should be updated to state that it is for regulatory services, listing emergency services as an example. This is needed because positioning and determining a UE’s location is needed for multiple regulatory services. Similar changes are proposed to 8.4.1, 8.4.3, 8.4.4, and 8.4.6.

---------- First Change ----------

### 8.4.1	Description

Positioning, Navigation and Timing (PNT) is fundamental for a wide range of sectors, such as transportation, critical infrastructure (e.g. 6G system) and regulatory services (e.g. emergency services, LI). Nowadays, GNSS is the backbone technology to fulfil these PNT needs. However, the dependency of these sectors on GNSS certainly results in major threats and risks, in case of unavailability or disruption of GNSS, with severe safety, legal and economic impacts. The resilient positioning use case is based on the demand to withstand and recover rapidly from PNT disruptions anywhere.

In addition, the 6G system may assume the need to obtain the UE location with a valid GNSS solution for the operation of 6G NTN solutions. In such situations, the degradation or unavailability of GNSS positioning, such as due to jamming, spoofing or obstructions, may lead to a degradation or loss of the 6G NTN communication capability, as it may currently occur in 5G NTN operations. For 6G NTN, there may be alternative communication approaches, but if these approaches are limited or not successful, a 6G integrated communication and positioning system can provide a GNSS-independent positioning solution for 6G NTN communications.

The satellite network operator of the 6G system is able to provide the resilient positioning service, by ensuring the availability of 3GPP positioning technologies for the service area, and independently of the availability of non-3GPP technologies (e.g. GNSS). This resilient positioning service can then be exploited by any kind of 6G user (e.g. aircrafts and drones [25]) that may experience GNSS degraded service.

---------- Second Change ----------

### 8.4.3	Service Flows

1. The 3GPP positioning service provided by the satellite network can be initiated by a third party or by the end-user.

2. This 3GPP positioning service can determine the UE location with a certain resilient accuracy level.

3. When triggering the 3GPP positioning service, if the UE fails to obtain the location data within a time limit or the location data is not accurate enough, the UE can select different satellite networks or improve the accuracy of location data combining multiple satellite networks.

4. The UE location is used to provide the resilient positioning service.

5. The UE reports its UE location and positioning accuracy level to the network.

---------- Third Change ----------

### 8.4.4	Post-conditions

The user or third party (e.g. regulatory service entity such as a PSAP) successfully obtains the resilient positioning service.

---------- Fourth Change ----------

### 8.4.6	Potential New Requirements needed to support the use case

[PR 8.4.5-1] The 6G system with satellite access shall be able to provide positioning service to the user or third party (e.g. regulatory service entity such as a PSAP) with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the following KPIs:

Table 8.4.6-1: Performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane landing | [10] m | [10] m | [99 %] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99 %] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |

Editor's Note:	Performance requirements on determining the UE location subject to regulatory requirements and operators' policies are FFS.