3GPP TSG SA WG 1 Meeting #109 	S1-250345

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Merger for Interworking between 6G & Other systems

Draft Spec:	3GPP TR 22.870v0.1.1

Agenda item:	8.1.1

Document for:	Discussion

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This pCR proposes a merger of contributions for the sub-clause in Clause 5 in the 6G TR.

1. Introduction

There were seven contributions addressing interworking between 6G and other systems (i.e., 5G, NTN, and non-3GPP systems).  This document proposes a merger of these eight contributions.

- S1-250019 (vivo, China Mobile, Qualcomm, China Unicom)

- S1-250034 (Qualcomm)

- S1-250054 (KT Corp.)

- S1-250271 (Ericsson)

- S1-250128 (NTT DOCOMO)

- S1-250035 (Qualcomm)

- S1-250157 (CableLabs)

- S1-250108 (SKY Perfect JSAT Corporation)

The rapporteur proposes not to use the use case template (i.e., pre-conditions, service flows, post-conditions, existing features partly or fully convering the use case functionality) and only use “Description” and “Potential New Requirements” sub-clauses.  Some portions of text from those sections from the original submitted contributions may be included in the subclcaue “Description”. Most text from pre/post conditions and service flows sub-sections were not included.

2. Discussion

Each contribuiton is color-coded to maintain the sourcing of the text.  All potential new requirements are present (without change/merging).

There are three aspects to this proposed merger:

Part 1:  5G-6G Interworking – includes text from S1-250019 (vivo, China Mobile, Qualcomm, China Unicom), S1-250034 (Qualcomm), 	S1-250054 (KT Corp.), S1-250271 (Ericsson), and S1-250128 (NTT DOCOMO)

Part 2:  6G with non-3GPP systems – includes text from S1-250035 (Qualcomm) and S1-250157 (CableLabs)

Part 3:  NTN = includes text from S1-250108 (SKY Perfect JSAT Corporation)

The proposed merges (below) are partioned into these three aspects with the intent to discuss each of these three aspects separately.

3. Proposal

It is proposed to review the following merger of proposed changes to clasue 5 in 3GPP TR 22.870 v0.1.1.

* * * First Change * * * *

### 5.2.2	Interworking between 6G and other systems

Editor’s Note:  Text to be provided. This clause to address potential (new) requirements for interworking with legacy 3GPP system(s), non-3GPP systems, NTNs and exceptions (i.e., interworking explicitly not supported).

#### 5.2.2.1 Description

Editor’s Note:  Part 1 of Description clause focuses on 5G-6G interworking.

In the 6G timeframe of 2030 and beyond, especially for initial deployments with limited 6G coverage, it is anticipated there will be a need to support co-existence of 6G and 5G. In such circumstances, the service should be standardised to ensure service continuity and interworking between 6G and 5G.

While 6G will enable new services, the system architecture evolution is expected to provide a smooth migration from a 5G system to a 6G system (6GS), allowing to reuse and to evolve existing 5G System business models and operational schemes. At the time of introduction of 6G the network may be dependent on 5G system (NR radio access) for full network wide coverage. Therefore, it is important to provide a seamless mobility between NR and 6G access to secure support of key services like voice and data, to enable a smooth introduction of 6G.

6G system aims to meet the growing demand in the consumer market by enhancing the system performance and user experiences in more efficient way. By introducing an infrastructure built specifically for 6G services, the full 6G feature set will become available with the promised 6G high performance (which might not be possible in case of a hybrid network) allowing the operators to offer/provide all the intended benefits of 6G to their users. Further, the separate 6G core network avoids dependency between the networks at migration.

Since the 6G network is separate and independent to the legacy 5G network, the UE will have to register individually and independently via simultaneous connections to the 6GC and 5GC networks, which imply the need of supporting Dual Registration.  In order to enable an MNO to control a UE being served by both networks; Dual Registration should use same subscriber identities when registering to the 5GC and 6GC.

Dual registration is a key feature that provides a pathway for the migration of existing 5G networks and 6G enabling the UE to access both networks simultaneously (via both 5G and 6G RATs) and maintaining service continuity with minimum service interruption for the subscribers in two 3GPP networks, as the UE can use both 6G and 5G access networks simultaneously for communication.

With 5G being standardized with multiple architecture options, it was difficult for vendors to implement all different options, which ended up as one of the pain points of 5G. In order to prevent this, 6G will likely be standardized with single standalone architecture from Day One. In 6G standalone architecture, 6G system (6GS) consists of 6G Access Network (AN), 6G Core (6GC) Network and UE. The 6G UE accesses to 6GC via 6G RAN.

In the 6G timeframe, there will be a need to support between 5G system (5GS) and 6GS for a network operator, where 5GS consists of 5G Access Network (AN), 5G Core (5GC) Network and UE as defined in TS 22.261 and TS 23.501. As the coverage of 6GS likely to be smaller at the early stage, it is important for 6G subscribers to support seamless network connection and service continuity during the movement of UE, it is necessary to support the core network interworking between 5GS and 6GS.

The construction of the 6G system/network takes a period of time. In the early stage of 6GS construction, 5G network coverage is continuous, but 6G network coverage may be not continuous. 5GS and 6GS will coexist for a long time.

A 6G UE that is capable of accessing 6G network (6G AN and 6GC) will also be capable of accessing 5G network (5G AN and 5GC). As 6G RAN likely to use wider spectrum compared to 5G RAN, 6G UE will support higher data rate than 5G UE, which will give enhanced user experience to 6G subscribers.

Whether 6GC is an evolved version of 5GC (i.e. e5GC) with backward compatible on most of the main nodes and interfaces, it is recommended to have a separate physical 6GC when commercially deployed in order to increase the stability of 6GS at the initial stage with differentiated performance.

Figure 5.2.2.1-1 5G and 6G interworking scenario

Editor’s Note:  Part 2 of Description clause focuses on 6G-non 3GPP interworking.

The proposal seeks to define requirements for 6G to enable simultaneous access to two 3GPP networks and a non-3GPP network. The aim is to enhance multi-access connectivity by leveraging concurrent data flows across multiple access networks, ensuring optimized performance, improved reliability, and seamless user experiences, particularly for high-bandwidth and low-latency applications. The handling of credentials across networks is TBD and will follow the security specifications outlined for 6G. The non-3GPP access includes untrusted, trusted, wireline and satellite access.

Multiple deployment scenarios that could be envisioned for this use case include:

- Two 3GPP (5G/6G) accesses in two different PLMNs (HPLMN/VPLMN) and a non-3GPP access in the HPLMN

- Two 3GPP (5G/6G) accesses in two different PLMNs (HPLMN/VPLMN) and a non-3GPP access in the same VPLMN as one of the 3GPP access

- Two 3GPP (5G/6G) accesses in two different PLMNs (HPLMN/VPLMN) and a non-3GPP access in a different VPLMN

3GPP Access

Non-3GPP Access

Core Network

Data Network

HPLMN

Device capable of connecting to two 3GPP and a non-3GPP access network simultaneously

3GPP Access

Figure 5.5.5.1-2: Multi-Access Connectivity

The access networks can be integrated directly into the HPLMN core network or via another core network.

Current ATSSS feature allows an ATSSS capable device to simultaneously connect to a 3GPP access and a non-3GPP access. Some of the associated requirements for this are captured under TS 22.261 clause 6.3. Current DualSteer feature allows a DualSteer capable device to simultaneously connect to two 3GPP networks. Some of the associated requirements for this are captured under TS 22.261 clause 6.50.

Assuming the device is capable of simultaneously connecting to two 3GPP networks and a non-3GPP network, then the device should be able to steer/switch/split traffic across two 3GPP networks and a non-3GPP network.

Editor’s Note:  Part 3 of Description clause focuses on NTN.

The satellite communication market is rapidly expanding and is expected to become a critical infrastructure by 2030 and beyond, given its anticipated applications in diverse fields such as IoT, remote area connectivity, and military and defence purposes.

Supporting Non-Terrestrial Networks (NTN) will be indispensable for 6G. Satellites, in particular, which provide wide-area coverage without relying on terrestrial infrastructure, play a crucial role in complementing ground networks by ensuring communication in unconnected regions or during disasters. They also contribute to building resilient industrial infrastructure and bridging the global digital divide.

In general, satellite operations are designed for long-term use. Specifically:

-	LEO satellites are expected to have a lifespan of over 5 years, with advancements in technology enabling operations exceeding 10 years.

-	GEO satellites typically have a lifespan of over 15 years, with many cases of operation extending beyond their designed lifespan.

Therefore, the 5G system utilized with communication satellite and ground equipment is to be deployed more than 10 years from the implementation. The long lifecycle of satellites contributes to the long-term stability of communication networks. Considering this extended operational potential, it is essential for the 6G system to continue supporting the functions of Non-Terrestrial Networks (NTN), which were standardized in the 5G. This long-term characteristic serves as a foundation for the continued utilization of NTN technologies in 6G.

Given the longevity of satellites and the effectiveness of NTN technology established under 5G, it is both rational and essential to support these functions in 6G as well. This ensures the development of a sustainable communication infrastructure.

Additionally, commercial deployment of 5G NTN is still in its early stages, with many operators expected to launch services in the latter half of the 2020s. Considering this timeline, the period allocated for standardizing and developing 5G NTN has been short, making it challenging for NTN specifications and service models to fully mature within 5G. This lack of maturity could lead to missed business opportunities. However, replacing 5G system implemented in the late 2020s with 6G system would be impractical for many satellite operators. This difficulties with the replacement extend to not only for operators to provide new features with as less investment as possible but also for users to replacing a relatively expensive VSAT UE. Therefore, while aiming to enhance NTN capabilities and add essential functionalities under 6G, these should not be limited exclusively to new specifications for 6G satellite radio access. Instead, it is necessary to build upon and integrate the existing foundation of 5G NTN systems while providing new features.

#### 5.2.2.2	Potential New Requirements

Editor’s Note:  Part 1 of Potential New Requirements clause focuses on 5G-6G interworking.

[PR 5.2.2.2-00a] The 6G system shall be able to support service continuity with 5G system.

[PR 5.2.2.2-00b] The 6G system shall support interworking with 5G system.

[PR 5.2.2.2-00c] The 6G system shall be able to support service continuity during mobility between 6G and 5G.

NOTE: FFS about mobility between 6G and 4G, e.g., for voice services.

[PR 5.2.2.1-00d] The 3GPP system shall be able to support Inter System Mobility between 5GS and 6GS, and the mobility procedures between a 6G core network and a 5G core network with minimum impact to the user experience (e.g. QoS, QoE).

[PR 5.2.2.1-00e] The 3GPP system shall be able to provide interface between 5GS and 6GS to ensure service continuity to UE capable of both 5GS and 6GS.

[PR 5.2.2.1-00f] The 6G system shall support both NR and 6G radio access.

[PR 5.2.2.1-00g] The 6G system shall support seamless mobility between NR radio access and 6G radio access for both data and voice services.

[PR 5.2.2.1-00h] 	A 3GPP system shall be able to support UE dual registration in 6GC and 5GC network using same subscriber identifier.

[PR 5.2.2.1-00I] 	A 3GPP system shall be able to support shared subscription information between 5G and 6G networks.

[PR 5.2.2.1-00j] 	The 3GPP system shall be able to maintain each registration independently.

[PR 5.2.2.1-00k] 	The UE shall support dual registration in 6GC and 5GC network using same subscriber identifier and be able to maintain each registration independently.

[PR 5.2.2.1-00L]	For using a certain service/application, the network operator shall be able to provide a mechanism enables the UE to decide either to use 6G or/and 5G network for that service/application if both networks are available.

Editor’s Note:  Part 2 of Potential New Requirements clause focuses on 6G-non 3GPP interworking.

[PR 5.2.2.1-00m] Based on operator policy, the 6G system shall be able to support simultaneous connectivity via different access networks (e.g. 6G and 6G, 6G and 5G, 6G and non-3GPP), to access one or more 3GPP services.

[PR 5.2.2.1-00n] Proposed Requirement 1: Subject to the device capability, HPLMN policy and network control, 6G shall support mechanisms to enable traffic steering/switching/splitting of user data across multiple access networks (i.e. two 3GPP and one non-3GPP access network).

NOTE 1: The 3GPP access networks and the non-3GPP access network may belong to the same or different PLMNs (either HPLMN or VPLMN)

NOTE 2: The user data is assumed to be anchored in the HPLMN

Editor’s Note:  Part 3 of Potential New Requirements clause focuses on NTN.

[PR 5.2.2.1-00O] The 6G system with satellite access shall support all the features provided by 5G system with satellite access (e.g. VSAT UE capabilities, multi-orbit satellite access, and emergency communications) from day one.

[PR 5.2.2.1-00p] The 6G system with satellite access shall strive to leverage the 5G system with satellite radio access, considered its commercial deployment.

* * * End of Change * * * *