3GPP TSG SA WG 1 Meeting #109 	S1-250194

Athens, Greece, 17 – 21 Feb 2025	(revision of S1-25xxxx)

Source:	Nokia

Title:	DP for TR22.883 PR consolidation

Agenda item:	7.2 (FS_EnergyServ_Ph2)

Document for:	Discussion

Contact:	Laurent-Walter Goix (laurent-walter.goix@nokia.com)

Abstract: This document is a discussion paper to prepare the consolidation of potential requirements for FS_EnergyServ_Ph2. A proper pCR containing the agreements will be proposed with the appropriate format for inclusion into TR 22.883.

1. Introduction

The following changes are proposed with respect to consolidation categories:

- Renaming existing category “Exposure of information about network energy-related characteristics”

- Adding a new category dedicated to the “Energy-related characteristics as service criteria”

The categories are intended to potentially map with cl 6.15a “Energy Efficiency as a Service Criteria” of TS 22.261 whenever relevant. Some categories may require the creation of new sections under that clause.

Proposed consolidation is then as follows:

- Exposure of network energy-related characteristics: 4 CPRs

- Service adjustments based on energy-related characteristics: 5 CPRs

- Charging-related aspects: 1 CPR

- Energy-related characteristics as service criteria: 1 CPR

Rev0411

- Updated reflecting the text agreed during drafting sessions on 17/02 and 18/02

- New proposals (highlighted in grey) to accommodate comments received during the drafting sessions

Text highlighted in yellow corresponds to changes between the original PR and the CPR. In the right (original PR) column, highlighted text was removed in the CPR. In the left column (CPR), highlighted text was added with respect to the original PR.

Text highlighted in green corresponds to text or actions which were discussed during preparation calls with no further comment.

Text highlighted in blue correspond to proposals to handle ENs.

2. Proposal for discussion

### 1	Exposure of information about network energy-related characteristics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.1-1 | Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to collect, and expose to authorized 3rd parties, the carbon equivalent emissions resulting from the use of the communication service, related to one or more specific UEs of home network subscribers (e.g. fleet of vehicles, IoT devices, company phones etc), over a specific time period (e.g. month etc). NOTE 1: This requirement does not apply to the use of the communication service when roaming. NOTE 2: Exposing carbon equivalent emissions can be based on data volume and operator’s energy consumption and carbon intensity information. In particular, it is expected such exposed information to be provided on demand, not more granularly than on a per-day basis. NOTE 3: The exposed carbon equivalent emissions can further include information to allow to compare subscribers of e.g. the same PLMN, NPN or slice over the same time period. Such information can take the form of e.g. a range (i.e. minimum and maximum) or average of carbon equivalent emissions per subscriber, or the form of a category, class or score of carbon emissions (e.g. A to F, 1 to 5). | PR.5.6.6-1 [PR.5.3.6-1] The 5G network shall be able to collect information on energy consumption and carbon emission that are attributed to the subscriber. NOTE: Attribution to subscribers will be based on a combination of measurements, service configuration, statistics, computations and estimates. The level of accuracy of attribution determines what can be done with the information, from providing information to the subscriber to be part of some carbon accounting scheme. | NOTE 2 may apply to other CPRs. Proposed to merge PR.5.3.6-1 into this existing CPR, whilst removing the energy consumption aspect (already covered by R19 at this granularity). Proposed to remove the NOTE of PR.5.3.6-1 and simplify NOTE 2 to move these considerations to normative work in general description clause. |
| CPR 6.1.1-2 | Subject to operator’s policy and user consent, the 5G network shall be able to expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow. NOTE 1: One of the goals of exchanging Energy Consumption data and CO2e data for a specific individual service data flow is to enable applications to provide an indication to the user during the communication service. The frequency of such indications should be meaningful to the user and is assumed to be configurable, e.g. frequency or updates of thresholds for reporting (e.g. based on e.g. a session, down to once per minute). | [PR.5.5.6-1] The 5G network shall be able to provide exposure of Energy Consumption data and CO2e data for a specific service data flow, to an Application Function. NOTE 1: Energy Consumption data and CO2e data for a specific service data flow is determined through calculations based on e.g. measurements, service settings, external information and models. NOTE 2: The exposure of Energy Consumption data and CO2e data is assumed to be configurable, e.g. frequency or updates of thresholds for reporting. The aim is to provide useful data to participants in the end to end chain. The frequency of data exposure can be down to once per minute. NOTE 3: One of the goals of exchanging Energy Consumption data and CO2e data is to enable applications to provide an indication of Energy Consumption and/or CO2e data for a specific individual service data flow to the user during the communication service. The frequency of such indications of energy usage and CO2e data should be meaningful to the user (e.g. based on e.g. a session, once per minute). | Reworded for clarity Added “user consent”, for the ability to opt-in/out Merged the various NOTEs as some content was overlapping, and some considerations eg from NOTE1 are proposed to be moved to normative work in general description clause. |
| CPR 6.1.1-3 | Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy-related characteristics of the network resources used in serving a UE. NOTE: This information can be used by applications to schedule non time-critical data transfer to or from the UE. | PR.5.8.6-1 | Reworded for clarity to relate to UE (with user consent) and to generalize to any authorized 3rd party |
| CPR 6.1.1-4 | Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current energy-related characteristics of their serving network. NOTE 1: A subset of target UEs can be selected, for example, based on their location, their energy consumption in the network, their service experience, etc. NOTE 2: Future (e.g. predicted) information can refer to the next hour(s) or day(s), to the remaining serving time before shutdown/shortage etc. | [PR.5.9.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current and planned service availability, service performance characteristics and energy-related characteristics of their serving network. NOTE 1: A subset of target UEs can be selected, for example, based on their location, their energy consumption in the network, their service experience, etc. NOTE 2: Planned information can refer to the next hour(s) or day(s), to the remaining serving time before shutdown/shortage etc. | See proposed definition for “service availability characteristics” below |

### 2	Service adjustments based on energy-related characteristics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.2-1 | Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to provide means to degrade service performance (e.g. QoS, bitrate) to meet energy rationing constraints. NOTE: This assumes that the degradation in service performance will reduce energy consumption in the 5G network to meet energy rationing constraints. | PR 5.2.6-2 [PR 5.2.6-1] Subject to user consent, operator policy, and regulatory requirements, the 5G network shall be able to provide means to reduce service performance at the flow level in response to energy rationing. NOTE 1: This assumes that the reduction in service performance will reduce energy consumption in the 5G network. | Reworded for clarity, also to refer to operator’s policy and to refocus on experienced QoS. Added NOTE. Proposed to merge PR 5.2.6-1 into already agreed CPR. Proposed rewording to clarify the energy rationing situation (as per the second PR). Proposed to move the content of the NOTE of the existing CPR to normative work in general description clause. |
| CPR 6.1.2-2 | Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support – and allow authorized 3rd parties to provision - subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons. NOTE: This requirement implies that the policies could disallow some network energy saving action to be performed for some service with QoS criteria e.g. based on applicability conditions (e.g. slice, application). | [PR.5.7.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall support subscription policies that include tolerance to service performance degradation aimed at network energy saving for services with QoS criteria. NOTE 1: Tolerance corresponds to allowed service performance degradation and applicability conditions (e.g. time, geographical area, slice, application). [PR.5.7.6-2] Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to perform energy saving actions resulting in service performance degradation (e.g. QoS parameters, maximum bitrate) for services with QoS criteria, according to the tolerance to service performance degradation. NOTE 2: This requirement implies that the tolerance could disallow some network energy saving actions to be performed for services with QoS criteria. | Merged 2 PRs and removed the word “tolerance” |
| CPR 6.1.2-3 | Subject to operator’s policy, the 5G system shall be able to support a means for the UE to request network actions targeting per UE energy saving, based on pre-agreed subscription policies. | [PR.5.1.6-1] The 5G network shall be able to support a mechanism for the subscriber to provide their preference for the energy saving actions to be provided by the network, e.g., by providing energy saving service options to the subscriber. NOTE 1: The mapping between energy saving service options and network actions is defined by the operator. Editor’s Note: It is FFS to clarify that applying network energy saving actions will not increase the overall 5G system energy consumption compared with not applying them. | Reworded for clarity, also to refer to operator’s policy and user consent, removing the notion of “options”. Updated note to clarify the potential entities involved. --- Suggest to address EN in the normative phase as generic statement to be captured in the TR conclusions (not specific to this requirement) |
| CPR 6.1.2-4 | Subject to operators’ policy, regulatory requirements, 5G network shall provide mechanisms to adjust communication service (e.g. user plane path, suitable Service Hosting Environment) considering the change of energy supply mix of the network as one of the factors. NOTE 1: It is assumed that 5G network can obtain energy supply mix information. How to obtain this information is out of scope of this requirement. The adjustment is not assumed to be real-time. It is up to the operators’ policy to define the time difference between obtaining the change on energy supply mix information and the actual adjustment of the communication service. NOTE 2: Degradation of service experience resulting from the adjustment is subject to subscription-based user consent. | [PR.5.4.6-1] Based on regulatory requirements, operators’ policy and agreement with 3rd party, 5G network shall provide mechanisms to adjust communication service (e.g. reselection of UPF, service hosting environment) considering the change of energy supply mix of the network as one of the factors. NOTE 1: It is assumed that 5G system can obtain energy supply mix information, how to obtain this information is out of scope. The adjustment is not assumed to be real-time, it’s up to the operators’ policy to define the time difference between obtaining the change on energy supply mix and providing adjustment on communication service. Editor’s Note: It is FFS on how to describe adjustment on communication service and whether there are stage 1 examples. | Aligned terminology with stage 1 with updated examples, leveraging terms already used in TS 22.261 (thus proposing to resolve EN and delete it from original PR) Added NOTE 2 to address user consent considerations when applicable. |
| CPR 6.1.2-5 | Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to support a means to configure and provision policies enabling a UE to take action (e.g. defer background traffic, change access type, disable video service etc) based on the energy-related characteristics received from their serving network. NOTE: The resulting UE actions, if any, can depend on the UE type or class (e.g. MPS/MCS, IoT, smartphone etc). In some cases, the device operating system can be designed to centrally manage all non-time critical traffic to optimize both network and UE battery usage, also by leveraging this information. | [PR.5.9.6-2] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to support a means to configure and provision policies enabling a UE to take action (e.g. defer background traffic, change access type, disable video service etc) based on the received service availability and/or service performance and energy-related characteristics of the serving network. NOTE 3: The resulting UE actions, if any, can depend on the UE type or class (e.g. MPS/MCS, IoT, smartphone etc). In some cases, the device operating system can be designed to centrally manage all non-time critical traffic to optimize both network and UE battery usage, also by leveraging this information. | Reworded to focus on “energy-related characteristics” term |

### 3	Charging-related aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.3-1 | Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria (e.g. to a lower bitrate) due to energy saving. NOTE: Such charging events can result in incentives to the subscriber. | [PR.5.7.6-3] Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria due to energy saving. NOTE 3: Such charging events can result in incentives to the subscriber. | Added example as most of QoS criteria relate to bitrate. |

### 4	Energy-related characteristics as service criteria

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.4-1 | Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network. | [PR.5.9.6-3] Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and planned energy-related characteristics of their serving network. | See proposed definition of “energy-related characteristics” |

energy-related characteristics: information which characterize e.g. the type, value or ratio of energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions.

NOTE:	It is assumed that the operator can configure which energy-related characteristics to provide to which type of entity, e.g. UE, authorized 3rd party, etc.