3GPP TSG-SA WG1 Meeting #109 	S1-250427

Athens, Greece, 17-21 February 2025	(revision of S1-250161, 417)

Source:	CATT

pCR Title:	Consolidation for emergency communications and MCX via satellite access

Draft Spec:	3GPP TR 22.887 v1.0.0

Agenda item:	7.3.1

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution discusses a way-forward to consolidate the requirements specific to emergency communication and mission critical services and provides the proposal of consolidation.

1. Introduction

The study on satellite access phase4 (FS_5GSAT_Ph4) has got good progress. 4 use cases relevant to public safety and mission critical services were captured in TR 22.887 v1.0, and several potential requirements were identified during the study. This contribution intends to discuss the consolidation of the requirement specific to emergency communication and mission critical services based on the current progress, for the preparation of the normative work.

2. Reason for Change

There are 4 use cases relevant to public safety and mission critical services captured to TR22.887 v1.0 as listed below:

- Use case on Resilient Satellite Communication for Public Safety (5.3)

- Use case on Public Warning Service(5.9)

- Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay(5.10)

- Use case on emergency communication using satellite access(5.11)

Also, the consolidation framework was agreed in SA1#108 and clause 6.4 is defined for the category of emergency communication and mission critical. As per the progress of the study, it’s time to discuss how to consolidate all the identified requirements of this topic.

Here is a rough analysis and associated change proposal for each potential requirement, as depicted in the following table.

| UC# | Original Potential Requirements | Discussion and Proposal |
| --- | --- | --- |
| 5.3 | [PR 5.3.6-001] Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication. NOTE: The resilient communication can be achieved through a single satellite or multiple satellites over different types of orbits. | The use case takes a scenario of public safety as an example, to discuss the need for resilient satellite communication. However the need is generic, not limited to public safety field. It is more like a case about multi-orbit satellite access for public safety service. Proposal: consider PR 5.3.6-001 in the consolidation of multi-orbit relevant requirements. |
| 5.9 | [PR 5.9.6-001] If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via Non-Terrestrial Networks. | According to the argument in S1-250159, the use case and identified requirement will be covered since Rel-19, so there is no need to consider it in the consolidation phase. Proposal: Remove 6.4.1 PWS with satellite access. |
| 5.11 | Functional Requirements: [PR 5.10.6-001] A 5G system with satellite access, shall support delivery of a cached content from a content caching application via a broadcast/multicast service for mission critical services. [PR 5.10.6-002] A 5G system with satellite access shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE for mission critical services. [PR 5.10.6-003] A 5G network with satellite access shall be able to support multicast services via Mobile Base Station Relays using satellite access. [PR 5.10.6-004] A 5G system with satellite access shall be able to support C2 communication of UAV operation with required QoS via a mobile base station relay using satellite access . NOTE: QoS is end-to-end QoS, with respect to KPIs for command and control as defined in table 7.2-1 TS 22.125. Performance KPI: Table 5.10.6.2-1 Video data over satellite access | PR1 and PR2 are about content caching, and partially covered by existing requirements of TS 22.261, so the gap need to be clarified in consolidated requirements. PR4 and partial KPIs state the enhancement on C2 communication, specific to UAV. KPIs also mention the performance requirement for the transmission between UAV mounted UE and UE/application server under satellite access. The consolidation need to distinguish the above cases. Proposal: Define functional CPRs and KPIs in clause 6.4.1 and 6.4.2 for each capability considering smoothing the normative work in wording. |
| 5.11 | [PR 5.11.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system should be able to support means to provide smaller granularity location information (compared with original cell information in serving satellite) of emergency communication user to the trusted 3rd party when the user triggers the emergency communication via satellite access. | The granularity of location information needs to meet the request of 3rd party in different usages e.g. emergency calls, charging, location-based services, lawful interception, etc. as the existing requirement did. Reference requirement in TS 22.261: The 5G system shall provide 5G positioning services in compliance with regulatory requirements. Proposal: Define one CPR in clause 6.4.1 |

Change to 427,

The requirements of use case 5.11 are not mature enough to do the consolidation, and remove from the revision.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v1.0.0

* * * First Change * * * *

6.4	Emergency service using satellite access

6.4.1	Consolidated functional requirements

The potential requirements corresponding to the support of emergency service using satellite access are listed in the table below.

Table 6.4.1-1 – Consolidated Requirements for emergency service using satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4.1-1 | Subject to the regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to provide location information of a UE that uses only satellite access for emergency service to a PSAP. NOTE: the accuracy of a UE’s location information will be in compliance with regulatory requirements. | [PR 5.11.6-001] |  |

* * * End of Changes * * * *