3GPP TSG SA WG 1 Meeting #109 	S1-250670

17-21 February 2025, Athens, Greece	(revision of S1-250323)

(revision of S1-250287)

Title:	Update of the Use Case in clause 9.2

Agenda item:	8.1.5

Source:	Huawei, HiSilicon, OPPO, China Unicom

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution provides an update to the “Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud” captured in clause 9.2.

1. Introduction

“Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud” has been captured in clause 9.2 with the initial potential new requirement. This contribution proposes the updates to this use case.

2. Reason for Change

Offloading rendering task to cloud (including core network, operator managed data network, edge or central cloud) is a promising trend to support advanced multi-media services in the future. However, the latency for offloading rendering task to cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage. UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile device to dynamically offload part of rendering task to the cloud to improve the user experience.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.1.1.

* * * First Change * * * *

## 9.2	Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

### 9.2.1	Description

Usually, the limited image rendering capability of mobile devices poses challenges for services that require high-quality image processing capability like high order super-resolution and de-noising algorithm, as well as high-quality rendering capability like ray tracing effect for gaming. As shown in Figure 9.2.1-1, the GPU computing power for image rendering increasing in last ten years and the predication up to 2030 for mobile phone platform as well as the computing power required for advanced services has been summarized as below based on [16], [17] and [18]:

Figure 9.2.1-1: increasing computing power: offered by mobile GPU vs. required by services

It can be observed that with the failure of Moore's law, the growth of mobile platform computing power gradually slows down. Offloading rendering task to cloud (including core network, operator managed data network, edge or central cloud), where sufficient computing power is hosted, is a promising trend to support advanced multi-media services in the future while lowering the weight and the power consumption of the XR glasses and mobile phones. UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile device to dynamically offload part or all of rendering task to the cloud to improve the user experience. As shown in Figure 9.2.1-2 the offloading decision can be based on communication link status and computing resource availability in the cloud (core network, operator managed data network, edge or central cloud) in order to maintain deterministic rendering task processing latency. There’s an expectation for 6G systems to consider such dynamic rendering offload framework to adapt to the synergized multi-media service processing trend.

Figure 9.2.1-2: UE-Network-Cloud synergized multi-media operation framework

The rendering task split function is a function provided by the operating system (OS) of the terminals to map the rendering task request from the application (APP) to local or remote rendering resources. Upon a rendering task is requested to be executed, the task split function can select the local and/or the remote rendering resources based on the relevant factors, such as required processing quality, radio link status, estimated latency, available computing resource in core network, operator managed data network, edge or central cloud, to ensure the deterministic user experiences for the requested multi-media service. The typical services which can be benefited from UE-Network-Cloud synergized multi-media operation include photo enhancement and ray tracing for gaming and so on.

The UE-Network-Cloud synergized operation raises several new issues, as follows:

-	uplink data rate: The rendering task offloading operation usually requires high uplink data rate to achieve higher service availability, considering the metadata to be uploaded for the rendering task is usually large with tight latency, e.g. 5Mbits/50ms, 150Mbits/1.5s (see detail in service flow part). However, the current network typically cannot guarantee sufficient uplink data rate especially at cell edge.

-	awareness of real time communication link capability and cloud computing resource availability: The rendering task split function might not know the exact communication link capability such as data rate, latency, and UE power consumption for data transmission; the computing resource availability in core network, operator managed data network, edge or central cloud is also unknown, which make it hard for the UE to make the splitting/offloading decision. Moreover, the latency for offloading rendering task to cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage, so the UE needs to be aware of the real time network and computing resource status to switch between local and remote rendering.

-	Computing task level PDB requirement guarantee: The uplink metadata burst for a rendering task offloading arrives in a random way, and the latency requirement is intended for the whole data burst (i.e. the metadata of the task) rather than an individual packet. As shown in Figure 9.2.1-3, the current GBR QoS flow framework guarantees the data rate in a fixed averaging window and fulfils latency in a per packet basis which is suitable for the periodic traffic. For the burst data carrying the metadata for rendering tasks, the latency and data rate should be guaranteed toward the whole data set rather than a specific packet. Therefore, the fixed averaging window as for periodic traffic is not suitable for bursty traffic. There is no suitable QoS type to guarantee the deterministic transmission latency for unpredictable uplink busty data traffic.

Figure 9.2.1-3: Burst latency requirement

Moreover, in comparison to the accidental single-packet loss whose effect can be mitigated by the packet-loss-resilient solution, the successive packet loss or burst packet loss will incur more frequent video and audio buffering and play stalling. To solve such issue, AI/ML based approach to concealing the lost packets draws a growing attention, which is capable of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [9.2x1].

### 9.2.2	Pre-conditions

1. Rendering resources are deployed in the cloud (core network, operator managed data network, edge or central cloud).

2. The UE operating system supports to map the rendering task from the APP to the local (in the mobile phone) and/or the remote (in the cloud) rendering resources.

### 9.2.3	Service Flows

Synergized photo enhancement case:

1.	User A is in a tourism attraction, taking photos with his mobile phone. Once he clicks the photo button, the phone senor produces raw data of the photo, including up to 6 frames (4K per frame) of raw files spanning 300ms for HDR reconstruction. Raw files retain the original uncompressed image data from the camera sensor, which is usually larger than compressed image format such as JPEG and PNG, and has unique advantages for imaging processing in practice.

NOTE 1: 2 to 10 frames are needed for HDR reconstruction based on [19]; 6 frames are assumed in this use case.

2.	Upon reception of the rendering task request from camera APP for photo enhancement, the rendering task split function inside the UE OS determines whether or not to upload the raw data to the cloud for more powerful post-processing to enhance the photo quality, for which some factors would be considered such as potential delay and UE power consumption for data transmission.

The rendering task split function obtains necessary information from the network, including real time communication link capability and cloud computing resource availability, and coordinates with the core network about whether the rendering task can be fulfilled (e.g., having bandwidth and latency assurance, locating appropriate computing resources for processing the photos, etc.).

NOTE 2: it is assumed that the core network is able to obtain the computing resource availability in core network, operator managed data network, edge or central cloud.

3.	If the rendering task split function decides to upload the photo to the cloud, the raw data for the photo is compressed to around 150Mbits (based on the assumption of 6 frames × 4K raw data and compressed ratio assumption in [20] [21]) and uploaded to the cloud within around 1.5s and downloaded the processed photo from the cloud. The UE can reduce the number of frames for uploading based on communication link capability. Therefore, the required uplink data rate in radio layer would be 150Mbits/1.5s=100Mbps.

NOTE 2: E2E latency of 3s is assumed based on users' patience statistics as shown in [22], where 1.5s is allocated for uploading to the cloud while 1.5s for processing in the cloud and downloading to the UE.

4.	If the rendering task split function decides not to upload the photo to the cloud, the local post-processing will be performed.

5.	When the user A views the photos, the downloaded photo or the local enhanced photo can be shown to the user.

Synergized gaming enhancement case:

1.	On the train back home to celebrate Chinese New Year, User A starts a delay-sensitive FPS (first-person shooter) game displayed on XR glasses with friends online, where the gaming APP at the connected mobile phone produces the scene metadata, including 3D objects, lighting data, and materials, user actions, and then encodes the scene before transmits it to the XR glasses. The typical data size after compression can be 5-20Mbits (assuming middle to high complexity 3D model including 0.35 to 2 million vertexes (200 bits per vertex) 3D models and compression ratio assumption for 3D model in [23]).

To win the battle, User A and friends must frequently exchange the status of their game roles by voice call supported by the gaming APP, all the time during the game even when network connection is poor (e.g. when the train going through the tunnel).

2.	The gaming APP at the mobile phone submits the metadata to rendering task split function inside the OS for rendering, the rendering task split function can seek to utilize the more powerful computing in the cloud for higher quality such as ray tracing effect. The rendering task split function determines how to split the metadata to the cloud by considering some factors such as potential delay and mobile phone power consumption for data transmission. For example, the mobile phone selects to offload the slow changed scene background rendering to the cloud while performing rendering for fast moving objects using local rendering resources.

The rendering task split function further coordinates with the core network to check whether the rendering requirements can be fulfilled, and sends an offloading request to the core network if all conditions are met.

3.	The mobile phone uploads the metadata allocated for the cloud rendering within 50ms [24] and download the rendering result.

4.	The mobile phone combines the local rendered result and cloud rendered result to the user A.

5.	Once the radio condition goes worse or the rendering resource is not available, the full local rendering can be activated to maintain user experience.

### 9.2.4	Post-conditions

User A continues enjoying high quality photo taking and high-quality gaming experience.

### 9.2.5	Existing features partly or fully covering the use case functionality

The split rendering architectures defined in TR 26.928 [50] and TS 26.565 [51], and the corresponding QoS requirements in TS 22.261 [14], allow a UE to offload rendering requirement to the cloud for XR rendering.  These architectures assume that the metadata including 3D models of the gaming and APP logic is pre-installed in the cloud, therefore only user pose information is needed to be transferred in the uplink and there is no dynamic offload decision on whether to render a specific scene in the cloud or UE local resources (i.e. fully rely on the rendering result in the cloud). Therefore, the uplink requirement and dynamic offload decision requirement are not covered by the existing split rendering use cases and part of the downlink requirements on rendering result delivery are covered by the existing split rendering use cases.

It is worth noticing that Edge Enabler Client (EEC) is defined in TS 23.558 [52], which provides supporting functions needed for Application Clients. There is some similarity between EEC and the “rendering task split function” described in this use case. In this use case, instead of communicating with the Edge Enabler Server (EES) in the DN that only manages the edge applications, the rendering task split function coordinates with the core network for splitting/offloading rendering tasks.

In TS 22.261 clause 7.6.1, there is requirement for 5G system to support interactive task completion during voice conversation: the 5G system shall support low-delay speech coding for interactive conversational services (100ms, one-way mouth-to-ear).

### 9.2.6	Potential New Requirements needed to support the use case

[PR-9.2.6-1] Subject to user consent, the 6G system shall support burst type QoS with the KPI requirements summarized below：

| Use cases | Burst size (note 1) | Max Allowed latency for a burst (note 2) | Service bit rate: user-experienced data rate | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement | UL: [150Mbits] | UL: [1500ms] | UL: [100Mbit/s] | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement | UL: [5-20Mbits] | UL: [50ms] | UL: [100–400Mbit/s] | Stationary or Pedestrian | Countrywide |
| NOTE 1: assuming 6 x 4K raw pictures and compression ratio in [21] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [23] for gaming enhancement. NOTE 2: 1500ms is derived from the E2E latency of 3s (based on users' patience statistics as shown in [22]) and 1.5s for processing in the cloud and downloading. 50ms uplink latency is derived from [24]. |  |  |  |  |  |

[PR-9.2.6-2] The 6G system shall provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real-time network status and the computing resource availability in the core network, operator managed data network, edge or central cloud.

NOTE 1:   in the case of tethered XR headset connected to a UE, the latency, computing resource consumption and power consumption (incurred by application processing and communication processing) of both the UE and the XR Glasses need to be also considered.

NOTE 2:  it is assumed that the core network is able to obtain the information of computing resource availability in core network, operator managed data network, edge or central cloud, which enables the core network to locate appropriate computing resources for the rendering tasks.

[PR-9.2.6-3] Subject to operator policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed throughput (current and/or predicted, within the boundary of the 6G system) of the associated ongoing session.

[PR-9.2.6-4]: The 6G system shall be able to provide deterministic user experience for multi-party call with the KPI requirements summarized below:

| Use case | Max. mouth-to-ear delay | Audio-video synchronisation thresholds | Max. duration of consecutive packet losses | Throughput (UL and DL) | Availability | UE speed |
| --- | --- | --- | --- | --- | --- | --- |
| multi-party call | 100ms (note 1) | - in the range of [125 ms to 5 ms] for audio delayed - in the range of [45 ms to 5 ms] for audio advanced (note 2) | 100 ms (note 3) | >=30Mbps (note 4) | 99% (note 5) | up to [500km/h] (note 6) |
| NOTE 1: one-way delay [9.2x2]. NOTE 2: as defined in TS 22.261 [14] clause 7.6.1. NOTE 3: it is referring to the capable of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [9.2x1]. NOTE 4: it is derived based on 4K 60fps video encoded with HEVC [9.2x3]. NOTE 5: it means the probability to provide the above KPIs during the time that a user intends to use the above services. NOTE 6: it is to consider the high-speed train scenario as in TS 22.261 [14] clause 7.1, which is intended as a targeted value and not a strict requirement. |  |  |  |  |  |  |

* * * Second Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[9.2x1] 	https://cloud.tencent.com/developer/article/1654264

[9.2x2] 	ITU G.114: One-way transmission time

[9.2x3]	S4aV200634: "Traces and Configurations for VR2, CG and AR2"

* * * Third Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GPU				Graphics Processing Unit