3GPP TSG SA WG 1 Meeting #109 	S1-250673

Athens, Greece, 17-21 February 2025	revision of S1-250119

Title:	Use Case On Glasses-free 3D Communication

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda Item:	8.1.5

Document for:	Approval

Source:	China Mobile

Contact:	Shenxiu Du, dushenxiu@chinamobile.com

Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This document proposes a new use case on glasses-free 3D communication.

FIRST CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

......

[x]	ITU-T Recommendation G.114-0 (5/2003): "One-way transmission time"

[y]	3GPP TS 26.925: "Typical traffic characteristics of media services on 3GPP networks".

SECOND CHANGE (all new text)

9.x	Use case on glasses-free 3D communication

9.x.1	Description

As a video display effect enhancement technology, glasses-free 3D technology can be widely applied in video display scenarios, which also means that glasses-free 3D technology has broad application prospects in the field of communications. At present, various video communication services such as video telephony, video conference, video CAT and Video CRS already have a large user scale. Introducing glasses-free 3D technology into the 6G system and making it one of the characteristic functions of 6G will effectively enhance the video display effect of video services and give users a brand-new usage experience.

Glasses-free 3D technology generally refers to a display technology in which users can directly see 3D videos on a display device without wearing wearable devices. The essence of glasses-free 3D technology is that the display device utilizes optical technologies such as grating technology to make the user's two eyes see two different pictures respectively. If these two pictures are from different angles of the same object, the brain of person will mistake them for three-dimensional objects, thus achieving the 3D display effect.

Since the display device needs to display two images simultaneously in one frame, and the physical hardware such as resolution and grating arrangement varies among different display devices, when generating the glasses-free 3D video stream, the two images cannot be directly arranged well in the same frame. Instead, the two images must be completely separated, for example, placed side by side (left-right forma) or one above the other (top-bottom format). Then, after receiving the glasses-free 3D video stream, the display device analyzes it and superimposes it into a glasses-free 3D video based on the physical hardware requirements of the device itself.

Figure 9.x.1-1  Left-right format:The images of the left eye and the right eye are transmitted in the same frame, and the horizontal resolution is halved.

Figure 9.x.1-2  Top-bottom format:The images of the left eye and the right eye are transmitted in the same frame, and the vertical resolution is halved.

Glasses-free 3D communication refers to the application of glasses-free 3D technology in video communication services, providing enhancement of visual display effects for video communication services such as video telephony, video customized alerting tones, video conferences, etc. Users can directly view images, videos, or other information content with a stereoscopic visual effect without wearing auxiliary devices, offering users an immersive communication experience with a sense of depth and realism.

In the glasses-free 3D communication scenario, the terminal will be one of the main display devices. In this scenario, the terminal should first report its supported glasses-free 3D capabilities to the network side. After the terminal and the network complete the signaling negotiation and media negotiation for glasses-free 3D communication, the network will send the glasses-free 3D video stream in the corresponding format according to the terminal's capabilities. If the video stream that the network needs to send does not match the terminal's glasses-free 3D capabilities, the network can use AI technology to perform additional processing on the video stream, such as converting a 2D video stream into a 3D video stream. After the terminal obtains the glasses-free 3D video stream, the application layer will decode and reconstruct the video stream according to the requirements of the physical properties of its screen.

In addition, the glasses-free 3D video decoding and reconstruction capabilities provided by the application layer are scalable, and the AI technology used by the network side to convert video streams into glasses-free 3D ones is also scalable. If there are more formats of glasses-free 3D video streams, both the terminal and the network side can be expanded to support the new formats.

9.x.2	Pre-conditions

1. UEA supports decoding and playing the glasses-free 3D video.

2. UEB does not support glasses-free 3D communication.

9.x.3	Service Flows

- UEA request to establish a glasses-free 3D video telephony with UEB.

- UEA reports to the 6G system that only glasses-free 3D video in left-right format is supported .

- The 6G system conducts glasses-free 3D video media negotiation with UEB.

- UEB reports to the 6G system that  glasses-free 3D communication is not supported.

- UEB answers the video telephony.

- 6G system converts the video stream from UEB into a glasses-free 3D video stream in left-right format with the help of AI technology.

- The 6G system sends the converted glasses-free 3D video stream to UEA.

- UEA receives the glasses-free 3D video stream and decodes and plays the stream.

9.x.4	Post-conditions

User A can see glasses-free 3D videos on UEA without the help of any wearable device.

9.x.5	Existing features partly or fully covering the use case functionality

Take the glasses-free 3D video telephony as an example. Essentially, it is an enhancement of the visual display in video telephony. Although the 6G network may need to handle the conversion from 2D video stream to 3D video stream, for users, the experience of glasses - free 3D video telephony they perceive should not be inferior to the video telephony quality that the 5G network can offer. Meanwhile, glasses - free 3D video telephony requires the network to provide a video quality of no less than Full HD to guarantee users' visual experience. Therefore, the KPIs for glasses - free 3D video telephony could refer to the relevant indicators defined in ITU-T G.114 [x], TS 26.925 [y], etc.

9.x.6	Potential New Requirements needed to support the use case

[PR 9.X.6-1] Subject to user consent and operator policy, the 6G network shall support glasses-free 3D communication for devices that support glasses-free 3D communication, e.g. glasses-free 3D video telephony, glasses-free 3D customized alerting tones, etc.

[PR 9.X.6-2] Subject to user consent and operator policy, the 6G network shall support glasses-free 3D communication between devices that support glasses-free 3D communication and devices that does not support glasses-free 3D communication.

[PR 9.X.6-3] The 6G system shall support the following KPIs.

NOTE: 	The coding formats of glasses-free 3D video streams are identical to those of the 2D video streams.And  although glasses-free 3D communication may introduce new latency in the 6G network, the service experience provided by glasses-free 3D communication should not be inferior to that of 2D communication in the same scenario. Hence,  for the 6G network, the KPI of the glasses-free 3D video telephony is consistent with those of the 2D video telephony.

| Use Cases | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed |
| Glasses-free 3D video telephony | [< 150 ms] | [3-12 Mbps] | [99.9%] | N/A | N/A | N/A | N/A | Stationary, pedestrian |

END OF CHANGES