3GPP TSG-SA WG1 Meeting #109 	S1-250730

Athens, Greece, 17-21 February 2025	(revision of S1-250168)

Source:	CATT

pCR Title:	Use Case  on Intelligent UAV Swarms

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes a use case about 3GPP 6G system leading the collaboration of the computing and communication for intelligent UAV swarms.

1. Introduction

The 5G system has supported the AI data transmission between UAVs via direct network connection, and between a UAV and edge servers. However, using UAV swarms for executing a complex task raises additional demands in dynamic task allocation, load balancing and formulation control, which not only requests basic communication services, but also request the coordination of multiple UAVs during the communication services, as well as the assistance of the network resources to meet the requested QoS, which is not discussed before and can be considered in 6G. This contribution proposes a use case about 6G system deeply involved in the coordination of the computing and communication for intelligent UAV swarms.

2. Reason for Change

Introduce a new use case for the application of 6G system in UAV communication.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.1.1

* * * First Change * * * *

6.X	Use case on Intelligent UAV Swarms

### 6.X.1	Description

UAVs, with their agility and rapid deployment, have become indispensable tools in public safety scenarios such as earthquakes, fires, and floods. Equipped with cameras, sensors, and lightweight computing devices, UAVs can acquire real-time images and environmental data of the disaster areas and execute the tasks scheduled by the local or remote control system, such as surveillance, monitoring, and detecting target object in specific locations. As the application of AI, those intelligent UAVs with AI capabilities (e.g. with AI models, AI Agent) can be more powerful in perception, decision-making and control, and can perform more complex tasks such as initial target recognition and dynamic path planning, etc.

The data collected by a single UAV is inherently limited, and the situation gets worse when the disaster area is large. Therefore, it has become a trend to use UAV swarms to achieve full coverage on the disaster area and execute complex tasks together with mutual collaboration. However, the coordinated operation of multiple UAVs working in swarms raises challenges to the whole UAV control system. For instance, the capabilities of compute, AI, and energy endurance vary among UAVs of a swarm, so that they may have diverse abilities in data processing locally. Additionally, each UAV will face differentiated wireless condition at its location, which may impact its achievable transmission QoS. Therefore, when the control platform assigns a task to the UAV swarm, the worst UAV will be the bottleneck of the overall performance, which may be experienced an overloaded local computing. For the UAV swarms, besides of the legacy communication service, sensing service, the 6G system is expected to assist the control platform or UAVs to coordinate the computing, AI services and communication to achieve the expected performance efficiently.

When certain UAV is unable to complete the task with expected QoS due to the limited AI or computing capabilities, the coordinated computing service can help a UAV to find alternative UAVs or collaborative UAVs closer to the data source or the UAV, so that to share the data via inter-UAVs or end-edge-end, and execute the tasks instead or together. On the other hand, the 6G system can leverage the edge computing servers to offload the workload of UAVs, gather the data from different types of UAVs and the environmental data sensed by the base station, deploy complex AI models, and infer comprehensive and intuitive decisions for command and rescue.

This use case illustrates how a 6G system assists the control system and UAV swarms in the coordination of computing and communication.

Figure 6.X.1-1 Coordinated Computing and Communication Services for Intelligent UAV Swarms

### 6.X.2	Pre-conditions

UAV#A, #B, and #C are equipped with cameras and sensors for images and environmental data collection and processing components with limited computing capability while UAV#D is equipped with high-performance processing components, and AI models for thermal image recognition.

Edge server #E is provisioned the AI model for the generation of high-definition 3D maps.

All the UAVs are deployed around the disaster area, and have the subscription for communication, sensing, and the coordinated computing services.

All the UAVs have been registered in the 6G network with the capabilities.

### 6.X.3	Service Flows

- The control system receives messages indicating that people are trapped in the damaged library, and immediately initiates a mission of search for the trapped people and sends it to the nearby UAV #A.

- UAV #A analyzes the mission and identifies the lack of AI models for thermal image recognition, so it sends a request to 6G core network for coordinating computing resources for AI interference.

- The 6G core network select UAV #D based on the request and the information of the computing resources (e.g. computing capability, AI capability, power consumption, location), and informs UAV #A and UAV #D of the coordination result.

- UAV #A discovers UAV #D and sets up the connection with UAV #D to transmit the collected images and environment data.

- UAV #D infers the detection of trapped people based on local AI models for thermal image recognition and the data provided by UAV #A. It then feedback the inference result to UAV #A.

- UAV #A feedbacks the presence, number and rough location information of the trapped people to the control system.

- The control system formulates the rescue plan and initiates a mission to acquire the precise 3D location of the trapped people and send it to UAV #A.

- UAV #A analyzes the mission and identifies the lack of the data within a certain height, and the lack of AI models for the generation of high-definition 3D map, so it sends the request to 6G core network for coordinating computing resources for AI interference, and discovers other UAVs covering the target 3D location.

- The 6G core network select MEC #E based on the request and the information of the computing resource (e.g. computing capability, AI capability, power consumption, location), and informs UAV #A and MEC #E of the coordination result.

- UAV#A sets up a connection to MEC #E to transmit the collected images and environment data. Meanwhile, it informs UAV#B and UAV#C to transmit data to MEC #E.

- MEC #E gathers the data from UAV #A, #B, and #C, generates high-resolution 3D maps for the trapped people and then sends the 3D map to UAV#A.

- UAV #A feedbacks the precise location with 3D map to the control system.

6.X.4	Post-conditions

The rescue teams find and rescue the trapped people based on the precise location and 3D maps.

### 6.X.5	Existing features partly or fully covering the use case functionality

In TS 22.261 [x] clause 6.40.2.2,

Based on user consent, operator policy and trusted 3rd party request, the 5G system shall support a means to authorize specific UEs to transmit data (e.g. AI-ML model data for a specific application,) via direct device connection in a certain location and time.

### 6.X.6	Potential New Requirements needed to support the use case

[PR-6.x.6-1] Subject to the user consent, the 6G system shall support mechanisms to acquire and maintain the information of trusted computing resources (i.e. authorized UEs or edge servers) for coordinating the usage of computing resources on the UE’s request.

[PR-6.x.6-2] Subject to operator’s policy, the 6G system shall be able to select computing resource(s) (i.e. authorized UE(s) or edge server(s)) based on UE’s request and route the data traffic efficiently between the UE and selected computing resources to meet the required aggregated QoS.

[PR-6.x.6-3] Subject to operator’s policy, the 6G system shall support mechanisms to monitor the selected computing resource(s) (i.e. authorized UEs or edge server(s)) and reselect the computing resource(s) in case of events (e.g. when the selected UE is deregistered, when the selected edge server is overloaded).

[PR-6.x.6-4] The 6G system shall be able to collect charging information for the computing resource(s) coordinated by the 6G network e.g. per request, per UE, or per application.

* * * End of Changes * * * *