3GPP TSG-SA WG1 Meeting #109 	S1-250962

Athens, Greece, 17-21 February, 2025	(was S1-250176)

Title:	Use case on Critical Immersive Communications

Agenda Item:	8.1.5 (6G study - Immersive Reality)

Source:	Samsung

Contact:	DongEun Suh <dongeun.suh@samsung.com>

Abstract: This paper presents a use case on critical immersive communications

Discussion

The contribution proposes a use case on critical immersive communications where packets from independent application traffic flows are required to be delivered in a timely manner.

Proposal

It is proposed to make the change listed below to TR 22.870.

Begin changes

9.x	Use case on Multiple Application Media Synchronization

9.x.1	Description

In critical immersive communications such as 3D remotely controlled repairs or surgery, users may be equipped with multiple devices for multiple media components (e.g., haptic device for pressure, VR glasses for video, wireless headphones for audio) and each device receives traffic for the corresponding media component from the networks.

Unless the user receives synchronized information from all application media flows involved in the remotely controlled activity – in this example, presented to the user through haptic gloves and headset – the user can easily make errors, e.g. destroying the equipment that is being remotely repaired, or harming the patient by remote surgical procedures.

Figure 9.x.1-1 An example of remotely controlled repair.

Then, the tighter synchronization between different media components is required. For example, a group of data carrying multiple media components is defined as a chunk of haptic data and chunk of video data and the group of data need to be delivered collectively within a required time window.

Also, the group of data including more than one media components can be delivered via more than one traffic flows and/or UEs when each device is receiving corresponding media component via specific flow.

In a case when the network condition is poor or varying, it would be hard to support this level of synchronization without the network to understand the inter-dependent packets across multiple flows.

The corresponding 5G requirements from TS 22.261 below is defined for the multi-modality but it does not cover the above mentioned scenario.

-The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

-The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

In this use case there are different distinct application flows that must be synchronized, whereas in existing requirements, the media for a single application is synchronized.

9.x.2	Potential New Requirements needed to support the use case

[PR.9.x.2-1]	Subject to operator policy, the 6G system shall support the synchronization of independent application traffic flows, to be delivered to more than one device (i.e., UE or tethered devices).

Editor’s Note: Specific KPIs for the synchronization requirement are FFS.

End of changes