3GPP TSG-SA WG1 Meeting #109 	S1-250970

Athens, Greece, 17-21 February 2025	(revision of S1-250881 ofS1-250812 of S1-250312)

Source:	Siemens, TNO, KPN, Fraunhofer IIS, Airbus, Robert Bosch GmbH

New use case title:	New use case on Cooperating Mobile Robots

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.7

Document for:	Approval

Contact:	Michael Bahr (bahr et siemens dod com)

Abstract: This use case proposes to address Industrial Communication between collaborative robots over 6G networks. It is proposed to be added to the “Further Use Cases on Industry and Verticals” clause in the 6G TR 22.870.

---------- First Change ----------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: " 	Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[a]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains"

[b]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025

---------- Next Change: New Text ----------

## 11.x	Use Case on Cooperating Mobile Robots

### 11.x.1	Description

At the centre of this use case are autonomous robots with the ability to move, sense their environment, and perform a productive task collaboratively. These robots can communicate with one another, with other machines, and with nearby humans to perform individual tasks that contribute to a common cooperative objective. The purpose of communication is safety and cooperation, to enable a group of robots to perform tasks beyond their individual capabilities and to enable individual robots to perceive their environment beyond their local capabilities.

This use case focuses primarily on local ad hoc connectivity embedded in private networks. Scenarios may include industrial manufacturing campus, construction site, and smart living.

In this context, the network of the future is envisioned to enable local cooperation among robots, to support autonomous task-solving by the robots, to enhance the safety of human-machine interaction, and to ensure that only authorized machines and humans can participate in task solving.

Figure 11.x.1-1: Cooperating Mobile Robots Use Case, highlighting two example scenarios

Scenarios revolve around the need to complete a task that surpasses the capabilities of a single robot. The solution lies in robot cooperation, where multiple robots form a cluster to collaboratively accomplish the task. These robots may utilize local ad hoc connectivity for

coordinating and controlling their actions for successful task execution,

exchanging raw or processed sensor data,

sharing AI/ML models, weights, and federated learning, as well as

ensuring safety for humans nearby in the event of an emergency.

Autonomous mobile robots (AMRs) collaborate for a specific task in a spatially confined area for a limited time. Such a group of AMRs needs to communicate with each other, requiring periodic-deterministic communication (time synchronization, high reliability and very low latency) for their collaborative industrial control. Characteristics of the scenarios need to be leveraged, for instance, such as no relative mobile between AMRs moving in unison and potential line of sight when using direct device communication instead of direct network communication.

The 6G system can be an important enabler for

addressing the communication requirements of machines in the future. This includes existing industrial automation systems, as well as the emerging type of autonomous embodied agents that encompasses cooperating industrial robots, service robots, and cobots

Potential sustainability impacts of the use case are clustered by main topic / KVI. For each KVI, a selection of key values is provided together with some background information.

Energy: i.e., focus on the energy usage for the correct operation of robots, both on communication links, as well as in the data processing, data storage, and data collection aspects

-	[  to be decreased] Total energy usage (kWh) in communication dimensions for the cobots

-	[  to be decreased] Total energy usage (kWh) for data transfer (optimising packets, and data volume)

Selected background information:

-	Potential benefits: Resource efficiency: Functionalities may be provided by machines with less materials, energy, and waste generated

-	Potential areas of attention (risks to be mitigated): Energy is consumed and materials are used to manufacture, deploy, and operate cobots and associated services.

Materials/Waste:

-	[  to be increased] Life expectancy of robots: making better and longer-lasting hardware and/or software, easily upgradable, modular. Avoid having to replace the entire cobot because of failures in the 6G-related aspects

-	[  to be increased] # of virtualised functionalities: Preventing or avoiding material usage for dedicated hardware, when these features / or capabilities could be virtualised.

Selected background information:

-	Potential areas of attention (risks to be mitigated): The disposal of machines and devices results in increased electronic waste.

Safety:

-	[  to be decreased] # of injuries at work/level of severity of work-related injuries/perception of risk: Aimed at ensuring the system will be reliable and thus, no accidents will be produced because of failures in the communication links, or misinterpretation of data in the AI/ML algorithms/Sensing/Positioning/etc.

Selected background information:

-	Potential benefits: Safer work environment leading to less injuries.

Trustworthiness/Privacy/Security:

-	[  to be increased] Level of acceptance of cobots by humans: Ensuring that the appropriate privacy and security requirements are in place, that no personal data is at risk, that cobots will be resilient and responsive in case of hacking, etc. Overall, ensuring they are trustworthy and accepted.

-	[  to be decreased] # of data leaks/breaches/cyber attacks, with personal information compromised: Measuring the impact of the above KVI.

Selected background information:

-	Potential benefits: Increased autonomy by robots supporting people with disabilities.

-	Potential areas of attention (risks to be mitigated): People’s privacy may be breached by unauthorized use of robots’ and cobots’ sensors.

Productivity/Efficiency: Ensure that 6G is reliable and resilient, and so, no downtimes in a given production scenario are due to failures in the communication-related aspects.

-	[  to be decreased] Downtimes.

-	[  to be increased] Production KPIs (distinctive per factory).

Selected background information:

-	Potential benefits: Increased local and global productivity, cost efficiency and enhanced competitiveness from the use of the collaborative robots.

### 11.x.2	Pre-conditions

The 6G network is dedicated to a specific purpose and to private use, e.g., manufacturing, construction, farming, living, and similar.

The 6G network is located in a well-confined geographical area such as factories, industrial sites, enterprises, construction sites, or fields. In several scenarios, functionality and data storage are on-premise.

In outdoor scenarios, wireless 6G communication for wide area connectivity is available.

Electrical power is generally available. Power efficiency is assumed for battery-driven devices like mobile robots.

### 11.x.3	Service Flows

#### 11.x.3.1	Scenario: Cooperative Carrying With Mobile Robots

Cooperative carrying with mobile robots is a concept where multiple autonomous mobile robots (AMRs) work together to transport an object that exceeds the carrying capacity of a single robot (see also Figure 11.x.1-1). The special focus is on collaboration between several AMRs, deterministic industrial communication between AMRs, including direct device connectivity (time synchronization, highly reliable and low latency communication).

- The AMRs gather around the object to be carried collaboratively. They are in close proximity of each other.

- One of the AMRs initiates the communication group of related UEs, the other AMRs join this group. Communication resources are properly allocated for providing the AMRs with the required level of QoS.

- At different stages of the service flow, the AMRs may communicate with e.g. digital twin applications or AI/ML compute servers in the 6G network.

- Industrial communication with time synchronization and deterministic highly reliable and low latency communicaton is used between the collaborating AMRs. The collaborative robots exchange sensor data and industrial control information with each other for cooperative carrying.

- The collaborating AMRs move in unison as a group. They have sensing capabilities as well as AI capabilities for task planning and autonomous operation.

- After placing the object at its final destination, the AMRs leave the communication group of related UEs.

The special focus of this scenario is on collaboration between several AMRs, deterministic industrial communication between AMRs in a moving subnetwork (time synchronization, highly reliable and low latency communication).

#### 11.x.3.2	Scenario: Autonomous Construction Site

On an autonomous construction site, different types of robots work collaboratively in unison to construct a building (see also Figure 11.x.1-1). The transportation of materials, such as bricks, must seamlessly integrate with advanced construction machinery like 3D printers. Other robots are responsible for post-processing tasks, such as drilling holes autonomously. Humans interact on-site and remotely, for instance, with digital twins and video cameras for virtual guidance and for remote control of construction machines.

This scenario uses:

- Industrial communication for the exchange of industrial control information (time synchronization, deterministic URLLC communicaton).

- sensing and localization capabilities for, e.g., positioning, location services (absolute and relative), collision avoidance, and detection of human presence, using various sources such as 6G devices with ICAS or HD video cameras.

- Immersive Extended Reality for local and outdoor walkthroughs of future construction stages at the construction site but also in remote offices. Virtual guidance for complex construction aspects for previews and better understanding, remote participation of stakeholders.

- Support for digital twins of the buildung that provide detailed information about all project stages.

- Support for artificial intelligence applications on autonomous construction machinery.

### 11.x.4	Post-conditions

The cooperating mobile robots finished the assigned task(s) collaboratively. They will continue to the next task to be done collaboratively.

### 11.x.5	Existing features partly or fully covering the use case functionality

Direct device connection: TS 22.104 [a] clause 7.2 provides service requirements on direct device connection for cyber-physical control applications. In TS 22.104 [a] clause 7.2.1, however, the use of direct device connection is bound to certain conditions (not being served by a RAN, use of different spectrum).

Non-Public Networks: TS 22.261 [14] clause 6.25.2 provides requirements on non-public networks.

Energy efficiency: Energy efficiency is a general requirement on wireless communication. However, energy saving in the 6G network must not endanger successful, efficient, and high-quality industrial manufacturing.

TS 22.104 [a] clause 7.2.2 provides service requirements to support direct device connection between a group of UEs for periodic deterministic communication (both unicast and multicast) with respective service performance requirements in Table 5.2-1 related to cooperative carrying. These service performance requirements cover also the service performance requirments of this use case expect increased localized density and increased UE speed. Also the mobility of the group of UEs is covered in this clause.

TS 22.104 [a] clause 7.2.3.2 provides service requirements for clock synchronization over direct device connection.

TS 22.104 [a] Clause 7.2.4 requires service continuity only between an NPN and a PLMN, but not the general case.

TS 22.261 [14] Table 7.3.2.2-1 defines performance requirements for horizontal and vertical positioning. While the required speed would be covered by every positioning service level, the necessary accuracies are only covered partially (1 m) or not at all (0.1 m).

### 11.x.6	Potential New Requirements needed to support the use case

[PR11.x.6.1] The 6G System shall be able to support the following performance requirements to support Cooperating Mobile Robots

Table 11.x.6-1: Performance Requirements for Cooperating Mobile Robots use case [b]

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (ms) | Application data rate (Mb/s) | Reliability (%) | Message size (byte) | Service area (m²) | Transfer interval (ms) | Connection density [devices/m2] | UE speed (km/h) |  |
| (2) Cooperating Mobile Robots – 6G Network Services | [1 to 10] | [<10] [<250] (note 2) | [99.999 to 99.99999] (note 4) | (note 3) | [20 up to 1,000,000] (note 5) | [≤ 10] | [≤ 0.5] (note 1) | [< 20] | Robot to Network for services such as digital twin applications, AI/ML compute servers. |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: The higher data rate applies when e.g., immersive XR, AI/ML traffic, digital twin data, is exchanged. Otherwise the lower data rate applies. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real-time industrial processes requires very high service reliability. Can be lower for non-real-time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario and size of location/collaboration area (collaborative carrying mobile robots (11.x.3.1), factory, construction site (11.x.3.5), field (11.x.3.4)) |  |  |  |  |  |  |  |  |  |

[PR11.x.6.2] The 6G System shall be able to support the following positioning performance requirements to support Cooperating Mobile Robots

Table 11.x.6-2: Positioning Performance Requirements for Cooperating Mobile Robots use case [b]

| Use Cases | Positioning accuracy (m) | Velocity (km/h) | Remarks |
| --- | --- | --- | --- |
| Cooperating Mobile Robots | [<0.1] [<1] | [< 20] | Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. Tasks like robot localization can work with less accurate positioning. |
| Human presence detection | [0.4] | Pedestrian | [40 cm] resolution within an area defined by [1 m] range around the robot |