3GPP TSG-SA WG1 Meeting #110 	S1-252255

Fukuoka, Japan, 19-23 May 2024	(revision of S1-252xxx)

Title:	TR 22.870 pCR on UE Radio Status Monitoring for Availability

Agenda Item:	8.1.2 (FS_6G-REQ – System and Operation Aspects)

Source:	Samsung, EUTC, Ministère d’économie et des finances, DSIT, NIST, 
	SyncTechno, FirstNet, BMWK

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This use case seeks to add 6G to achieve service objectives that led to agreed but not supported requirements for Smart Energy Infrastructure (SEI).

Introduction

In Release 18, the FS_5GSEI  study (TR 22.867) included use case 5.7 Remote DSO management of connectivity for Smart Energy. To summarize, the use case considered what would help an energy utility Distribution Service Operator (DSO) to improve the availability of their service: delivering energy to end consumers. The resulting stage 1 feature, SEI, included several requirements that were addressed in downstream work, including the SA5 feature Network Service and Operations for Energy Utilities (NSOEU).

One particular key requirement, however, has not been addressed in downstream work. This 6G use case revisits this requirement in its essential form and seeks to add a potential requirement to TR 22.870 in a more actionable way than is present in TS 22.261.

Discussion

The requirement this use case considers in the 6G context was added in the 5G system normative standard TS 22.261 – see https://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_93E_Electronic_2021_09/Docs/SP-211070.zip in 22.261 CR 0547:

The 5G system shall provide a means by which an MNO can inform authorised 3rd parties of changes in the

-    RAT type that is serving a UE;

-    cell ID;

-    RAN quality of signal information;

-    assigned frequency band.

This information listed above shall be provided with a suitable frequency via OAM and/or 5G core network.

The purpose of this requirement, to summarize, is to allow the energy utility DSO to monitor critical factors of their communication service to identify signs of imminent failure. If the signal strength diminishes consistently beyond a certain point, or the assigned frequency band is unexpected and indicative of abnormal service, there is a heightened likelihood of communication service outage. The observation of significant RAN quality of signal degradation and unexpected change in assigned frequency band has been observed by DSOs to be strongly correlated with communication service availability failure.

Rather than wait until this outage occurs, the DSO can direct their equipment in a substation, essentially a router to activate a secondary or alternative communication access. For example, the router’s mobile wireless access interface, a UE, can initiate registration of a second USIM to an alternative PLMN, or the router could initialize a VSAT session via satellite access, or take some other action to obtain communication service.

The highlighted two information elements were not exposed by any standard for Rel-18, Rel-19 nor will they be in Rel-20 (5G-Advanced.) This is unfortunate because these are actually the most significant of the four elements in terms of predicting failure of communication service availability.

It was hoped that the network could expose this information. On study of the support for network analytics and prediction exposure from the 5G system, it was observed that this functionality is quite different than monitoring. Though the NWDAF has access to the assigned frequency and signal strength of a UE, this information is not exposed. There are good reasons for this: The information could be sensitive to the operator. Second, monitoring specific values associated with a UE session and periodically reporting its value is not consistent with other information exposure to 3rd parties performed by the NWDAF – rather abstract ‘analytics’ and ‘predications’ are exposed.

Effectively, the current capabilities of the 5G system do not include a ‘performance monitoring’ interface to third parties for RAN aspects that are at the granularity of single UEs.

For energy utilities, high level information rather than the data itself (e.g. analytics and predictions) is much worse than what is possible today. DSOs monitor the information directly from the substation router, including information known to the UE. The UE itself has access to the values (assigned frequency band, signal strength). These values can be obtained by higher levels (in the router), and sent periodically ‘over the top’ to the DSO’s operation center. There are two problems with this ‘solution’ however:

1/ it can only be sent very seldom, or else the communication channel will fill with such monitoring data.

2/ if there is indeed an issue, and the UE loses access, it is impossible to send the information over-the-top.

Due to the low granularity of data collection and the loss of data just when it is needed most, DSOs seek to get this information directly from the MNO – who has it but at this point does not expose it.

References in discussion section

[1] 3GPP TR 22.867 “Study on 5G Smart Energy and Infrastructure”.

[2] 3GPP TS 22.261 “Service requirements for the 5G system”.

Proposal

It is proposed to make the change listed below to TR 22.870, 0.2.1.

Begin changes

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DSO	Distribution Service Operator

DT	Digital Twin

GPU	Graphics Processing Unit

LLM	Large Language Model

LMM	Large Multimodal Model

M-IoT        	Massive Internet of Things

NDT	Network Digital Twin

NLP	Natural Language Processing

PII	Personal Identifiable Information

RAG	Retrieval-Augmented Generation

SLM	Small Language Model

UAM	Urban Air Mobility

UAV	Uncrewed Aerial Vehicle

Next change

## 11.X	Use Case on Monitoring UE Radio Characteristics

### 11.X.1	Description

The availability of the telecommunication system depends on the energy service. Though autonomous power supply does exist, this cannot sustain service through a prolonged energy service outage. The stability and availability of energy distribution service depends increasingly on mobile telecommunications, as operations and management ‘smart energy’ services are crucial. There is thus a profound inter-dependence between energy services and mobile telecommunications.

Both mobile telecommunications and energy service are of great social importance; lack of availability of either can lead to financial and other loss, as well as danger to human life and welfare. It is therefore crucial to address energy service resilience, both as it is vital to mobile telecommunications business and because of its immense role in all aspects of modern human life.

This use case revisits use case 5.7 in TR 22.867 [x] in the context of 6G. The use case is not intended to provide functionality that is general to all customers in all scenarios. It is specific to energy utility DSOs. They operate in the following scenario. In any given country, to provide power to millions of consumers, energy distribution substations are located everywhere energy is used. Each of these thousands of substations distributes energy to hundreds of consumers. These substations require substantial remote control and management to remain operational, and are often deployed in locations where fixed access is impractical.

Substation

DSO operation centre

R  UE

PLMN

Figure 11.X.1-1: Energy Distribution Substation

The Substation includes an IP network managed by the DSO operation centre. Essentially all the substations are logically part of the DSO IP network. The network requires access, here R, from the 3GPP perspective a split terminal equipment UE, has a network interface that accesses services of the PLMN. The UE’s terminal equipment is not a smart phone, but an IP router.

A UE gathers significant information about the network as part of its essential function. This includes the radio signal strength and the frequency band it operates on. It is possible for the terminal equipment to obtain this information from the UE’s mobile termination on most platforms.

High availability (greater than 99.99%, or 1.8 seconds per hour) can only be achieved in practice if risks are identified and avoided. For this reason, routers in energy utility substations, all information that could be correlated with failure is gathered routinely and analysed. This data is monitored by the DSO operation centre. Very strong correlations have been observed between lack of availability in the mobile telecommunication system following significant radio signal strength degradation and irregular use of frequency bands.

Proactive measures to activate a back-up communication access can be initiated early, before access fails. For example, a multi-USIM UE can register with an alternative PLMN when the PLMN serving it exhibits significant radio signal strength degradation. In this way, communication service availability can be achieved for DSO substations. Reactive measures, e.g. selecting and registering with a PLMN after communication loss, are too time consuming to meet high service availability requirements.

While the radio signal strength and frequency band used by the UE can be monitored locally, on the UE, and communicated to the DSO operation centre ‘over the top,’ there are two fundamental problems with this. First, due to the demand for essential communication for energy service operations, measurements can only be sent infrequently, e.g. once a minute. Second, if the communication service is no longer available, data cannot be sent ‘over the top’. It is therefore impossible for the DSO Operation Centre to determine if the cause of communication loss is e.g. radio signal strength.

### 11.X.2	Pre-conditions

An energy utility has deployed thousands of energy substations. These substations use mobile telecommunications for wireless communications for smart energy services critical to achieving high availability energy distribution service to energy consumers. Each of these substations has deployed a UE that provides networking communication for the customer premises equipment in the substation. This UE supports multiple USIMs but not simultaneous communication on multiple PLMNs.

The subscriptions for these UEs includes the possibility to monitor radio characteristics: this is not a general capability shared by all subscriptions.

### 11.X.3	Service Flows

The energy utility, E, an authorized third party, requests that relevant radio information including RAN quality of signal information and assigned frequency band be exposed from the MNO M, at a suitable frequency, e.g. once per 15 seconds.

### 11.X.4	Post-conditions

The information is delivered by M to E. It is possible for E, based on this information, to decide that the RAN quality of signal information has declined consistently or to an inappropriate degree, or the assigned frequency band changes unexpectedly. In this case, E can initiate communication services on the UE to an alternative PLMN. This can increase the communication service availability.

### 11.X.5	Existing features partly or fully covering the use case functionality

It is possible to determine the RAN quality of signal information and assigned frequency band by the UE (internally), and to send this information in an unspecified manner (i.e. ‘over the top’). This has deficiencies with respect to the use of the information to achieve higher availability, as described in 11.X.1.

Though the requirement listed under 11.X.6 was specified in 5G (22.261 [14], 6.10.2) this requirement was not addressed in 5G and would not satisfy the needs of this use case if the solution were based on existing support for network analytics and predictions.

### 11.X.6	Potential New Requirements needed to support the use case

[PR 11.X.6-1] Subject to operator policy, regulatory requirements and user consent, the 6G system shall provide a means by which an MNO can expose the following information, as data rather than statistical extrapolation or low precision abstract information (e.g. ‘excellent’ or ‘poor’), associated with a UE to authorised 3rd parties at a suitable frequency:

-    RAN quality of signal information;

-    assigned frequency band.

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345.

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023.[xx]	ISO/IEC 22989: "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology"

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/5g-new-calling/

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/remote-damage-assessment/

[85]	 GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Reference links for  https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/visualised-voice-calling/

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[90]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[94] 	5G-ACIA White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation”

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU G.114: One-way transmission time.

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020.

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442.

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872.

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42.

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025.

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html, U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, Jul. 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service.

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C. ”Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021.

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. Online Link.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Online Link.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. Online Link.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. Online Link.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. Online Link.

[135]	Network-Enabled Robotic and Autonomous Systems, Next G Alliance, May 2023. Online Link.

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228:  “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[143] 	Test Report from Utility Broadband Alliance “2024 UBBA Plugfest Testing Report” (https://www.ubba.com/ubba-resources/2024-ubba-plugfest-testing-report/).

[x]	3GPP TS 22.867, “Study on 5G Smart Energy and Infrastructure”.

End of changes