3GPP TSG-SA WG1 Meeting #110 	S1-252257

Fukuoka, Japan, 19-23 May 2024	(revision of S1-252xxx)

Title:	UE requirements considerations

Agenda Item:	8.1.1 (FS_6G_REQ: General)

Source:	Samsung

Document for:	Discussion, Endorsement

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: Some principles for the consideration of UE requirements are explained, motivated and proposed for endorsement.

Introduction

This document considers what a UE is and to which extent and in which ways it is standardized by 3GPP.

In SA1, many requirements target the UE. In some cases, the requirement assumes an opportunity for standardization that extends beyond the scope of 3GPP.

References in discussion section

[1]	TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	TS 24.002: “GSM - UMTS Public Land Mobile Network (PLMN) Access Reference Configuration”.

[3]	TS 27.007: “AT command set for User Equipment (UE)”.

[4]	TS 23.002: “Network architecture”.

[5]	TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”.

[6]	TS 22.226: “Global Text Telephony (GTT); Stage 1”.

Discussion

# 1. What is a UE

In 21.905 [1], a UE is defined as

User Equipment (UE): Allows a user access to network services. For the purpose of 3GPP specifications the interface between the UE and the network is the radio interface. A User Equipment can be subdivided into a number of domains, the domains being separated by reference points. Currently the User Equipment is subdivided into the UICC domain and the ME Domain. The ME Domain can further be subdivided into one or more Mobile Termination (MT) and Terminal Equipment (TE) components showing the connectivity between multiple functional groups.

Essentially, a UE is a combination of the UICC, MT and TE.

| Domain | Standardized by | Requirements’ applicability |
| --- | --- | --- |
| UICC | ETSI | USIM applications in CT6 |
| Mobile Termination (MT) | 3GPP | All (maybe not RAN3, CT3, CT4, CT6) |
| Terminal Equipment (TE) | Not standardized | Interfaces to 3GPP MT and USIM are defined by 3GPP. Application layer standards are defined in 3GPP. |

This architecture is shown by the following figure from TS 24.002 [2]:

The standards supporting R are defined in CT1 and are essentially data standards (e.g. TS 27.007 [3], etc.)

In fact one MT can terminate multiple TEs, and even allocate IP addresses amongst them using IP prefix based delegation.

The TE commonly considered is the mobile telephone, smart phone, etc. The category of TE is much broader and comprises anything that is integrated with a MT, e.g. an IoT sensor, a router, a home appliance, an automobile…

# 2. What is a Subscriber

TS 21.905 [1] defines a subscriber as

Subscriber: A Subscriber is an entity (associated with one or more users) that is engaged in a Subscription with a service provider. The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of users authorised to enjoy these services, and also to set the limits relative to the use that associated users make of these services.

This general term is further refined to certain kinds of subscriber:

Service subscriber: Entity which subscribes to the General Packet Radio Service (GPRS) service.

GPRS subscription is essentially a subscriber to ‘data service’, the TE is allocated with an IP address when it uses this service.

MTC Subscriber: A MTC Subscriber is a subscriber, i.e. a legal entity having a contractual relationship with the network operator to provide service to one or more MTC Devices.

NOTE:	Typically a M2M service provider is the party holding subscriptions in order to provide connectivity between MTC Devices and the MTC Server. In practise certain roles can collapse, e.g. the network operator acts as the same time as Service Provider.

What is interesting here is that the subscriber is not necessarily the same as entity as a person at the place where the MTC Device is, who is using the MTC Device, etc.

There is a concept of a ‘mobile subscriber’ but no definition of this in TS 21.905. The International Mobile Subscriber Identity (IMSI) is the identity of this subscriber.

TS 23.002 [4] defines over a hundred ways that the subscriber is provided with services. Policies are related to subscriptions and used in many ways by different functions of the 3GPP system. It is not however defined how subscription policies are created, modified, etc. This is out of 3GPP scope, and occurs at a business support system level.

# 3. What is a User

Work is done on the User’s behalf but not because the User directly interacts with the 3GPP system. There are only a few teleservices defined in 3GPP that include user interaction.

In 23.002 [4], there is only a single mention of a ‘User’ doing or activating anything: initiating calls (4.1.2.1.1, the MSC Server “terminates the user-network signalling and translates it into the relevant network-network signalling.” The User dials a phone number, hangs up, etc.)

In contrast, 22.280 [5] user interaction is extensively defined. A MCX Terminal Equipment has many requirements, as a result. This service requirements specification is exceptional amongst 3GPP specifications, however. Only a few other 22 series specifications include requirements for support of a User, e.g. TS 22.226 [6] GTT for text based telephony.

# Observations

A few requirements have been expressed in 3GPP standards either from GSMA (as business requirements, e.g. display of RAT representations such as 5G) or regulators (for interfaces for public safety, e.g. warning systems, emergency calls). Requirements of this kind are added to comply with external regulatory or business needs, not as an initiative within 3GPP.

1.	Human-machine interface requirements are out of 3GPP scope.

Some scenarios UEs do not have Users (e.g. MTC Devices). In any case, a User cannot be compelled to act in a certain way based on a standards requirement.

2.	There can be no requirement placed on the User of the TE

Subscribers are not necessarily users. In IoT and enterprise scenarios, a business often has many subscriptions for devices that are deployed in various ways, some with no user at all (e.g. appliances or routers), some with users that do are not the subscriber, e.g. enterprise employees.

3.	There can be no assumption that the subscriber interacts with the TE

There are many scenarios in which a TE is not highly integrated with the MT, for example a MT serving as a gateway or router for multiple TEs. In this case, only a minimum of information needs to be exchanged between the TE and MT, e.g. to assign the IP address for the TE. The point is that some TEs make use of a very reduced subset of the capabilities defined in TS 27.007. [3]

4.	There can be no assumption that the TE will be aware of or use of information known to the MT

# Proposal

It is proposed to agree to the following principles.

1.	Requirements on UEs shall be assumed to have requirements only on the MT or USIM, not on the TE.

2.	Any UE requirement that is not on the MT or USIM but only on the TE needs additional clarification. If the requirement is only on the TE, or any requirement that involves interaction with the User or Subscriber, this shall be explicitly stated.

If these principles are agreed, SA1 can discuss further whether and if so how to capture these principles, so they are well understood by all readers of stage 1 specifications.