3GPP TSG SA WG 1 Meeting #110 	S1-252311

Fukuoka, Japan, 19-23 May 2025

Source:	Philips International B.V.

pCR Title:	Update use case 5.5.1 on FWA

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes to update use case 5.5.1 in TR 22.870 with additional aspects related to having multiple FWA devices.

1. Introduction

Use case 5.5.1 describes the benefits of Fixed Wireless Access as a means to circumvent wired connectivity to people’s home with equivalent or even better performance.

2. Reason for Change

In order to have good wireless network performance within people’s homes a single wireless access point built into or otherwise attached to a Fixed Wireless Access device is typically not sufficient, since good wireless coverage across all areas of people’s homes is difficult to achieve in particular in higher frequency bands for Wi-Fi such as 5GHz and 6GHz.  Thus, it would be beneficial for a user to employ multiple FWA devices to increase coverage within people’s homes and e.g. backyard or in a multi-floor house, but also to provide better performance in terms of capacity, reliability, or Quality of Experience since the load can be shared amongst the multiple FWA devices. This would also align with recent Wi-Fi developments that provide support for multi-AP wherein multiple AP cooperate when serving a wireless device.

3. Proposal

Update use case 5.5.1  with additional aspects related to having multiple FWA devices, for inclusion in 3GPP TR 22.870.

* * * First Change * * * *

### 5.5.1	Use case on Fixed Wireless Access (FWA)

#### 5.5.1.1	Description

Fixed Wireless Access (FWA) in 5G NR has transformed the Home Internet business and has become a significant revenue generator for mobile network operators (MNO). It is expected that FWA services will continue to grow in 6G. While FWA has been supported since LTE, 5G's speed and flexibility has enabled it to emerge as a transformative and successful 5G service. FWA has become a key strategy in expanding high-speed broadband services to previously unserved or underserved areas (i.e., a tool to bridge the Digital Divide), for in-building services, and enterprise (i.e., private network) deployments.

Some initial deployments of FWA used "fallow capacity" in markets where enhanced Mobile Broadband (eMBB) usage was not in high demand, but the underlying network protocols, procedures, and resource management schemes of the macro networks are designed for eMBB services and Smartphone devices that require mobility support. FWA devices (e.g. Residential Gateway with cellular backhaul) have a very different mobility profile, traffic usage pattern, and are able to take advantage of higher output power improving their in-building link performance.

FWA should provide a quantitative improvement in service by using 6G services and capabilities (e.g. AI/ML).

A Customer Premise Network (CPN) may benefit of access through multiple FWA devices as means to improve coverage, capacity, and reliability. For example:

•	Using multiple RG units will be able to ensure improved coverage in the whole CPN (independently of the non-3GPP network within the customer network).

•	Using multiple RG units will be able to handle more UL/DL traffic as required in the CPN. By using load balancing between RG units, data traffic to/from multiple devices connected to the RG, in particular devices that are in coverage of both RGs, higher and more consistent throughput can be achieved.

This is illustrated by means of Fig. 5.5.1.1-1 wherein the CPN is represented by means of a home network comprising five devices D1 to D5. The five devices are served by two residential gateways (FWA devices), namely RG1 and RG2. The two residential gateways cooperate to provide connectivity to the CPN. It is possible to observe that if, e.g., RG2 were not present, the connection to, e.g., D4 and D5, would be potentially much worse (e.g., in terms of capacity or latency), in particular for multi-floor concrete homes, family homes, homes with a garden/summer house, or condos with shared/community Wi-Fi. However, the usage of multiple residential gateway units allows improving coverage, increasing capacity, improving Quality of Service, and improving the reliability of the system.

Fig. 5.5.1.1-1 – Usage of multiple residential gateway units for increased coverage, capacity, and reliability in a customer premises network

#### 5.5.1.2	Pre-conditions

Operator A offers FWA services to provide connectivity in homes.

User A subscribes to a Fixed Wireless Access of Operator A and receives a RG1 from Operator A.

#### 5.5.1.3	Service Flows

User A installs and registers his RG1 with Operator A. If not yet preconfigured, this may involve installing a USIM (USIM1) to RG1.

User A places RG1 in/near the living room of the ground floor. RG1 provides connectivity to multiple smart devices in the home network. User A and his family enjoy great connectivity when making video calls or watching movies in the smart TV.

User A devices to go to work to his office on the first floor above the garage. User A uses the office for working purposes and online computer gaming after work, where gaming requires very low latency. Regrettably, connectivity speed in the first floor is a bit slower and QoS is variable.

User A orders a second RG device (RG2) from Operator A and the corresponding USIM, USIM2.

User A installs the USIM (USIM2) to RG2, if the RG2 was not yet preconfigured. User A installs RG2 in his office.

#### 5.5.1.4	Post-conditions

User A and his family enjoy of great FWA connectivity in the whole house and their garden.

#### 5.5.1.5	Potential New Requirements needed to support the use case

[PR 5.5..1-1] The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices).

[PR 5.5.1-2] The 6G system shall support FWA in relevant bands taking into consideration the regulatory requirements for each specific band.

[PR 5.5.1-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real-time resource allocation for FWA.

[PR 5.5.1-4] The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services.

[PR 5.5.1-5] The 6G system shall support efficient FWA connectivity.

Editor's Note: this requirement is FFS.

[PR 5.5.1-6] The 6G system shall support using multiple RGs offering FWA connectivity for the same network operator in a CPN.

[PR 5.5.1-7] The 6G system shall support load balancing between multiple RGs offering FWA connectivity to devices in a CPN.

* * * End of Change * * * *