3GPP TSG SA WG 1 Meeting #110 	S1-252323

19-23 May 2025, Fukuoka, Japan	   (revision of S1-250286)

Source:	Huawei, China Telecom, China Mobile, vivo

pCR Title:	Use case on data service for efficient collection and distribution of diversified data

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Fangyuan Zhu< zhufangyuan@huawei.com >

Xu Xia<xiaxu@chinatelecom.cn>

Yu Menghan <yumh1@chinatelecom.cn>

Xiaonan Shi<shixiaonan@chinamobile.com>

Yanchao Kang<kangyanchao@vivo.com>

Abstract: This document provides a new use case of data service provided by 6G system, which focuses on efficient collection and distribution of diversified data originated inside 6G system between data providers and data consumers.

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

Editor’s Note: This definition could be updated as the study goes on to align with the potential new understand of AI agent.

AI service: an 6G service (e.g., AI model inference) to support AI-related activities, which is provided by 6G core network to the 6G user, considering the required quality of the service (e.g., inference accuracy, latency).

NOTE 1:	The above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

Editor’s Note:	The definition of AI service is FFS.

Digital Twin: A real-time representation of physical assets in a digital world.

NOTE 2: This definition was taken from ITU-T Recommendation Y.3090 [113].

Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data stored in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement. One subscriber can have one or more Intelligent Communication Assistants.

Network Digital Twin: virtual replica of (part of) a mobile network to emulate (or simulate) the behaviour of the actual network.

Editor’s Note: it is FFS to update this definition.

Personal Data: any information relating to a user or subscriber that can be used to, either directly or indirectly, identify that user or subscriber, or to distinguish that user or subscriber from others.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

Data service: a service offering multiple types of data (e.g. AI related data, 3GPP sensing data, positioning data) originated or processed inside the 6G system, which is provided by the 6G system to the data consumer.

NOTE3: the data consumer could be trusted third-party or UE or network function.

* * * Second Change * * * *(new text)

# 5	System and Operational Aspects

## 5.x	Use case on data service for efficient collection and distribution of diversified data

### 5.x.1	Description

To provide both communication service and services beyond communication (e.g., AI service, sensing service), various types of data will be originated or processed inside 3GPP system. For example:

- Sensing data, which is related to different verticals, e.g., UAVs, smart home, V2X, factories, railways, public safety, etc, in order to enable novel sensing applications. Sensing data can be processed into different types or formats, which are related to sensing measurement, and sensing result (e.g., trajectory).

- AI data, e.g., AI/ML training data, AI/ML inference data, AI model data or AI/ML inference results, which can be used to improve network performance or provide AI services to applications.

- Positioning data, which is related to different environments (e.g. rural, urban, indoor).

- Network operation data, e.g. data related to configuration, performance, logs, and alarms, which can be obtained from network entities for network operation and maintenance management.

- NF context data, including context data distributed in multiple NFs, e.g., UE context for mobility management, session context data for session management, policy context data for policy, etc., which can be gathered and shared among multiple stateless NFs, for the purpose of cloud native and serverless design paradigm and simplified storage of state information.

The characteristics of these various data originated or processed inside 6G system can be listed as below:

- Substantial volume: multiple types of data are usually originated in wide range and huge amount. For example, 3GPP sensing data can be generated by multiple sensing nodes distributed in a wide area, such as UE or Base stations deployed in different areas.

- Diversified data needs: Different data consumers requires diversified types, formats and security and privacy protection of data, which results in different requirements for data collection, storage, delivery and processing. For example, sensing data needs to be collected, processed and aggregated in a timely manner under certain SLA requirements.

- Diversified data providers and consumers: The data providers that provide specific types of data are diversified. For example, the data providers that can provide AI data, sensing measurement data, positioning measurement data, and NF context data are highly diversified. Similarly, various data originated or processed inside 6G system can be consumed by different data consumers, such as different third parties.

In order to make full use of the above data originated or processed inside 6G system to provide more comprehensive intelligent capability and improve operational efficiency, the 6G system needs to support data service offering multiple types of data to data consumers.

### 5.x.2	Pre-conditions

- The mobile network operator #A supports data service.

- The XR headset as a UE supports XR application, which is the subscriber of the 6G network deployed by mobile network operator #A. The XR headset can be connected to mobile network operator #A’s 6G network. The XR headset is equipped with 3GPP sensors.

### 5.x.3	Service Flows

Figure 5.x.3-1: Data service provided by 6G system

- Bob is visiting this theme park and playing a game using a XR headset in the environment. His XR headset as a UE sends a request of XR cloud rendering gaming to XR Application server.

- In order to provide a better XR gaming experience and handle the complicated XR rendering task, the XR Application server as data consumer, requests the data service provided by the mobile network operator #A. Based on the request from XR Application, the mobile network operator #A determines that different data from sensing service, AI service, positioning service, and NF context management service can be used jointly for a single XR Application for XR rendering task.

The above multiple types of data can be collected, distributed, and delivered inside the 6G system via an efficient data delivery path which fulfils the performance requirement such as transmission latency, e.g., within 20ms [50]. The network operator #A can also guarantee data security and privacy during the life-cycle of data. For example, before exposing multiple types of data to the XR application, multiple types of data is pre-processed by mobile network operator #A. The content of the processed data is provided to the XR application regarding users’ demand.

- The XR application server receives its required data and performs XR pre-rendering using the multiple types of data, and then sends the pre-rendered XR data to Bob’s XR headset with the assistance of the mobile network operator #A.

- Bob’s headsets further performs local rendering and displays the XR game scenes to Bob , and the XR game player enjoys a good XR game experience.

### 5.x.4	Post-conditions

Thanks to data service provided by the mobile network operator #A, Bob can experience a seamless XR gaming service.

### 5.x.5	Existing features partly or fully covering the use case functionality

In TS 22.261 [14], the following requirements are specified for efficient user plane in which user plane paths can be selected or changed to improve the user experience or reduce the bandwidth pressure, when a UE or application changes location during an active communication, or due to operational needs in the service hosting environment (e.g. based on usage information):

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path between UEs attached to the same network, modifying the path as needed when the UE moves during an active communication.

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network; or

-	an application server located in a customer premises network or personal IoT network.

Compared to 5G system, 6G system introduces new services beyond communication, such as sensing and AI. New types of data in wide range and huge amount are originated or processed within 6G system, which is different from application traffic data delivered in the traditional data transmission pipelines. The data providers and data consumers involved in the services beyond communication can be highly diversified, e.g., network entities from core network, RAN nodes, UEs, third party.

### 5.x.6	Potential New Requirements needed to support the use case

[PR-5.x.6-1]: Subject to operator’s policy, the 6G system shall support a mechanism to determine the multiple types of data (e.g., AI related data, 3GPP sensing data) and corresponding data providers, based on the request from the data consumer.

[PR-5.x.6-2]: Subject to operator’s policy, the 6G system supporting data service shall support a mechanism to efficiently collect multiple types of data from data provider, considering high volume of the collected data and latency requirements requested from data consumer.

[PR-5.x.6-3]: Subject to operator’s policy, the 6G system supporting data service shall support a mechanism to flexibly configure data delivery path between data provider and data consumer, considering the topology among data sources and data consumers.

[PR-5.x.6-4]: Subject to operator’s policy, the 6G system supporting data service shall support a mechanism to process multiple types of data originated inside the 6G system.

[PR-5.x.6-5] Subject to operator’s policy, the 6G system supporting data service shall support a mechanism to efficiently store multiple types of data originated or processed inside the 6G system for data reuse.

[PR-5.x.6-6] Subject to operator’s policy, the 6G system supporting data service shall support a mechanism to guarantee data security and privacy during the life-cycle of data.

[PR-5.x.6-7] Subject to operator’s policy, the 6G system shall support a mechanism to expose multiple types of data to the data consumer.

NOTE: the data consumer in the above requirements could be trusted third-party or network function or UE.

* * * End of Change * * * *