3GPP TSG SA WG 1 Meeting #110 	S1-252326

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Title:	Use Case on smart elderly care

Agenda item:	8.1.6

Source:	Huawei, HiSilicon

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution provides a new use case on smart elderly care using embodied AI. This UC is submitted to the Immersive Communication section due to its close relationship with multi-media services.

1. Introduction

This use case is about smart companion for the elderly using real-time cloud-assisted inference. A humanoid intelligent robot is equipped with multiple sensors to capture real-time physical environment information as well as facial and body information of the elderly. That information can be uploaded to the cloud for training and inference to assist the humanoid intelligent robot performing multi-tasks in a real-time interactive manner to the elderly.

2. Reason for Change

A new use case is proposed with the associated potential requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change (new content) * * * *

## 9.x	Providing Multi-task Immersive Interactive Companionship to Elderly

### 9.x.1	Description

Embodied Artificial Intelligence (e.g. humanoid robots, quadruped robots, L4+ automated driving vehicles, drones, etc.) can access large language model (LLM) in the cloud (including operator managed data network, edge or central cloud) for both data training and inference to handle complicated tasks. One typical real-time cloud-assisted inference use case is smart companion for the elderly. A humanoid intelligent robot is equipped with multiple sensors including cameras which capture real-time physical environment information as well as facial and body information of the elderly. That information can be uploaded to the cloud for training and inference to assist the humanoid intelligent robot performing multi-tasks in a real-time interactive manner to the elderly.

### 9.x.2	Pre-conditions

LLM is deployed in the cloud (including operator managed data network, edge or central cloud), and the intelligent terminals are capable of accessing LLM for training and inference. The intelligent terminals are not necessary to be AI-enabled, but is able to run AI applications with the assistance from the cloud/network. Sensors (e.g. cameras) are equipped at the intelligent terminals to obtain environmental data including monitoring/detecting environment information. Intelligent terminals are able to use cellular network for uploading these data for training and inference in the cloud, and receiving real-time inference results. The consent is given by the user(s) to upload such information to the cloud.

### 9.x.3	Service Flows

The following introduces the service flows of cloud training and inference for intelligent terminals in this use case where the embodied robots (such as humanoid robot of Unitree G1 [9x-1]) are used to provide smart elderly care.

1. To enable real-time interactive AI applications, the embodied robot needs to establish the connection to the cloud for training and real-time inference.

2. Upon authentication and connection set-up, with the required user consent the embodied robot uploads the real-time environment information and the elderly’s video/image information captured by its equipped cameras via secure network connection to the cloud.

3. It is assumed that the LLMs in the cloud are well trained and learned from tens of thousands of real environment and elderly behaviour data. Upon receipt of the uploaded video data, the LLMs can accurately identify the elderly's needs; and then derive the real-time companion indications through the cloud inference. In addition, with the required user consent the cloud LLMs can keep a record of the elderly's behaviour at different times.

4. The inference result, such as the companion indication information (e.g., initiating active communication with the elderly), is then to be delivered to the embodied robot with low latency through network to instruct it to perform specific communication with the elderly.

Figure 9.x.3-1: example of cloud training and inference for intelligent terminals

### 9.x.4	Post-conditions

Benefiting from the support of LLMs, cloud AI will bring help to intelligent terminals besides the on-device AI. Moreover, cloud AI can provide better model training with more data samples and computing power and better real-time inference with larger models and computing power, which can better enable the real-time cloud inference applications of intelligent terminals.

### 9.x.5	Existing features partly or fully covering the use case functionality

The data rate requirements to 5G system are listed in Clause 7.1 and Clause 7.10 of TS22.261 [14]. As in Table 7.1-1 of [14], up to 50Mbps UL experienced data rate are required for the general outdoor scenarios. As in Table 7.10.1-1 of [14], 144 Mbps UL experienced data rate are required for split AI/ML inference between UE and Network server.

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Urban macro | 50 Mbit/s | 25 Mbit/s | 100 Gbit/s/km2 (note 4) | 50 Gbit/s/km2 (note 4) | 10 000/km2 | 20 % | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 2 | Rural macro | 50 Mbit/s | 25 Mbit/s | 1 Gbit/s/km2 (note 4) | 500 Mbit/s/km2 (note 4) | 100/km2 | 20 % | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 5 | Dense urban | 300 Mbit/s | 50 Mbit/s | 750 Gbit/s/km2 (note 4) | 125 Gbit/s/km2 (note 4) | 25 000/km2 | 10 % | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (note 1) |
| NOTE 1: For users in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. NOTE 4: These values are derived based on overall user density. Detailed information can be found in [10]. |  |  |  |  |  |  |  |  |  |

Table 7.10.1-1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 15 ms | 144 Mbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| 100 ms | 1.5 Mbit/s |  |  |  | # 100 ms | # 150 Mbit/s | 1.5 MByte/‌frame |  | # Enhanced media recognition |
|  | # 4.7 Mbit/s |  |  |  | 12 ms | 320 Mbit/s | 40 kByte |  | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |  |  |

### 9.x.6	Potential New Requirements needed to support the use case

[PR 9.2.6-4]: The 6G system shall be able to support the KPI requirements summarized in the table below.

NOTE:     For the cloud real-time inference case, there are several levels of data rate requirements based on the camera numbers and resolution information configured for the intelligent terminals (e.g. intelligent robots). The requirements summarized in the table consider 3 different scenarios, where different types of video streams are assumed: I) existing product; II) an upgrade of I with front view cameras (dual camera similar to “human eyes”) replaced with 4K resolution 30 fps refresh rate; III) upgrade of II with additional 2 cameras covering 360°replaced with 4K resolution 30 fps refresh rate.

Table 9.x.6- 1: Requirements real time video uploading for immersive interactive companion to elderly

| Scenarios | Number of cameras (with video resolution) (note1) | Uplink KPI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service bit rate (note2) | Max allowed UL end-to-end latency | Communication service availability | Reliability |
| I | 1080p * 6 | 20 Mbit/s | 20 ms (note 3) | 99.999 % | 99 % (note 4) |
| II | 1080p * 4 + 4K * 2 | 60 Mbit/s |  |  |  |
| III | 1080p * 2 + 4K * 4 | 100 Mbit/s |  |  |  |
| NOTE 1: It is assumed 6 RGB cameras are equipped for the robot. NOTE 2: Data Rate = Video resolution * (Bits per colour*3) * Refresh rate / Compress ratio. The data rate is estimated assuming (similar to the real-time video uploading in [9x-2]): compression ratio of 240 and 8 bits per colour; 1080p video resolution, refresh rate of 15 fps; and 4K video resolution, refresh rate of 30 fps. NOTE 3: Max allowed UL end-to-end latency is the delay budget to successfully transmit a video frame. 15 ms delay on radio interface is assumed as captured in TR 38.838 [9x-3] for video uploading in AR use cases. NOTE 4: Reliability value is as captured in TR 38.838 [9x-3] for video uploading in AR use cases. |  |  |  |  |  |

* * * Second Change * * * *

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

… …

[9x-1] 	https://www.unitree.com/cn/g1

[9x-2] 	https://std.samr.gov.cn/hb/search/stdHBDetailed?id=2001448F0D0D32F5E06397BE0A0AAD9C

[9x-3]	3GPP TR 38.838: "Study on XR (Extended Reality) Evaluations for NR".